Image-Based Radiodensity Profilometry Measures Early Remodeling at the Bone-Callus Interface in Sheep

Tianyi Ren1, Karina Klein2, Brigitte von Rechenberg2,3, Salim Darwiche2,3, and Hannah L. Dailey†1

1Department of Mechanical Engineering and Mechanics, Lehigh University, Bethlehem, PA, USA

2Musculoskeltal Research Unit (MSRU), Vetsuisse Faculty, University of Zurich, Switzerland

3Center for Applied Biotechnology and Molecular Medicine (CABMM), University of Zurich, Switzerland

†Correspoinding Author:

Hannah Dailey

Department of Mechanical Engineering & Mechanics

Packard Laboratory, Room 556

19 Memorial Drive West

Bethlehem, PA 18015

USA

Email: hlr3@lehigh.edu

Abstract (150-250 words: currently 222)

Bone healing has been traditionally described as a four-phase process: inflammatory response, soft callus formation, hard callus development, and remodeling. The remodeling phase has been largely neglected in most numerical mechanoregulation models of fracture repair in favor of capturing early healing using a pre-defined callus domain. However, in vivo evidence suggests that remodeling occurs concurrently with repair and causes changes in cortical bone adjacent to callus that are typically neglected in numerical models of bone healing. The objective of this study was to use image processing techniques to quantify this early-stage remodeling in ovine osteotomies. To accomplish this, we developed a numerical method for radiodensity profilometry with optimization-based curve fitting to mathematically model the bone density gradients in the radial direction across the cortical wall and callus. After assessing data from 26 sheep, we defined a dimensionless density fitting function that revealed significant remodeling occurring in the cortical wall adjacent to callus during early healing, a 23% average reduction in density compared to intact. This fitting function is robust for modeling radial density gradients in both intact bone and fracture repair scenarios and can capture a wide variety of the healing responses. The fitting function can also be scaled easily for comparison to numerical model predictions and may be useful for validating future mechanoregulatory models of coupled fracture repair and remodeling.

Keywords: Remodeling, Bone fracture healing, Micro-computed tomography, Tibia, Ovine osteotomy

Acknowledgments

The preclinical studies from which this data was obtained were funded by the Johnson & Johnson Family of Companies. The authors wish to thank Beat Lechmann (Johnson & Johnson Family of Companies) for agreeing to grant access to the ovine study data for our analyses. This material is based upon work supported by the National Science Foundation under Grant No. 1943287. Any opinions, findings, and conclusions or recommendations expressed in this material are those of the author(s) and do not necessarily reflect the views of the Johnson & Johnson Family of Companies or the National Science Foundation.

Conflicts of interest

BvR is a paid consultant and receives research support as a principal investigator from Johnson & Johnson Family of Companies. The other authors report no conflicts relevant to this work.

Introduction

The fracture healing process is commonly described as a four-phase concept: inflammation phase, soft callus formation phase, hard callus formation phase, and bone remodeling phase (Claes et al. 2012; Pivonka and Dunstan 2012; Einhorn and Gerstenfeld 2015; Ghiasi et al. 2017). Numerical simulations have been used to predict mechanobiological processes in the regions of fracture healing. Numerous finite-element (FE) models have been developed to analyze local mechanical responses to varying loading conditions (Ament and Hofer 2000; Claes et al. 2002; Lacroix and Prendergast 2002; Shefelbine et al. 2005; Simon et al. 2011; Steiner et al. 2013, 2014; Wilson et al. 2017; Wang and Yang 2018; Pietsch et al. 2018; Ghiasi et al. 2019; Engelhardt et al. 2021). Such models have been used to study different factors that impact the healing process, for example the mechanics of the fixation hardware, or outcomes following inverse dynamization (Wehner et al. 2014; Wilson et al. 2017). Some studies have also emphasized that the initial phase of healing is critical for modeling of the healing process (Ghiasi et al. 2019). Recently, we applied a mechanoregulatory model of fracture healing to show that the rate of healing depends on the geometry of the fracture and that all fracture types experience a small healing delay with torsional instability (Ren and Dailey 2020). We also showed that modifying a strain-based mechanoregulatory model of fracture healing to include a geometric proximity function can successfully restrict callus formation to the region near the fracture line and achieve simulation domain independence (Schwarzenberg et al. 2021).

These techniques hold promise to predict outcomes under different mechanical or biological conditions and in response to new treatment strategies, but they all share a key limitation: the cortical wall is assigned static mechanical properties and included in the mechanical loading simulations, but it is excluded from the mechanoregulatory healing algorithms. These models neglect remodeling activity during fracture healing because this last stage of fracture healing is often described conceptually as having limited overlap with the repair phase. However, in vivo data suggests that remodeling of the cortical wall occurs concurrently with soft callus development and mineralization. For example, previous reports generated from nanoindentation testing have provided highly localized evidence of decreasing bone density in the radial direction through the cortical bone toward the callus (Preininger et al. 2011). This evidence indicates the remodeling phase of secondary bone fracture healing is associated with structural changes in both the callus and the adjacent cortical bone throughout the healing process and that these effects should be included in early-stage fracture repair models.

The key barrier to developing new mechanoregulatory models that can consider coupled repair and remodeling is the lack of available validation data. The validation strategy used for most recent mechanoregulation models has focused on the global structural mechanics of the limb and qualitative comparison of model predictions to histological analysis of tissue in the callus.  For example, the assessment of a model’s performance in predicting the spatial and temporal development of tissues in the callus is usually based on only the general trends of mechanical data (e.g., torsional rigidity) (Ren and Dailey 2020; Schwarzenberg et al. 2021) or temporal data (e.g., bridging time, interfragmentary motion) (Lacroix and Prendergast 2002; Isaksson et al. 2006; Wehner et al. 2014; Pietsch et al. 2018). Ideally, the localized tissue composition and distribution should also be investigated to assess whether tissue growth and differentiation criteria within the algorithms are functioning correctly. The historical gold standard for assessing mechanical properties within tissue samples has been nanoindentation testing. This method has been used to probe the micro-scale properties of bone and callus and identify gradients in porosity and density (Manjubala et al. 2009; Preininger et al. 2011; Mora-macías et al. 2017). However, these physical measurement techniques involve time-consuming destructive postmortem testing with specialized equipment and the resulting data cannot be obtained in vivo, or for large regions of tissue, or in large numbers of samples. Regrettably, these limitations mean that generalizable quantitative data describing remodeling across the cortical wall into callus does not currently exist.

Accordingly, the purpose of this study was to develop a numerical technique for noninvasively quantifying structural remodeling adaptations at the bone-callus boundary in sheep by investigating the radiodensity data from micro-computed tomography (µCT) scans. We hypothesized that measurable cortical remodeling happens early in the healing period, in conjunction with bridging of the external callus in secondary fracture healing.

Methods

Animal Specimen Information

Twenty-six adult female Swiss alpine sheep (2-3 years old, weighing 59 – 87 kg) were part of two previously completed research studies with two different tibial osteotomy models stabilized by medial plating. Group 1 consisted of data from twelve animals with a 17 mm tibia defect augmented with autografts and stabilized with a 13-hole stainless steel plate (broad straight veterinary 3.5 mm LCP, 172 mm in length, with 3.5 mm bicortical screws; DePuy Synthes®). Sheep in group 1 were sacrificed 12 weeks after surgery. Group 2 consisted of data from fourteen animals with a 3 mm gap tibia defect stabilized with a 12-hole stainless steel plate (broad straight veterinary 3.5 mm locking compression plate (LCP), 159 mm in length, with 3.5 mm bicortical screws; DePuy Synthes®). Sheep in group 2 were sacrificed 9 weeks after surgery. All experiments were conducted at the Musculoskeletal Research Unit in Zürich, Switzerland, according to the Swiss laws of animal protection and welfare and authorized by the local governmental veterinary authorities (License No. ZH 183/17).

Micro-Computed Tomography (μCT) Scanning

After animal sacrifice, both intact and osteotomized tibiae were excised, stripped of soft tissue, and all hardware was removed, taking care not to disrupt the callus. Samples were then wrapped in saline-soaked gauze, and µCT scanned using an XtremeCT II Micro-CT scanner (Scanco Medical AG, Bruettisellen, Switzerland) with an X-ray voltage of 68 kVp and X-ray current of 1470 µA. The resulting scans had an isotropic resolution of 60.7 µm.

Image Segmentation

A region of interest (ROI) was selected at three different locations of each operated limb diaphyseal segment scan: 10 mm from the most proximal aspect of the tibia, 10 mm from the most distal aspect of the tibia, and just proximal to the fracture line at the level of maximum callus radius (Fig. 1). The proximal and distal ROI were above and below the most proximal and distal screw holes. Sampling slices from the intact limb were also taken at the corresponding levels. At each sampling location (region of interest), a total of 20 slices were selected for analysis.

An automated segmentation algorithm was developed in MATLAB 2021a (The MathWorks, Inc., Natick, Massachusetts, USA) to define the outer boundary of the callus as well as the area enclosed by the pericortical boundary of the preexisting cortical bone. The detailed workflow can be seen in Fig. 2. Post-segmentation data processing was carried out on all 2D tomograms located within each ROI.

The process for segmentation of cortical bone and callus was based in part on previously reported methods (Buie et al. 2007; Morgan E, Mason Z, Chien K, Pfeiffer A, Barnes G, Einhorn T et al. 2009; Bissinger et al. 2017). To complete this task, the algorithm required two initial estimates of the segmentation thresholds. The selection of these thresholds affects the cortical bone and callus boundaries of the output mask and requires subsequent refinement. First, the soft tissue region was excluded using connected-component labeling. Noise was then reduced by applying a median filter. Next, the original dataset was re-thresholded and masked with the detected boundaries of the bone-callus region. Dilation and erosion operations were then used to determine the bone and callus boundary. The initial estimates of the segmentation thresholds were then refined by minimization of spline energy to achieve smoothness of the detected tissue boundaries for each sheep. A locally convex boundary criterion was also used to correct any boundary defects caused by small regions of lower-density cortical bone. Finally, a mask of the callus, cortical bone, and non-bone regions was created by combining the output of the previous steps.

The slice extraction and segmentation processes were repeated at all three levels of height in both the intact and operated tibiae. No callus was identified in the intact limbs.

Radiodensity Data Sampling and Curve Fitting

The process for radiodensity sampling is illustrated in Fig. 3. In each 2D tomogram within the region of interest, the centroid of the cortical bone was calculated from the endocortical boundary (red boundary) in the mask geometry. A polar array of sampling lines was created in 1-degree increments throughout the entire 360-degree angle range, with each line having its origin at the centroid and extending through the cortical wall and callus. The radiodensity values along the profile lines were sampled in 60 µm increments in the radial direction, a distance equal to the in-plane resolution of the scan. Pixel data was retrieved from the grayscale image using a bicubic interpolation technique. Data quality was checked for each profile line and individual profile lines were excluded from the data set if the callus was not present or too thin (thickness smaller than 10mm, measured in the radial direction along a profile sampling line). Radial sampling was repeated using the same procedure for all 20 slices in each ROI and pooled to create a point cloud that defined the radiodensity profile line for each ROI in each animal (Fig. 1c).

In this animal model, medial plating produced asymmetric callus formation, as can be clearly seen in Fig. 3c. Cortical wall thickness was also not constant across all radial profile lines. To enable a combined curve-fitting analysis of the profilometry samples from within each region of interest, a registration and scaling procedure was defined to create a consistent dimensionless coordinate system for the cortical wall and for the callus, when present. Each profile line was registered to set its linear coordinate system to zero at the bone-callus boundary, or the pericortical boundary if intact. Negative linear coordinates corresponding to the cortical wall were normalized to the local cortex thickness of each profile line (Fig. 3c/d), such that r = -1 was the endocortical boundary and r = 0 was the pericortical boundary. For the positive linear coordinates, the normalization procedure depended on whether callus was present. For the intact limbs, positive coordinates were normalized to the local cortex thickness (Fig. 5b). For the operated limbs with callus, positive coordinates were normalized to the doubled local callus thickness, producing r = 0.5 at the outer boundary of the callus mask and ensuring data sampling beyond the callus into the soft tissue zone (Fig. 5c). The resulting average number of sampling points per region of interest was 170,340.

To enable comparisons between the treated and untreated limbs of all animals, the sampled radiodensity values were also normalized to the median cortical bone density of the intact diaphysis of each animal. Within each animal, all local radiodensity values sampled from the profile lines were normalized to this baseline median cortical bone density, regardless of limb (intact or operated) and ROI location (proximal, midshaft, and distal).

Curve fitting techniques were then used to mathematically model the collective radial density profile using the sampling points collected at each ROI. To develop an appropriate fitting function for measuring local structural changes, we started with the simplest case, an intact cortical wall fit with a double sigmoid function:

| fr=11+e-a1(r-b1)*11+e-a2r-b2 | (1) |
| --- | --- |

where in general for a sigmoid function, the parameters a sets the width of the transition zone from a function value of 0 to a function value of 1, and the parameter b sets the center of this transition zone.

Next, a logical linear term and a constant were added to the double sigmoid function to account for the possibility of a nonzero density gradient in the cortical wall (r<0) and nonzero background radiodensity:

| fr=11+e-a1(r-b1)*11+e-a2r-b2+d+cr r<0 0 r≥0 | (2) |
| --- | --- |

where c is the linear slope applied only within the cortical wall and d is the background density constant.

To adapt this method for profilometry samples with callus, an additional sigmoid term was added:

| fr= 11+e-a1(r-b1*(g1+1-g11+e-a2r-b2)+cr+d1 , &r<0g21+e-a3(r-b3+d2, &r≥0 | (3) |
| --- | --- |

where the constants g1 and g2 in the sigmoid terms allow for non-unity in the upper sigmoid plateau and the constants d1 and d2 allow for nonzero background radiodensity on both the internal and external bounds of the fitting function. In this piecewise definition, the cortical function (r<0) is a generalized double logistic function, and the callus function is a single logistic function.

Parameter values for constants in Eq. 2-3 were fit using the nonlinear least-squares method via the MATLAB Curve Fitting Toolbox (R2021a). In addition to this standard curve-fitting technique, a multi-objective optimization method was introduced to fit the parameters of the function defined in Eq. 3 with additional constraints requiring position and slope continuity of the piecewise fit function at r=0 as described in Eq. 4. This multi-objective optimization method can be formulated to find the coefficients of fr, minimizing the residuals (sum of the squares), of all the data points:

| min a,b,g,di=1m(yi-fr<0ri)2 mina,b,g,d,ck=1n(yk-fr>0rk)2 s.t.fr<00 -fr>00=0 fr<0'0- fr>0'0=0 | (4) |
| --- | --- |

where ri and rk are the set of data points in the cortical wall or external to the cortical wall,n and m are the total number of data points of the cortical wall or the callus. The minimization problem was solved using the MATLAB Optimization Toolbox (R2021a). After the fitting parameters were obtained for each sheep, the resulting fit function, fr, was evaluated to calculate the slope at the bone-callus interface to quantify the extent to which remodeling had produced a smooth functionally graded interface at the bone-callus boundary. For intact bone not bounded by callus, an additional custom MATLAB script was also used to calculate the average density value in the cortical wall plateau, which was reported as the mean cortical density of each fitted curve.

Statistical Analysis

All statistical analyses were performed using SPSS Statistics 27 (IBM Corp; Armonk, New York, USA). A value of P = 0.05 was used as the threshold for statistical significance. A two-tailed paired-sample t-test was used to analyze if the fitting function constants and the evaluated slope of the fitting function at the pericortical boundary were significantly different between the operated and contralateral limbs.

Results

Choice of Curve Fitting Function

Several alternative methods were investigated and critically evaluated with respect to their ability to capture the essential features of the bone density profile curves. The fitting results of the double sigmoid function (Eq. 2) are shown in Fig. 4a. This function provided a reasonable approximation for the intact cortical wall, but it was not able to capture the increasing slope of the density in the radial direction for intact cortical bone and it missed both the cortical and callus plateaus in the data for the operated tibia. Adding a logical linear term and a constant produced a good fit with R2≥0.93 for the double sigmoid function (Eq. 3) with cortical bone, but this fitting function failed to capture the remodeling behavior evidenced by a decrease in density in the cortical wall at the bone callus boundary (Fig. 4b). The piecewise logistic function (Eq. 4) was then used to get a better fit when callus was present, this function provided a better fitting quality (R2≥0.95), but when fit using a nonlinear least squares minimization, the function was not continuous at the bone callus interface (Fig. 4c).

For both the intact and operated tibiae, the best fitting method was to use the multi-objective optimization method (Fig. 4d) to identify the parameters of the fitting function shown in Eq. 4. This produced continuous fit functions with goodness of fit statistics showing an overall R2≥0.87 for all intact tibiae and operated tibiae. In all samples, all six fitting parameters had 95% confidence intervals that did not include zero. The fitting parameters for all 26 animals at each of the six sampled ROIs can be seen in the supplementary digital content.

The performance of the final fitting function (Eq. 4 with multi-objective optimization) is shown for a different animal in Fig. 5. This single function successfully captured several observed characteristics of both the intact and operated limb. Specifically, in the intact tibia, it captured the observed increase in radiodensity in the radial direction from the endosteal to the pericortical boundary (Fig. 5a). For the operated limb, the fit curve captured the overall decrease in radiosity in the cortical wall and the localized remodeling at the pericortical boundary bounded by callus (Fig. 5b).

Each parameter of the fitting function was included to capture the observable features of the profilometry data in different regions of the fitted curve and at transitions between regions. The resulting constants can then be associated with physical features related to the bone-callus radial density function. The magnitudes of parameters a1 and a2 define the width of the region of density change at the periosteal and endocortical boundarys of cortical bone. The parameters b1 and b2 define the location of the center of the transition of the periosteal and endocortical boundarys of cortical bone. Parameter a3 defines the width of the region of density change at the outer boundary of the callus, while parameter b3 sets the location of the center of the transition between callus and the surrounding soft tissue. The linear term c in the cortical bone zone (r ≤ 0) defines the increasing density of cortical bone from the endosteal to pericortical boundary. Finally, the constant d1 captures the average density of cortical bone and the term d2 compensates for the noise of the data outside the callus. Finally, g1and g2 capture the cortical density at the pericortical boundary of the injured limb.

Cortical Tissue Alterations in Operated Limbs

In addition to profilometry sampling at the level of the fracture, we also analyzed cortical bone at the proximal and distal ends of the diaphysis of the intact and operated limbs. The resulting profilometry curves of all animals combined are shown in Fig. 6. Comparing the operated group to their matched intact tibiae at both the distal and proximal ends of the bones, there were several changes in the cortical walls of the operated tibiae, both at the level of the fracture and at the proximal and distal ends of the bone. Table 1 shows the comparisons between the operated and intact limbs based on two summary parameters derived from the fitting functions—density slope at the pericortical boundary and mean cortical wall density. Each parameter was evaluated at the proximal, midshaft, and distal ROIs in both the intact and osteotomized limbs. Both parameters were significantly different between operated and intact for all ROIs, although the cortical density decreases in the operated limb were 10x larger at the midshaft than at the proximal and distal ends of the bone. Considering all animals, sampling at the proximal and distal ends of the bone showed that the mean cortical density was 1.3% and 2.2% lower respectively in the operated limbs compared to intact (p < 0.001).

All animals also showed evidence of remodeling in the cortical wall bounded by callus (Fig. 6d). The preexisting increase in radiodensity toward the pericortical boundary that can be seen in the intact bones (Fig. 6a) disappeared in the osteotomized bones (Fig. 6c). At the pericortical boundary bounded by callus, these changes corresponded to a 23.4% drop on average from the peak density of the native cortical wall. Overall, comparing the intact and operated groups at the midshaft, there were statistically significant differences in all obtained radiodensity profile fitting function parameters except for g1 (p ≤ 0001; see supplementary Table S1). The slope of the fitted density profile curve at the pericortical boundary was also significantly reduced in operated limbs compared to intact at all three ROIs (p ≤ 0.007; see Table S1).

Discussion

This study presents a scalable mathematical method for non-invasive quantification of tissue mineralization density of the cortical tissue matrix and callus within an ovine tibial shaft. The results show that remodeling alterations occur not only in the callus tissue due to mineralization but also in the cortical tissue adjacent to the repair zone throughout the healing process. The findings are consistent with expectations from destructive physical testing and confirm the presence of a decreasing gradient in mineral density of the cortical bone at the vicinity of a fracture during secondary healing. We previously showed that at 9 weeks, most of these osteotomies had achieved a structural rigidity comparable to intact tibiae (Schwarzenberg, 2020). This profilometry curve fitting data shows that in sheep, significant adaptive changes have occurred in cortical bone bounded by callus, concurrent with structural bridging as early as 9 or 12 weeks post-op. It should be noted that the remodeling of cortical bone was measurable not only near the fracture site, but also to a lesser extent at the proximal and distal end of the operated limbs. The methods described herein could be used to measure remodeling as a temporal in vivo outcome measure from imaging only. These measures could be useful as an indicator of healing speed by establishing the status of a fracture on the continuum of mechanical and structural adaptations that arise during healing.

The intended purpose of this investigation was to provide a useful set of validation data for the future development of mechanoregulatory models that include the cortical wall and address coupled remodeling at the cortical-callus boundary. The results confirm that significant changes do occur on the cortical side of the cortical-callus boundary and that not addressing these changes is a critical limitation in the current computational models of bone fracture healing. These models usually assume a time-invariant value for cortical bone stiffness, which does not reflect the in vivo reality of a 23% drop in cortical radiodensity at the cortical-callus boundary. The normalized generalizable fitting function we reported could be used to assess whether mechanoregulatory models that include the cortical zone are capturing the appropriate magnitude of changes, and whether the distribution of predicted material properties at the boundary and within the callus region are valid.

Investigators who are interested in applying our normalized, dimensionless remodeling curve to validate their dimensional mechanoregulation models may wish to use the following scaling procedure. First, observe that the fitting curve cannot be used directly because the function has been normalized in both the radius coordinates (r) and radiodensity values (y axis). To dimensionalize the radial coordinates, recall that the radial coordinates were normalized to the cortical wall thickness for r<0 and to the doubled callus thickness for r > 0. To scale this function appropriately in r, identify the cortical wall thickness, t, and callus thickness, tc. The y coordinates in our fit curve were normalized to the median radiodensity [HU] of the intact limb for each animal. For the N = 26 sheep analysed in this study, the resulting average intact tibia radiodensity was 3164 ± 10 HU. The apparent radiodensity [HU] must be scaled to bone mineral density by means of a linear conversion derived from a phantom scan. This density value can then be converted to a Young’s modulus using a species-specific scaling law. For ovine cortical bone, we recommend the equation derived by (Schwarzenberg et al. 2021), which produces an average Young’s modulus, Eintact = 12.9 GPa for intact bone. After selecting Eintact and matching t and tc to the chosen computational model, the following scaled remodeling function can be used to validate the model predictions:

| Er=Eintact* (11+e-a1(rt-b1*(g1+1-g11+e-a2rt-b2)+rct+d1) , &r<0g21+e-a3(rtc-b3+d2, &r≥0 | (5) |
| --- | --- |

Where the fitting constants (a1, b1, g1, a2, b2, c, d1, a3, b3, g2, and d2) are drawn from Table S1. This dimensionalized remodeling curve will have radial coordinates r that are zeroed at the cortical-callus boundary, but are dimensionally scaled to match the physical size of the geometry in the computational model.

This study is not without limitations. First, the sheep imaging data were only available at the time of sacrifice. As a result, no information about the temporal variations of local mechanical properties of the cortical tissue is currently available for this study. However, due to the inclusion of both large-gap and small-gap healing models, the selected sheep groups represent a wide range of healing responses and callus sizes, suggesting that the numerical methods and resulting fitting function we have proposed are robust. Another key limitation is that we applied the segmentation algorithm to evaluate only a small fraction of the 2D tomograms that comprise the entire cortical wall and focused on gradients in the radial direction. Remodeling effects in the cortical wall also have an axial dependency on proximity to the fracture line, which has not been explored here. Additional work would be required to achieve profilometry analysis of tissue density profiles in a more general 3D sense. Finally, it is important to note that the resolution of the medical images themselves could also influence the curve fitting quality. In this study, we have used high-resolution of micro-CT scans that were obtained post-mortem. Applying these same techniques to in vivo computed tomography would likely require larger voxel sizes and may not provide enough data points for curve fitting purpose in the area of transition especially at the cortical-callus boundary. We recommend that future studies explore whether larger voxels are inferior for detection of remodeling effects in the cortical wall.

Conclusions

This study showed that structural remodeling at the bone-callus interface can be detected in sheep using radiodensity profilometry from micro-CT scans. It also showed that the density of cortical bone adjacent to callus, which correlates with its elastic properties, is not constant during the early healing period and that this region represents an integral part of the process of coupled fracture repair and remodeling. The scalable function presented herein for mathematically representing density gradients in the cortical wall and callus may be useful for validating future mechanoregulatory models of fracture repair.

Figure Captions

Figure 1: Fig. 1 – (a) Coronal slice view from a micro-CT scan of an ovine tibial osteotomy at 9 weeks post-op. In this study, radiodensity gradients were sampled in the direction indicated by the red arrow. (b) Three regions of interest (ROI) were identified for sampling in each intact and each operated limb: proximal, distal, and at the level of the fracture. (c) Each ROI consisted of 20 axial slices that were analyzed to measure radial radiodensity profiles in the cortical wall and callus region.

Figure 2: Regions of cortical bone and callus were identified using an automated segmentation method developed in MATLAB 2021a. This method requires two global threshold inputs, which are used to initialize the splines that define the endosteal, pericortical, and callus boundaries, and subsequently refined as shown.

Figure 3: (a) A polar array of radial profile lines was created from the cortical centroid through the cortical wall and callus, if present, and radiodensity values were sampled from the underlying scan data. (b) For intact bones, each profile line was registered to set its linear coordinate system to zero at the pericortical boundary and sampling points were extended up to one cortical thickness into the surrounding soft tissue. (c) For samples with callus, profile lines were registered to zero at the bone-callus boundary and sampling points were extended to double the local callus thickness. (d) Resulting compiled profilometry sampling data for one animal at the level of the callus in the osteotomized limb. All radiodensity values were normalized to the median cortical bone density of the intact tibia for that animal.

Figure 4: Profilometry data sampled at the level of the fracture for intact tibiae (black, left) and operated tibiae (red, right). Fit curves from the three tested fitting equations in panels (a)-(c), with all parameters obtained following a least-squares minimization approach using the Curve Fitting Toolbox in MATLAB. Fit curves in panels (c) and (d) both use Eq. 4, but additional continuity constraint at r = 0 was achieved in (d) using multi-objective optimization.

Figure 5: Profilometry sampling points and resulting fitted curves using the method of Fig. 4(d) in a different animal. The radial coordinate registration procedure allowed consistent referencing of key physical locations across different animals: endocortical boundary, pericortical boundary, and callus outer boundary (if present). The density normalization procedure eliminated baseline differences in bone properties between animals by scaling all sampling data points to the median cortical bone density for the intact tibia in each animal.

Figure 6: Profilometry curves were fitted from data sampled at the same levels at the proximal end (a) or distal end (b) end of intact tibiae (black curves) and operated tibiae (red curves). Profilometry curves were also fitted from data sampled at the midshaft of all intact tibiae (c, black curves) and operated tibiae (d, red curves).

Table 1: Comparison of profilometry curve fitting parameters between operated and contralateral groups.

References

Ament C, Hofer EP (2000) A fuzzy logic model of fracture healing. J Biomech 33:961–968. https://doi.org/10.1016/S0021-9290(00)00049-X

Bissinger O, Götz C, Wolff KD, et al (2017) Fully automated segmentation of callus by micro-CT compared to biomechanics. J Orthop Surg Res 12:1–9. https://doi.org/10.1186/s13018-017-0609-9

Buie HR, Campbell GM, Klinck RJ, et al (2007) Automatic segmentation of cortical and trabecular compartments based on a dual threshold technique for in vivo micro-CT bone analysis. Bone 41:505–515. https://doi.org/10.1016/j.bone.2007.07.007

Claes L, Eckert-Hübner K, Augat P (2002) The effect of mechanical stability on local vascularization and tissue differentiation in callus healing. J Orthop Res 20:1099–1105. https://doi.org/10.1016/S0736-0266(02)00044-X

Claes L, Recknagel S, Ignatius A (2012) Fracture healing under healthy and inflammatory conditions. Nat Rev Rheumatol 8:133–143. https://doi.org/10.1038/nrrheum.2012.1

Einhorn TA, Gerstenfeld LC (2015) Fracture healing: Mechanisms and interventions. Nat Rev Rheumatol 11:45–54. https://doi.org/10.1038/nrrheum.2014.164

Engelhardt L, Niemeyer F, Christen P, et al (2021) Simulating Metaphyseal Fracture Healing in the Distal Radius. Biomechanics 1:29–42. https://doi.org/10.3390/biomechanics1010003

Ghiasi MS, Chen J, Vaziri A, et al (2017) Bone fracture healing in mechanobiological modeling: A review of principles and methods. Bone Reports 6:87–100. https://doi.org/10.1016/j.bonr.2017.03.002

Ghiasi MS, Chen JE, Rodriguez EK, et al (2019) Computational modeling of human bone fracture healing affected by different conditions of initial healing stage. BMC Musculoskelet Disord 20:1–14. https://doi.org/10.1186/s12891-019-2854-z

Isaksson H, Wilson W, van Donkelaar CC, et al (2006) Comparison of biophysical stimuli for mechano-regulation of tissue differentiation during fracture healing. J Biomech 39:1507–1516. https://doi.org/10.1016/j.jbiomech.2005.01.037

Lacroix D, Prendergast PJ (2002) A mechano-regulation model for tissue differentiation during fracture healing: Analysis of gap size and loading. J Biomech 35:1163–1171. https://doi.org/10.1016/S0021-9290(02)00086-6

Manjubala I, Liu Y, Epari DR, et al (2009) Spatial and temporal variations of mechanical properties and mineral content of the external callus during bone healing. Bone 45:185–192. https://doi.org/10.1016/j.bone.2009.04.249

Mora-macías J, Pajares A, Miranda P, et al (2017) Journal of the Mechanical Behavior of Biomedical Materials Mechanical characterization via nanoindentation of the woven bone developed during bone transport. J Mech Behav Biomed Mater 74:236–244. https://doi.org/10.1016/j.jmbbm.2017.05.031

Morgan E, Mason Z, Chien K, Pfeiffer A, Barnes G, Einhorn T GL, Morgan EF, Mason ZD, et al (2009) Micro-computed tomography assessment of fracture healing: relationships among callus structure, composition, and mechanical function. Bone 44:335–344. https://doi.org/10.1016/j.bone.2008.10.039.Micro-Computed

Pietsch M, Niemeyer F, Simon U, et al (2018) Modelling the fracture-healing process as a moving-interface problem using an interface-capturing approach. Comput Methods Biomech Biomed Engin 21:512–520. https://doi.org/10.1080/10255842.2018.1487554

Pivonka P, Dunstan CR (2012) Role of mathematical modeling in bone fracture healing. Bonekey Rep 1:1–10. https://doi.org/10.1038/bonekey.2012.221

Preininger B, Checa S, Molnar FL, et al (2011) Spatial-temporal mapping of bone structural and elastic properties in a sheep model following osteotomy. Ultrasound Med Biol 37:474–483. https://doi.org/10.1016/j.ultrasmedbio.2010.12.007

Ren T, Dailey HL (2020) Mechanoregulation modeling of bone healing in realistic fracture geometries. Biomech Model Mechanobiol. https://doi.org/10.1007/s10237-020-01340-5

Schwarzenberg P, Ren T, Klein K, et al (2021) Domain-independent simulation of physiologically relevant callus shape in mechanoregulated models of fracture healing. J Biomech 118:. https://doi.org/10.1016/j.jbiomech.2021.110300

Shefelbine SJ, Augat P, Claes L, Simon U (2005) Trabecular bone fracture healing simulation with finite element analysis and fuzzy logic. J Biomech 38:2440–2450. https://doi.org/10.1016/j.jbiomech.2004.10.019

Simon U, Augat P, Utz M, Claes L (2011) A numerical model of the fracture healing process that describes tissue development and revascularisation. Comput Methods Biomech Biomed Engin 14:79–93. https://doi.org/10.1080/10255842.2010.499865

Steiner M, Claes L, Ignatius A, et al (2014) Disadvantages of interfragmentary shear on fracture healing - Mechanical insights through numerical simulation. J Orthop Res 32:865–872. https://doi.org/10.1002/jor.22617

Steiner M, Claes L, Ignatius A, et al (2013) Prediction of fracture healing under axial loading, shear loading and bending is possible using distortional and dilatational strains as determining mechanical stimuli. J R Soc Interface 10:. https://doi.org/10.1098/rsif.2013.0389

Wang M, Yang N (2018) Three-dimensional computational model simulating the fracture healing process with both biphasic poroelastic finite element analysis and fuzzy logic control. Sci Rep 8:1–14. https://doi.org/10.1038/s41598-018-25229-7

Wehner T, Steiner M, Ignatius A, et al (2014) Prediction of the time course of callus stiffness as a function of mechanical parameters in experimental rat fracture healing studies - A numerical study. PLoS One 9:1–16. https://doi.org/10.1371/journal.pone.0115695

Wilson CJ, Schütz MA, Epari DR (2017) Computational simulation of bone fracture healing under inverse dynamisation. Biomech Model Mechanobiol 16:5–14. https://doi.org/10.1007/s10237-016-0798-x