Engineering whiting events in culture: A microalgae-driven calcium carbonate and biomass production process at high pH and alkalinity with the marine microalga Nannochloropsis oceanica IMET1

Yi-Ying Leea, Lauren Jonasa, Russell Hilla, Allen Placea, Greg Silsbeb, Scott Hunsickerb, Elizabeth Northb, 1, and Yantao Lia, c, 1

aInstitute of Marine and Environmental Technology and bHorn Point Laboratory, University of Maryland Center for Environmental Science, Baltimore, MD 21202, USA

c Department of Marine Biotechnology, University of Maryland Baltimore County, Baltimore, MD 21202, USA

1 Correspondence:  Yantao Li, Institute of Marine and Environmental Technology, Baltimore, MD 21202. Phone: (410) 234-8833; Email: yantao@umces.edu

Elizabeth North, Horn Point Laboratory, University of Maryland Center for Environmental Science. Phone: (410) 221-8497; Email: enorth@umces.edu

Key words: carbon capture and storage; microbiologically induced calcium carbonate precipitation (MICP); monohydrocalcite; photobioreactor; whiting.

Abstract

Whitings are large scale calcium carbonate precipitation events in oceans and lakes, often during algal blooms. However, at the current ocean pH, formation of calcium carbonate is accompanied by releasing carbon dioxide into the surrounding environment, negating the carbon capture effect of photosynthetic microalgae on short geological time scales. This work demonstrated an ecosystem-inspired biotechnology innovation for carbon capture and storage in a high pH, high alkalinity Nannochloropsis culture through development of a microalgae-driven calcium carbonate and biomass production process. When growing the marine microalga Nannochloropsis oceanica IMET1 in photobioreactors, culture alkalinity more than doubled, increasing from 72.5 mg L-1 to a maximum level of 159.6 mg L-1. At the same time, culture pH increased from 7.9 to over 10 and maintained at pH 10.1 for 23 days with concomitant calcium carbonate production. X-ray diffraction analysis revealed the precipitated calcium carbonate was primarily monohydrocalcite. The lab culture was scaled up to a 340-L bioreactor, in which Nannochloropsis ash-free dry weight productivities ranged from 25.1 to 51.4 g m-2 d-1 and a maximum monohydrocalcite productivity of 133.4 g m2 d-1 was recorded. In this system, Nannochloropsis biomass contained about 23.9% lipids and the eicosapentaenoic acid content was about 1.8%. Together, these results suggest the microalgae-driven calcium carbonate and biomass production process efficiently captures and stores atmospheric carbon dioxide in the form of calcium carbonate while producing valuable bioproducts with applications in the nutraceutical market. Study of the bacterial communities associated with the Nannochloropsis culture identified four dominant species Maricaulis maris, Marinisubtilis pacificus, Gracilimonas sp., and an uncultured bacterium in the OD1 phylum with interesting features that warrant further investigation.

- Introduction

Clouds of fine grained calcium carbonate (CaCO3) precipitated in marine or freshwater environment during or after phytoplankton blooms have long been observed in nature and described as whiting events (Shinn et al., 1989; Thompson et al., 1997). In some places, whiting events occur frequently at a large scale covering the great lakes, e.g., Lake Michigan (58,000 km2) or smaller regions in oceans, e.g., the Gulf Coast of Florida and Bahama Banks (Dittrich and Obst, 2004; Long et al., 2017; Long et al., 2018; Yao et al., 2023). Whiting events also can occur repeatedly over time: whiting events were visible on MODIS on NASA’s Terra satellite images for 13.8% of days near the Gulf Coast of Florida over a 13-year time span (Long et al., 2017; Long et al., 2018). However, the impact of whiting events on global inorganic carbon cycle and climate change is still debatable, because whitings may be a source or a sink for inorganic carbon and organic carbon in oceans and lakes depending on how they are formed and deposited (Geyman et al., 2022; Legendre et al., 2015).

Globally, it is suggested that phytoplankton, including microalgae and cyanobacteria, can sequester 1100 Tmol carbon (C) per year, while calcification algae (i.e., CaCO3 forming) sequester up to 47 Tmol C per year (Raven, 2018). In the ocean, CaCO3 precipitation is often initiated by calcifying organisms in a process known as the carbonate pump (also known as carbonate counter-pump) through Reaction (1): Ca2++ 2HCO3-⇌CO2+CaCO3+H2O (Volk and Hoffert, 1985). At the current ocean pH (~8.1) bicarbonate is the dominant carbonate species and calcification processes lead to formation of CaCO3 from Reaction (1), during which about 0.6 CO2 is released to the surrounding water per CaCO3 produced (Legendre et al., 2015; Raven, 2018). As a result, CaCO3 formation through this reaction in the ocean has been suggested to have no effect on capturing atmospheric CO2 on short geological time scales (<100 years) (Legendre et al., 2015; Zeebe, 2012).

In contrast, emerging microalgal technologies that harness the power of photosynthesis have the potential to increase culture pH and alkalinity, thereby fostering carbon capture and utilization (CCU) through biological means (Daneshvar et al., 2022). Nannochloropsis spp. has attracted much industrial and academic interest because of their ability to grow rapidly, synthesize large amounts of lipids including high-value omega-3 fatty acids such as eicosapentaenoic acid (EPA), and tolerate changing environmental conditions (Cheng et al., 2022; Chua and Schenk, 2017). Maintaining axenic culture of algae has been a major challenge for the algal biotechnology industry, particularly for high-value nutraceutical and pharmaceutical markets (Vu et al., 2018). However, for algal carbon sequestration and biofuel production applications, it is not economically viable nor necessary to grow axenic algae cultures, because other microbes such as the bacterial community associated with microalgae may contribute to culture stability and algal growth stimulation (Lin et al., 2022; Wang et al., 2016). Moreover, previous work on the microbially induced calcium carbonate precipitation (MICP) process has identified bacterial species that contribute to CaCO3 precipitation (Chu et al., 2012; Ivanov and Chu, 2008). Prior work focused on bacterial processes directed by urea hydrolysis, denitrification, or sulfate reduction (Castro-Alonso et al., 2019; Zhu and Dittrich, 2016), while the role of bacteria in microalgae-driven carbonate precipitation process has not yet been investigated in depth.

In this work, our engineering principle is inspired by whiting events. Through an ecosystem-inspired biotechnology approach (Schweitzer et al., 2021), we aim to engineer a microalgae-driven calcium carbonate and biomass production (MadCAP) process for carbon capture, usage and storage. We chose to grow N. oceanica strain IMET1 in bioreactors nonaxenically, a well-characterized strain for which a genome sequence and genetic manipulation tools are available (Li et al., 2014; Radakovits et al., 2012; Vieler et al., 2012; Wang et al., 2014). We developed the MadCAP process with N. oceanica in the lab scale (1 L) and pilot scale bioreactors (340 L). The MadCAP process resulted in a large increase in culture pH and alkalinity, leading to simultaneous production of CaCO3 and algal biomass. Further analysis revealed precipitated CaCO3 was in the form of monohydrocalcite. Substantial amounts of monohydrocalcite and omega-3 fatty acids-rich algal biomass could be produced if properly scaled up. The bacteria closely associated with N. oceanica IMET1 and the free-living bacterial communities were studied, with the novel OD1 strain identified with some interesting features that need further study.

- Materials and Methods

2.1 Algae culturing and measurements in 1-Liter cultures

N. oceanica IMET1 was maintained and cultured in modified f/2 medium (Guillard, 1975; Guillard and Ryther, 1962) in which saltwater was adjusted to 17 - 34 ppt and the final concentration of NaNO3 and NaH2PO4∙H2O was adjusted to 0.9 g L-1 and 40 mg L-1, respectively. The seed culture was maintained at room temperature under continuous light illumination of 10 - 20 μE m−2 s−1 in Erlenmeyer flasks. Optical density, cell density, dry weight and pH were measured to assess algal growth. Optical density was measured by using a Nanodrop 2000c Spectrophotometer (Thermo Fisher Scientific, Pittsburgh, PA) at 750 nm (OD750). Cell density was measured using a hemocytometer (Hausser Scientific). For algal biomass dry weight measurements, algal biomass was collected on a GF/F glass fiber filter with 45 mm diameter (Whatman) under vacuum, washed with distilled water, dried at 100 °C for overnight and weighed on an analytical balance.

For the first two laboratory-scale studies, a 1-liter closed recirculating algal culture system was carried out in a horizontal culture chamber under continuous light illumination of 50 μE m−2 s−1 at room temperature with aeration by air sparging through air stones. Algae were grown in the algae side of the horizontal culture chamber separated by a Whatman Nuclepore Track-Etch polycarbonate membrane with 1 μm pore size. Different lines were set up with appropriate ports for media, algae sampling, filtrate sampling, and air venting to prevent contaminations of the clear side by the algae side (Fig. S1). The system was maintained in semi-continuous culture by supplying fresh f/2 medium to the algae side once the culture reached pH 10. Algae culture samples were collected from the algae side, and algal fluid samples were collected from the fluid (clear) side of the chamber. Measurement of water chemistry indexes (such as pH, salinity, calcium and total alkalinity) of algal fluid were performed by ZooQuatic Laboratory, LLC. To collect precipitate, 280 ml algal fluid was passed through a 3 µm polycarbonate filter and then an 1 µm polycarbonate filter.

In the third lab-scale study, the culture was maintained in a 1-liter Pyrex bottle under continuous light illumination of 50 μE m−2 s−1 at room temperature with aeration by air sparging through air stones. This study was conducted to determine whether addition of EDTA in the growth medium was essential for growth and for maintaining high pH in the Nannochloropsis cultures. Because EDTA in the culture medium is known to chelate with metal ions (such as Ca2+), its presence may affect calcium carbonate formation. The f/2 media was prepared with standard EDTA concentrations and with no EDTA.

In the fourth lab-scale study, sodium bicarbonate was added to the cultures to determine if it would enhance the MadCAP process. The cultures were maintained in 1-Liter Pyrex bottles under the same conditions as the third study. Various concentrations of sodium bicarbonate solution were added at the beginning of the experiment when the cultures were inoculated (0M, 0.02, 0.05, and 0.1 M sodium bicarbonate).

2.2 Bacterial communities associated with N. oceanica strain IMET1

For characterization of associated bacteria, N. oceanica strain IMET1 was grown with ambient air in the laboratory with either no addition of NaHCO3 or 0.02 M NaHCO3. Microalgal cultures and associated bacteria (20 ml samples) were vacuum-filtered through a 0.45 µm cellulose acetate filter (Advantec) with a Nalgene Reusable Filter Unit (Thermo Scientific), and the filtrate was collected. The filtrate was subsequently filtered through a 0.22 µm Sterivex filter (Millipore Sigma) using sterile tubing and a peristaltic pump. Filters were kept at -20°C. DNA from both size fractions was extracted using the DNeasy Powerwater kit (Qiagen) and sent for sequencing. DNA was sequenced on an Illumina paired-end platform (MiSeq 2 x 300) at the BioAnalytical Services Laboratory (BASLab) at IMET. Bioinformatic analysis was performed with QIIME 2 2023.5 (Bolyen et al., 2019). Raw forward and reverse sequence data were demultiplexed, quality filtered, and merged using the q2‐demux plugin followed by denoising with DADA2 (Callahan et al., 2016). Taxonomy was assigned to amplicon sequence variants (ASVs) using the q2‐feature‐classifier (Bokulich et al., 2018) with the classify‐sklearn naïve Bayes taxonomy classifier trained from the newest Silva database (version 132 – full length). Taxonomic identities were then confirmed manually through the NCBI blastn program at 99% or higher identity. A fasta file was created and exported from the QIIME 2 representative sequences artifact output file after denoising with DADA2. Sequences were uploaded to the NCBI blastn program and identities were confirmed using a percent identity alignment of 99% or higher. Corresponding alignment files were downloaded for each major ASV within the dataset. A phylogenetic tree was created with Méthodes et algorithmes pour la bio informatique (MAB) (Dereeper et al., 2010; Dereeper et al., 2008), alignment with MUSCLE (Edgar, 2004), a curation by Gblocks which eliminates poorly aligned positions and divergent regions (Castresana, 2000), PhyML for phylogeny (Anisimova and Gascuel, 2006; Guindon and Gascuel, 2003), and TreeDyn for tree rendering (Chevenet et al., 2006). For confidence analysis, data was resampled using 1,000 bootstrap replicates.

2.3 Algae culturing and measurements in the 340-L bioreactor

For the two scaled-up studies, a custom-built 340 L-photobioreactor system (Fig. S2) was used for the experiment. LED lights housed in the center of the bioreactor provided continuous illumination for the culture. The light channel and intensity of the LED lights was controlled by a custom program that also was used to fill and empty the bioreactor. Aeration was provided with ambient air through an air pump, and the flow rate was controlled by air valves and monitored by an air flow meter. For this system, f/2 media (with 34 ppt salinity) was made with artificial seawater supplied by Aquaculture Research Center at UMCES IMET. Cell density, optical density, pH and dry weight were measured to monitor the growth; aeration and culture level in the bioreactor were recorded to monitor the operation. The area productivity was calculated using an bioreactor area ratio of 1:1.2 to represent the bioreactor "facility" square footprint (Davis et al., 2012). Because of the size and non-axenic nature of the bioreactor experiments, there were no replicates for each experiment. Instead, these experiments were repeated independently in an effort to determine similar trends and observations were found.

2.4 Collection and analysis for calcium carbonate precipitates

Algal fluid collected from the 1 L-growth chamber was passed through a 3 µm polycarbonate filter, and the filtrate was then passed through a 1 µm polycarbonate filter to examine the presence of calcium carbonate precipitates. For cultures in the 340 L-bioreactor, all solid matter in a culture suspension was first collected by centrifugation at 2000x g for 10 min. The pellets were then resuspended in distilled water. To separate CaCO3 precipitate crystallines from microalgal biomass in the suspension, low force centrifugation was employed owing to the weight density of the CaCO3 particles being greater than Nannochloropsis cells. The separation was achieved by repeating low speed centrifugation (50x g - 230x g) and washing with distilled water until the supernatant became clear. Finally, the washed pellets were freeze-dried and weighted.

The precipitated carbonates were examined under dissection microscope.  The composition of the crystalline precipitate was determined by X-ray diffraction (XRD) analysis. The proportion of carbonates in the precipitate was estimated using a custom-built ‘Acid Test’ chamber that measured the CO2 (CO2Meter K30 10,000ppm CO2 Sensor) evolved when 90% phosphoric acid was mixed with a known mass of precipitate. The CO2 evolved from the precipitate was compared to a standard curve developed for CaCO3 (CaCO3(mg) = 0.0114*CO2(ppm) + 0.1503, R2 = 0.99) with proportional correction based on the molecular weight of CaCO3 and the crystalline carbonate identified by XRD.

2.5 Lipid content measurement and fatty acid analysis

Total lipids were extracted from algal biomass from the 340-L bioreactor experiment using Biogogno’s method (Bigogno et al., 2002). In brief, 100 mg dry algal biomass was treated with 2 ml methanol/DMSO (9:1, v/v) for 1 h at 50oC followed by another hour at 0oC with stirring. The mixture was spun down, and the first batch of supernatant was collected. The residual biomass was then treated with 4 ml hexane/diethyl ether (1:1, v/v) for 1 h at 0oC with stirring, and the supernatant was collected and combined with the first batch of supernatant. The treatment was repeated until the residual biomass lost color. Water was added into the collected solvent mixture (the collected supernatant) to make a mixture of methanol/hexane/ether/water 1:1:1:1 for phase separation. The organic phase containing lipids was collected in a pre-weighed vial and evaporated under a gentle stream of N2 gas. Total lipids content was calculated as the weight of the lipid extracts divided by the processed dry biomass. For fatty acid analysis, fatty acids in total lipids were converted to fatty acid methyl esters (FAMEs) with 1% H2SO4 in methanol at 85oC for 1.5 h, dissolved in hexane, and then profiled using TSQ Triple 8000 GC-MS System (Thermo Scientific). FAME standards (Sigma-Aldrich) and heptadecanoic acid (C17:0) (Sigma-Aldrich) were used as the external and internal standards, respectively.

- Results

3.1 Nannochloropsis culture maintained high pH and transiently increased alkalinity

In a laboratory-scale microalgae driven calcium carbonate and biomass production (MadCAP) process, pH increased rapidly when algae grew to exponential stage in the first horizontal bioreactor experiment (Fig. S1). In a culture of N. oceanica IMET1 grown under 17 ppt salinity, pH increased from 7.9 to 10.2 while cell density increased 10-fold within 6 days after inoculation (Fig. 1a). After day 13, the culture was shifted to semicontinuous growth conditions. During this stage, the culture was able to maintain pH 10.0 ± 0.3 for 42 days with little fluctuation (Fig. 1a).

The salinity, total alkalinity and calcium concentration in the 1 L-N. oceanica IMET1 culture were monitored over time. The salinity increased slightly from 17.1 ppt to 18.9 ppt, while total alkalinity gradually increased by about 2.2-fold, i.e., from 72.5 to 159.6 mg/L on day 16, and then gradually decreased afterwards but kept in a range between 88 and 128 mg/L (Fig. 1b). Calcium concentration remained relatively stable during the first 16 days and maximum reduction of about 28.6% was observed on day 20, indicating some calcium being either consumed by algae or precipitated.

The high pH algal fluid was tested for the presence of calcium carbonate precipitation. White and yellowish solids were visible on the 3 µm and 1 µm polycarbonate filters (Fig. 1c). Microscopic analysis revealed some crystalline solids on the filters (Fig. 1c, lower panel), indicating formation of carbonate precipitation. It was clear that the culture system needed to be scaled up to collect more precipitates for further analysis.

Similar results showed when N. oceanica IMET1 was grown under 34 ppt salinity in the second experiment in the horizontal culture chamber. In this culture, pH started at 7.8 and reached 10.2 within 4 days of batch culture. Afterwards N. oceanica IMET1 was cultured under semicontinuous conditions and the culture maintained at pH 10.1 ± 0.1 for 23 days (Fig. S3).

A third experiment was conducted in 1-L Pyrex bottles to determine how EDTA influenced pH and culture growth. Algae growth in the absence of EDTA was much slower (Fig. 2). The cell density and optical density of the IMET1 culture grown in the presence of EDTA were 4.4-fold and 5.7-fold higher, respectively, than that in the absence of EDTA (Fig. 2a). In this experiment, the culture grown in the presence of EDTA reach pH 10.0 within 3 days, and was able to maintain high pH (pH >10.5). When grown in the absence of EDTA, the culture pH increased slightly from 8.6 to 8.7 within 3 days and then remained stable, but was not able to increase beyond pH 8.8 throughout the entire growth course (Fig. 2b). These results suggest EDTA is necessary for maintaining Nannochloropsis growth and high culture pH.

The fourth lab-bench experiment was designed to explore the effect exogenous bicarbonate on the MadCAP formation (Fig. 3). Compared with cultures supplemented with sodium bicarbonate, the culture without sodium bicarbonate (0 M) grew better. The cell density of the culture without sodium bicarbonate addition was over 2-fold higher than the cultures supplemented with 0.02 M and 0.1 M sodium bicarbonate after day 9 (Fig. 3b). Besides the culture density, the culture pH without sodium bicarbonate increased rapidly from 8.6 at day 2 to 9.9 at day 4 and remained stable at 10.6 at day 6 and beyond (Fig. 3c). However, in this experiment the calcium carbonate precipitate in the culture without sodium bicarbonate addition was too low to measure (Fig. 3e). The cultures supplemented with 0.02 M and 0.1 M sodium bicarbonate were able to reach and retain a culture pH above 10 after day 4, and the precipitate yield was elevated by the addition of sodium bicarbonate (Fig. 3e). XRD analysis showed that the precipitate crystalline was composed of calcium carbonate (100% monohydrocalcite) from the culture supplemented with 0.02 M and 0.1 M sodium bicarbonate. Precipitates formed in the culture supplemented with 0.5 M sodium bicarbonate consist of largely magnesium carbonate (96.5% nesquehonite) and small portion of calcium carbonate (3.5% monohydrocalcite) (Fig. 4). However, 0.5 M sodium bicarbonate supplementation completely inhibited algae growth (Fig. 3a, 3b and 3d).

3.2 Bacterial communities associated with cultures of N. oceanica IMET1

To analyze the bacterial communities of N. oceanica, two fractions of samples from the fourth lab-bench experiment were collected and analyzed separately, including the bacteria closely associated with N. oceanica IMET1 (obtained on 0.45 µm-filter fractions together with the algae) as well as the free-living bacterial communities (0.45 µm-filtrates subsequently concentrated on 0.2 µm filters). As expected, the 0.45 µm fraction was dominated by chloroplast 16S rRNA gene sequences from N. oceanica. To better reveal the 16S rRNA gene sequences derived from bacteria, all chloroplast sequences were removed bioinformatically. The resultant bacterial communities are shown in Fig. 5.

The bacterial community of N. oceanica IMET1 was dominated by four species. In the bacterial community closely associated with N. oceanica strain IMET1, Maricaulis maris and Gracilimonas sp. were dominant on Day 0 and Day 4 regardless of whether microalgae were supplemented with NaHCO3 or not. Marinisubtilis pacificus, a member of the Microbacteriaceae family, also was found at a lower relative abundance. Interestingly, after days 0 and 4, all samples were completely dominated by a novel uncultured bacterium clone. Preliminary phylogenetic analysis (Fig. S4) suggests that this novel bacterium may be a member of the uncultured OD1 phylum.

3.3 Scaling up the Nannochloropsis culture in a large-scale bioreactor

To scale-up, a batch culture of N. oceanica IMET1 was initiated in the 340 L-bioreactor and then the volume was increased stepwise. At the beginning, 4 L of the IMET1 seed culture was inoculated into 80 L of f/2 media. The initial culture was grown under gentle aeration (< 0.5 L/min) and low intensity of blue light (ca. 20 µmol m-2 s-1) (Fig. S2c) to avoid cell death due to shear force and high-light stress. Cell density and OD750 increased from 2.55 x 106 cells/ml and 0.069 at day 0 to 14.2 x 106 cells/ml and 0.385, respectively, on day 10 (Fig. 6a and 6b). pH increased rapidly from pH 8.6 (at day 0) to pH 9.1 (at day 2) within the first 2 days and reached a peak of pH 9.3 at day 8 (Fig. 6c). At day 10, 100 L fresh media was added into the bioreactor to increase the culture volume to ca. 160 L (Fig. 6f) that diluted the culture to 5.84 x 106 cells/ml, with OD750 at 0.17 and pH at 7.5 (Fig. 6a, 6b and 6c). Aeration was increased gradually to promote algae growth. After 2 days of culture dilution, its pH increased to 9.0 (at day 12), and the cell density were also increased to the level prior to dilution (12.3 x 106 cells/ml). After day 12, the cell density and the optical density continued to increase, and pH remained between 8.9 – 9.1 (Fig. 6a, 6b and 6c). At day 17, when the cell density and optical density reached 16.8 x 106 cells/ml and 0.54, respectively, 80 L of the culture was removed from the bioreactor and 80 L of fresh media was refilled to maintain a semi-continuous culture and dilute the culture. At this time, the light source changed from blue light (20 µmol m-2 s-1) (Fig. S2c) to white light (50 µmol m-2 s-1) (Fig. S2d) to increase the light intensity. After day 17, cell density and optical density of the culture rapidly increased. In 3 days (at day 20), cell density and optical density bounced back to the levels as prior to the dilution at day 17 (Fig. 6a and 6b). Cell density continued to increase to reach the maximum of 29.1 x 106 cells/ml at day 23 (Fig. 6a), and then another 100 L of the culture was replaced with fresh media. The culture again responded rapidly after the refill of fresh media. At day 30, 150 L fresh media was filled in to increase the water level to full tank (ca. 80 in; 320 L) (Fig. 6f). The filling of fresh media resulted in a drop in cell density, optical density and pH, but the culture recovered in 3 days, as in the case of previous dilutions. Overall, N. oceanica IMET1 grew rapidly while increasing culture pH in the scaled-up bioreactor, and the semi-continuous culture status was maintained in the 340 L-bioreactor system.

3.4 MadCAP in the scaled-up bioreactor

In the second bioreactor experiment, we attempted to grow IMET1 in the 340 L-bioreactor so that it would reach a high culture pH (pH ≥10) that would facilitate calcium carbonate formation. In the first test of the bioreactor, the culture pH was stable at ~ pH 9 (Fig. 6c). In this test, the light intensity next to the light compartment in the 340 L-bioreactor was 50 μE m-2 s-1, and at the mid-point of the culture the light intensity was only 10-20 μE m-2 s-2 (Fig. 7a), hence the low culture pH level likely was due to light limitation. Possible limiting factors include low light transmission through the plastic material over the LED light strips that had become cloudy, low light input, long light path, etc.

To improve the illumination in the 340-L bioreactor, we replaced the LED light strips (Fig. 7b) with higher powered-LED lights (Fig. 7c). As a result, the light intensity in the headspace in the bioreactor chamber (i.e. above the culture level) was 680 μE m-2 s-1 at the point adjacent to the light compartment (the near point; Fig. 7a) and 450 μE m-2 s-1 at the mid-point (Fig. 7a) between the light compartment and the wall of the bioreactor (Table S1). Moreover, the light intensities in an algal culture with self-shading effect of cells in a diluted culture at day 0 were 320-350, 80-100 and 12-35 μE m-2 s-1 at the near, the middle and the far points (as indicated in Fig. 7a) from the light compartment, respectively (Table S1). Under the improved lighting conditions, pH of the culture in the bioreactor was able to increase from 9.0 to 10.0 within 5 days (Fig. 8; Table 1); cell density increased 1.8-fold from 26.6 x 106 to 47.2 x 106 cells ml-1, and optical density (OD750) increased 2.7-fold from 0.69 to 1.84 (Fig. 8; Table 1). The improved lighting conditions brought the maximal biomass dry weight to 0.5 g L-1 and the AFDW to 0.34 g L-1 on Day 19 (Fig. 8c). The area productivity of the algal dry weight and AFDW were 30.0 - 72.0 g m2 d-1 and 25.1 – 51.4 g m2 d-1, respectively, during the 19-day growth course (Fig. 8e). On Day 19 at the end of the experiment, the area algal dry weight and AFDW productivity were 39.8 g m2 d-1 and 25.2 g m2 d-1, respectively (Fig. 8e).

Precipitates were collected from the culture grown in the second 340-L bioreactor experiment. In this batch, the starting pH was 8.8, and the pH increased to 10.0 on day 2 and 10.6 on day 3 (Table 1). The yield of precipitates was 0.012, 0.20 and 0.03 g/L from the samples take on day 0, day 2 and day 3, respectively (Table 1). The precipitate was examined under dissection microscope (Fig. 9a), and the composition of the crystalline components was assessed by X-ray diffraction (XRD) analysis that indicated the crystalline components in precipitates were monohydrocalcite (Fig. 9c). When immersing the precipitate in 90% phosphoric acid on our Acid Test chamber, the concentration of CO2 evolved from precipitates (e.g., Fig. 9b) collected on day 0, 2, and 3 indicated that approximately 75%, 69%, and 61%, respectively, of the precipitate mass was monohydrocalcite. Considering these data, on day 2, a maximum monohydrocalcite productivity of 133.4 g m2 d-1 was achieved.

3.5 Lipid production in the scaled-up bioreactor

Nannochloropsis is known to produce high level of lipids and has abundant omega-3 polyunsaturated fatty acids (PUFAs), such as eicosapentaenoic acid (EPA). We evaluated total lipid content and fatty acid profile in Nannochloropsis biomass grown in the second run of the 340 L-bioreactor. Under the growth condition in our system, Nannochloropsis contains 23.9 ± 0.9% lipids of the dry biomass. The lipids consisted of eleven fatty acids that included seven different PUFAs (Fig. 10). The top three most abundant fatty acids were, palmitic acid (C16:0), palmitoleic acid (C16:1∆9) and EPA (C20:5) (Fig. 10). The EPA content in our culture was 1.8 ± 0.04% dry weight and 7.1 ± 0.5% over the total lipids.

- Discussion

Our MadCAP experiments were set up to mimic phytoplankton-induced whiting events in nature. Whiting events are often seen in freshwater lakes and certain water columns of oceans at a scale visible on satellites, ranging from about 0.1- 226 km2 in the Bahama Banks to about 58,000 km2 in Lake Michigan (Dittrich and Obst, 2004; Yao et al., 2023). Here, we showed that our laboratory and pilot scale MadCAP processes precipitated CaCO3 from Nannochloropsis culture in the form of monohydrocalcite (Fig. 9), which is readily transformable to calcite and aragonite that can be used as cementitious materials (Fukushi et al., 2011; Saridhe and Selvaraj, 2021), while Nannochloropsis biomass can be used as feedstocks for bioproducts and biofuels (Cheng et al., 2022). We grew N. oceanica in bioreactors from 1 L to 340 L, demonstrating our system is highly scalable while the non-axenic Nannochloropsis culture maintained its productivity at high pH and alkalinity during the course of our experiments (Figs. 1 and 8).

Under the current ocean pH (8.1), CaCO3 precipitation by calcifying organisms such as microalgae and macroalgae through Reaction (1) is known to release CO2 to the surrounding water (Legendre et al., 2015; Raven, 2018; Volk and Hoffert, 1985). As a result, CO2 removal and addition through photosynthetic organisms in the ocean is suggested to have no effect on total alkalinity (Gattuso et al., 1999). On the other hand, algal growth is known to couple with assimilation of N and P, such as NO3- and HPO42-, which can uptake H+ or liberate OH-, resulting in increase in culture pH and alkalinity (Gattuso et al., 1999). We demonstrated that in our MadCAP process, Nannochloropsis cultures grown with NO3- and HPO42- were able to increase culture alkalinity from 72.5 mg L-1 to a maximum level of 159.6 mg L-1 while maintaining culture pH at over 10 for 23 days (Figs. 1 and 2). At this pH, culture alkalinity is dominated by carbonate ions (Barker and Ridgwell, 2012) and CaCO3 is primarily formed through Reaction (2): CO32-+Ca2+⇌CaCO3. This leads to net sequestration of CO2 in the form of CaCO3, in contrast to Reaction (1) that leads to CO2 release. In line with this, recent work showed that increase of pH and total alkalinity was the key contributor of carbonate precipitates in the Bahamian carbonate mud, rather than phytoplankton-driven whitings at current ocean pH (Geyman et al., 2022).

Recently, an algal-bacterial co-culture was found to promote CaCO3 formation in saline environment, with the halophilic microalga Dunaliella salina and its mutualistic halophilic bacteria Nesterenkonia sp. isolated from saline environment (Gu et al., 2023). The co-cultivation stimulated CaCO3 precipitation by about 55% and promoted the assimilation of CO2 by 50% (Gu et al., 2023). It was estimated that with the productivity of their system, 2.3 Tg C yr-1 carbon sequestration is achievable in global saline lakes (Gu et al., 2023). In this work, we captured and stored CO2 in the form of monohydrocalcite, a hydrated amorphous calcium carbonate often found in recent sediments in saline lakes (Fukushi et al., 2011), and achieved a peak monohydrocalcite productivity of 133.4 g m2 d-1 growing under continuous light (Fig. 8). Assuming this productivity, we estimate a C capture rate of 13.6 g m2 d-1 or 4950.8 tons km-2 yr-1.  To reach the same 2.3 Tg C yr-1 carbon sequestration with monohydrocalcite, we need only 464.6 km2 land, just a fraction of the global saline lakes area (6x106 km2). However, it must be noted that the scaling up process for algal bioreactors at such scale would be much more complex and costly, and no commercial facility has reached that level yet (da Silva and Reis, 2015). In addition to carbon capture and storage, monohydrocalcite has been found to effectively absorb environmental contaminants, such as phosphorus, arsenic, and lead (Fukushi et al., 2016; Munemoto et al., 2014), thus may be advantageous over traditional carbon sequestration methods by remediation of these toxic compounds.

Using highly alkaline conditions to grow alkaliphilic algal cultures has been proposed as a way to improve carbon utilization efficiency and cost-effectiveness of algal biotechnology (Canon-Rubio et al., 2016; Chi et al., 2011; Wensel et al., 2014; Zhu et al., 2022). Biocarbonate has been used in previous work to lower carbon supply, biomass growth and harvesting cost, as well as labor and energy costs (Canon-Rubio et al., 2016; Chi et al., 2011; Wensel et al., 2014; Zhu et al., 2022). In this study, while addition of bicarbonate promoted CaCO3 formation, it didn’t increase culture pH and compromised algal growth (Fig. 3). These results suggest N. oceanica IMET1 is not an ideal strain for carbon capture with the biocarbonate feeding approach. Instead, its growth promotes an increase in the seawater culture medium pH and alkalinity and thus facilitates sequestration of CO2 in the form of CaCO3 when aerated with air. Our previous test reveals that N. oceanica IMET1 is tolerant to high (up to 12%) CO2 concentrations (Tan et al., 2021), as such we have been testing this strain for direct carbon capture from power plant flue gases with high concentrations of CO2 (6-12%).

N. oceanica IMET1 is a model oil-producing microalga and known to produce the high value omega-3 fatty acid EPA in its lipidome (Jia et al., 2015; Li et al., 2014; Wang et al., 2014; Xin et al., 2017). EPA-rich Nannochloropsis biomass has been proposed to be used in the aquaculture as well as nutraceutical industries, while the biorefinery idea of using Nannochloropsis spp. to produce EPA, other bioactive compounds, and biofuels has been proposed (Cheng et al., 2022; Chua and Schenk, 2017). Our work showed that N. oceanica IMET1 is also a promising alga for permanent carbon sequestration in the form of monohydrocalcite (CaCO3) while producing EPA. The EPA content of N. oceanica was moderate (Fig. 10), however, if successfully scaled up, at the current AFDW productivity (25.2 g m2 d-1, Fig. 8), we estimate 9,198 tons km-2 yr-1 of algal biomass productivity (AFDW) and 164.6 tons km-2 yr-1 of EPA (C20:5n-3) productivity. It is estimated that about 0.8 million tons of EPA+DHA per year is currently produced for human consumption (Hamilton et al., 2020; Tocher, 2015), falling short of the estimated nutritional demand of 1.4 million tons yr-1 required to supply the global population a recommended daily dosage of 500 mg EPA + DHA at an EPA:DHA ratio of 2:1 (Hamilton et al., 2020; Shang et al., 2017). At our current productivity, if EPA is produced as a byproduct, about 2,430 km2 land would be needed for Nannochloropsis culture to fill the gap between the recommended EPA dosage and the current market.

In nature and in most outdoor algae culture facilities, microalgae and bacteria often grow in non-axenic cocultures, but their molecular interactions remain poorly understood (Seymour et al., 2017). MICP is mostly studied with bacteria-induced CaCO3 precipitation directed by urea hydrolysis (Castro-Alonso et al., 2019; Zhu and Dittrich, 2016), while microalgae-bacteria interaction during CaCO3 precipitation in polyculture is not understood. As such, we characterized the bacterial communities associated with N. oceanica IMET1. Among the bacterial groups found to be associated with N. oceanica IMET1, Maricaulis maris (previously described in the Caulobacter genera) is a common aquatic and halophilic species that has been described as scavenger of carbon in oligotrophic habitats (JGI Genome Portal; (Abraham et al., 1999)). Gracilimonas sp. have been found associated with the photosynthetic bacterium Synechococcus (Choi et al., 2009). Previously, a community of Gracilimonas containing microorganisms collected from sediment slurries collected at Soap Lake (WA) was shown to have a comparable or higher CaCO3 precipitation rate when compared with the standard MICP strain Sporosarcina pasteurii (Skorupa et al., 2019), suggesting this bacteria may contribute to CaCO3 precipitation through MICP in our culture. Previous work with Nannochloropsis oculata observed autoflocculation of algal cells in the culture started at pH 9.5 (Tran et al., 2017) while this was not the case in this study. One possible explanation is the difference between the bacterial community in the algal culture, with Flavobacteriales being the dominant bacteria order in their N. oculata culture (Tran et al., 2017). Interestingly, after days 0 and 4, all our samples became heavily dominated by an uncultured bacterium clone related to the novel and enigmatic uncultured OD1 phylum. Our previous findings suggest that some bacteria promote algal growth in co-cultures through producing growth hormones such as auxin (Lin et al., 2022; Singh et al., 2018). Therefore, this is an intriguing finding that warrants further investigation because it is possible that this bacterium plays a role in stimulating growth of algal culture and in the process of MICP.

Acknowledgements

We thank Dr. Peter Zavalij, X-ray Crystallographic Center at University of Maryland College Park for XRD analysis. We thank the funding from The Bailey Wildlife Foundation (BWF) as well as the entire BWF project team and DOE Office of Fossil Energy and Carbon Management (FE-0031914, FE-0032188) for their support.

CRediT authorship contribution statement

Yi-Ying Lee: Methodology, Writing – original draft, Conceptualization, Data curation, Formal analysis. Lauren Jonas: Methodology, Data curation, Writing – review & editing. Russell Hill and Allen Place: Supervision, Writing – review & editing. Greg Silsbe and Scott Hunsicker: Supervision, Methodology, Data curation, Writing – review & editing. Elizabeth North: Methodology, Conceptualization, Writing – review & editing, Supervision, Project administration, Funding acquisition. Yantao Li: Conceptualization, Formal analysis, Writing – original draft, Writing – review & editing, Supervision, Project administration, Funding acquisition.

Declaration of competing interest

The authors declare that they have no known competing financial interests or personal relationships that could have appeared to influence the work reported in this paper.

References

Abraham WR, Strömpl C, Meyer H, Lindholst S, Moore ER, Christ R, et al. Phylogeny and polyphasic taxonomy of Caulobacter species. Proposal of Maricaulis gen. nov. with Maricaulis maris (Poindexter) comb. nov. as the type species, and emended description of the genera Brevundimonas and Caulobacter. Int J Syst Bacteriol 1999; 49 Pt 3: 1053-73.

Anisimova M, Gascuel O. Approximate likelihood-ratio test for branches: A fast, accurate, and powerful alternative. Syst Biol 2006; 55: 539-52.

Barker S, Ridgwell A. Ocean acidification. Nature Education Knowledge 2012; 3: 21.

Bigogno C, Khozin-Goldberg I, Boussiba S, Vonshak A, Cohen Z. Lipid and fatty acid composition of the green oleaginous alga Parietochloris incisa, the richest plant source of arachidonic acid. Phytochemistry 2002; 60: 497-503.

Bokulich NA, Kaehler BD, Rideout JR, Dillon M, Bolyen E, Knight R, et al. Optimizing taxonomic classification of marker-gene amplicon sequences with QIIME 2’s q2-feature-classifier plugin. Microbiome 2018; 6: 90.

Bolyen E, Rideout JR, Dillon MR, Bokulich NA, Abnet CC, Al-Ghalith GA, et al. Reproducible, interactive, scalable and extensible microbiome data science using QIIME 2. Nat Biotechnol 2019; 37: 852-857.

Callahan BJ, McMurdie PJ, Rosen MJ, Han AW, Johnson AJA, Holmes SP. DADA2: High-resolution sample inference from Illumina amplicon data. Nature Methods 2016; 13: 581-583.

Canon-Rubio KA, Sharp CE, Bergerson J, Strous M, De la Hoz Siegler H. Use of highly alkaline conditions to improve cost-effectiveness of algal biotechnology. Applied Microbiology and Biotechnology 2016; 100: 1611-1622.

Castresana J. Selection of Conserved Blocks from Multiple Alignments for Their Use in Phylogenetic Analysis. Molecular Biology and Evolution 2000; 17: 540-552.

Castro-Alonso MJ, Montañez-Hernandez LE, Sanchez-Muñoz MA, Macias Franco MR, Narayanasamy R, Balagurusamy N. Microbially Induced Calcium Carbonate Precipitation (MICP) and Its Potential in Bioconcrete: Microbiological and Molecular Concepts. Frontiers in Materials 2019; 6.

Cheng P, Li Y, Wang C, Guo J, Zhou C, Zhang R, et al. Integrated marine microalgae biorefineries for improved bioactive compounds: A review. Science of The Total Environment 2022; 817: 152895.

Chevenet F, Brun C, Bañuls A-L, Jacq B, Christen R. TreeDyn: towards dynamic graphics and annotations for analyses of trees. BMC Bioinformatics 2006; 7: 439.

Chi Z, O’Fallon JV, Chen S. Bicarbonate produced from carbon capture for algae culture. Trends in Biotechnology 2011; 29: 537-541.

Choi DH, Zhang GI, Noh JH, Kim WS, Cho BC. Gracilimonas tropica gen. nov., sp. nov., isolated from a Synechococcus culture. Int J Syst Evol Microbiol 2009; 59: 1167-72.

Chu J, Stabnikov V, Ivanov V. Microbially Induced Calcium Carbonate Precipitation on Surface or in the Bulk of Soil. Geomicrobiology Journal 2012; 29: 544-549.

Chua ET, Schenk PM. A biorefinery for Nannochloropsis: Induction, harvesting, and extraction of EPA-rich oil and high-value protein. Bioresource Technology 2017; 244: 1416-1424.

da Silva TL, Reis A. Scale-up problems for the large scale production of algae. Algal biorefinery: An integrated approach 2015: 125-149.

Daneshvar E, Wicker RJ, Show P-L, Bhatnagar A. Biologically-mediated carbon capture and utilization by microalgae towards sustainable CO2 biofixation and biomass valorization – A review. Chemical Engineering Journal 2022; 427: 130884.

Davis R, Fishman D, Frank ED, Wigmosta MS, Aden A, Coleman AM, et al. Renewable diesel from algal lipids: an integrated baseline for cost, emissions, and resource potential from a harmonized model. National Renewable Energy Lab.(NREL), Golden, CO (United States), 2012.

Dereeper A, Audic S, Claverie J-M, Blanc G. BLAST-EXPLORER helps you building datasets for phylogenetic analysis. BMC Evolutionary Biology 2010; 10: 8.

Dereeper A, Guignon V, Blanc G, Audic S, Buffet S, Chevenet F, et al. Phylogeny.fr: robust phylogenetic analysis for the non-specialist. Nucleic Acids Res 2008; 36: W465-9.

Dittrich M, Obst M. Are picoplankton responsible for calcite precipitation in lakes? Ambio 2004; 33: 559-64.

Edgar RC. MUSCLE: multiple sequence alignment with high accuracy and high throughput. Nucleic Acids Res 2004; 32: 1792-7.

Fukushi K, Munemoto T, Sakai M, Yagi S. Monohydrocalcite: a promising remediation material for hazardous anions. Sci Technol Adv Mater 2011; 12: 064702.

Fukushi K, Sakai M, Munemoto T, Yokoyama Y, Takahashi Y. Arsenate sorption on monohydrocalcite by coprecipitation during transformation to aragonite. Journal of Hazardous Materials 2016; 304: 110-117.

Gattuso J-P, Frankignoulle M, Smith SV. Measurement of community metabolism and significance in the coral reef CO2 source-sink debate. Proceedings of the National Academy of Sciences 1999; 96: 13017-13022.

Geyman EC, Wu Z, Nadeau MD, Edmonsond S, Turner A, Purkis SJ, et al. The origin of carbonate mud and implications for global climate. Proceedings of the National Academy of Sciences 2022; 119: e2210617119.

Gu W, Wu S, Liu X, Wang L, Wang X, Qiu Q, et al. Algal-bacterial consortium promotes carbon sink formation in saline environment. Journal of Advanced Research 2023.

Guillard RRL. Culture of phytoplankton for feeding marine invertebrates. In: Smith WL, Chanley MH, editors. Culture of Marine Invertebrate Animals. Plenum Press, New York, USA, 1975, pp. 26-60.

Guillard RRL, Ryther JH. Studies of marine planktonic diatoms. I. Cyclotella nana Hustedt and Detonula confervacea Cleve. Canadian Journal of Microbiology 1962; 8: 229-239.

Guindon S, Gascuel O. A simple, fast, and accurate algorithm to estimate large phylogenies by maximum likelihood. Syst Biol 2003; 52: 696-704.

Hamilton HA, Newton R, Auchterlonie NA, Müller DB. Systems approach to quantify the global omega-3 fatty acid cycle. Nature Food 2020; 1: 59-62.

Ivanov V, Chu J. Applications of microorganisms to geotechnical engineering for bioclogging and biocementation of soil in situ. Reviews in Environmental Science and Bio/Technology 2008; 7: 139-153.

Jia J, Han DX, Gerken H, Li YT, Sommerfeld M, Hu Q, et al. Molecular mechanisms for photosynthetic carbon partitioning into storage neutral lipids in Nannochloropsis oceanica under nitrogen-depletion conditions. Algal Research 2015; 7: 66-77.

Legendre L, Rivkin RB, Weinbauer MG, Guidi L, Uitz J. The microbial carbon pump concept: Potential biogeochemical significance in the globally changing ocean. Progress in Oceanography 2015; 134: 432-450.

Li J, Han DX, Wang DM, Ning K, Jia J, Wei L, et al. Choreography of Transcriptomes and Lipidomes of Nannochloropsis Reveals the Mechanisms of Oil Synthesis in Microalgae. Plant Cell 2014; 26: 1645-1665.

Lin H, Li Y, Hill RT. Microalgal and bacterial auxin biosynthesis: implications for algal biotechnology. Curr Opin Biotechnol 2022; 73: 300-307.

Long JS, Hu C, Robbins LL, Byrne RH, Paul JH, Wolny JL. Optical and biochemical properties of a southwest Florida whiting event. Estuarine, Coastal and Shelf Science 2017; 196: 258-268.

Long JS, Hu C, Wang M. Long-term spatiotemporal variability of southwest Florida whiting events from MODIS observations. International Journal of Remote Sensing 2018; 39: 906-923.

Munemoto T, Fukushi K, Kanzaki Y, Murakami T. Redistribution of Pb during transformation of monohydrocalcite to aragonite. Chemical Geology 2014; 387: 133-143.

Radakovits R, Jinkerson RE, Fuerstenberg SI, Tae H, Settlage RE, Boore JL, et al. Draft genome sequence and genetic transformation of the oleaginous alga Nannochloropsis gaditana. Nat Commun 2012; 3: 686.

Raven J. Blue carbon: past, present and future, with emphasis on macroalgae. Biology Letters 2018; 14: 20180336.

Saridhe SP, Selvaraj T. Microbial precipitation of calcium carbonate in cementitious materials – A critical review. Materials Today: Proceedings 2021; 43: 1232-1240.

Schweitzer H, Aalto NJ, Busch W, Chat Chan DT, Chiesa M, Elvevoll EO, et al. Innovating carbon-capture biotechnologies through ecosystem-inspired solutions. One Earth 2021; 4: 49-59.

Seymour JR, Amin SA, Raina J-B, Stocker R. Zooming in on the phycosphere: the ecological interface for phytoplankton–bacteria relationships. Nature Microbiology 2017; 2: 17065.

Shang T, Liu L, Zhou J, Zhang M, Hu Q, Fang M, et al. Protective effects of various ratios of DHA/EPA supplementation on high-fat diet-induced liver damage in mice. Lipids Health Dis 2017; 16: 65.

Shinn EA, Steinen RP, Lidz BH, Swart PK. Whitings, a sedimentologic dilemma. Journal of Sedimentary Research 1989; 59: 147-161.

Singh SK, Major SR, Cai H, Chen F, Hill RT, Li Y. Draft Genome Sequences of Cloacibacterium normanense IMET F, a Microalgal Growth-Promoting Bacterium, and Aeromonas jandaei IMET J, a Microalgal Growth-Inhibiting Bacterium. Genome Announcements 2018; 6: e00503-18.

Skorupa DJ, Akyel A, Fields MW, Gerlach R. Facultative and anaerobic consortia of haloalkaliphilic ureolytic micro-organisms capable of precipitating calcium carbonate. J Appl Microbiol 2019; 127: 1479-1489.

Tan A, Lee Y, Carpe S, Li Y. Carbon sequestration and biomass production with simulated power plant flue gas using the microalga Nannochloropsis oceanica IMET1. 2021 Algae Biomass Summit.  2021.

Thompson JB, Schultze-Lam S, Beveridge T, Marais DDD. Whiting events: biogenic origin due to the photosynthetic activity of cyanobacterial picoplankton. Limnology and oceanography 1997; 42 1: 133-41.

Tocher DR. Omega-3 long-chain polyunsaturated fatty acids and aquaculture in perspective. Aquaculture 2015; 449: 94-107.

Tran N-AT, Seymour JR, Siboni N, Evenhuis CR, Tamburic B. Photosynthetic carbon uptake induces autoflocculation of the marine microalga Nannochloropsis oculata. Algal Research 2017; 26: 302-311.

Vieler A, Wu G, Tsai C-H, Bullard B, Cornish AJ, Harvey C, et al. Genome, Functional Gene Annotation, and Nuclear Transformation of the Heterokont Oleaginous Alga Nannochloropsis oceanica CCMP1779. PLoS Genet 2012; 8: e1003064.

Volk T, Hoffert MI. Ocean Carbon Pumps: Analysis of Relative Strengths and Efficiencies in Ocean-Driven Atmospheric CO2 Changes. The Carbon Cycle and Atmospheric CO2: Natural Variations Archean to Present, 1985, pp. 99-110.

Vu CHT, Lee H-G, Chang YK, Oh H-M. Axenic cultures for microalgal biotechnology: Establishment, assessment, maintenance, and applications. Biotechnology Advances 2018; 36: 380-396.

Wang D, Ning K, Li J, Hu J, Han D, Wang H, et al. Nannochloropsis genomes reveal evolution of microalgal oleaginous traits. PLoS Genet 2014; 10: e1004094.

Wang H, Hill RT, Zheng T, Hu X, Wang B. Effects of bacterial communities on biofuel-producing microalgae: stimulation, inhibition and harvesting. Critical Reviews in Biotechnology 2016; 36: 341-352.

Wensel P, Helms G, Hiscox B, Davis WC, Kirchhoff H, Bule M, et al. Isolation, characterization, and validation of oleaginous, multi-trophic, and haloalkaline-tolerant microalgae for two-stage cultivation. Algal Research 2014; 4: 2-11.

Xin Y, Lu Y, Lee Y-Y, Wei L, Jia J, Wang Q, et al. Producing designer oils in industrial microalgae by rational modulation of co-evolving type-2 diacylglycerol acyltransferases. Molecular Plant 2017; 10: 1523-1539.

Yao Y, Hu C, Barnes BB. Mysterious increases of whiting events in the Bahama Banks. Remote Sensing of Environment 2023; 285: 113389.

Zeebe RE. History of Seawater Carbonate Chemistry, Atmospheric CO2, and Ocean Acidification. Annual Review of Earth and Planetary Sciences 2012; 40: 141-165.

Zhu C, Chen S, Ji Y, Schwaneberg U, Chi Z. Progress toward a bicarbonate-based microalgae production system. Trends in Biotechnology 2022; 40: 180-193.

Zhu T, Dittrich M. Carbonate precipitation through microbial activities in natural environment, and their potential in biotechnology: a review. Frontiers in bioengineering and biotechnology 2016; 4: 4.