Supplementary Tables and Figures

Table S1. Improvement of lighting conditions in the 340 L-bioreactor and Nannochloropsis culture analysis during the second scale-up experiment.

|  | No culture | Day 0 culture | Day 5 culture |
| --- | --- | --- | --- |
| Light intensity (μE m-2 s-1) |  |  |  |
| Near point | 680 | 320-350 | 320-350 |
| Mid point | 450 | 80-100 | 40-55 |
| Far point | N/D | 12-35 | 8-12 |
| Temperature (oC) | N/D | 22 | 25 |
| pH | N/A | 9.0 | 10.0 |
| Optical density at 750 nm (OD750) | N/A | 0.69 | 1.84 |
| Cell density (x106 cells/ml) | N/A | 26.6 | 47.2 |

N/D, not done.

N/A, not applicable.

Figure S1. A close-up view of a horizontal culture chamber. The algae side (the right side) and the fluid side (the left side) of the culture chamber was separated by 1 μm polycarbonate membrane (black arrow). Different lines (1, media out; 2, water sampling; 3, air out; 4, air out; 5, air in; 6, media in and algae sampling) were set up with appropriate bottles. Lines 3 and 4 were connected to an air-venting bottle to prevent contaminations of the fluid side by the algae side.

Figure S2. The 340 L-bioreactor system. Components in font view (a) and rear view (b) of the system are shown. LED lights housed in the center of the bioreactor are programed in blue channel (c) or white channel (d) to illuminate the bioreactors.

Figure S3. Growth indices of the Nannochlorpsis oceanica IMET1 in the second horizonal culture chamber experiment. The culture was grown in f/2 and 34 ppt salinity. Indices included cell density (106 cells ml-2, black), optical density (OD750, red), pH on the algae side (open green square), and pH on the clear side (solid green square)

Figure S4. Phylogenetic tree of partial 16S rRNA gene sequences associated with N. oceanica IMET1 microalgal cultures from the second bioreactor experiment. Sequences from this study are in bold. Nucleotide alignment was conducted with MUSCLE and curation with a built-in Gblocks utility. The tree was constructed using the maximum likelihood method and boot strap confidence values (expressed as percentages of 1000 replications) are shown at branch nodes. Branches with bootstrap values less than 50% were eliminated. TreeDyn and Adobe Illustrator were used for tree construction.