# A Review on Socio-technical Transition Pathway to European Super Smart Grid: Trends, Challenges and Way Forward via Enabling Technologies

Herman Zahid1, Adil Zulfiqar1, Muhammad Adnan1, Sajid Iqbal1, and Salah Eldeen Gasim Mohamed2

1Department of Electrical Engineering, National University of Computer and Emerging Sciences (FAST), Chiniot-Faisalabad Campus, Chiniot 38000, Pakistan; herman.zahid@gmail.com, adil.zulfiqar@nu.edu.pk, m.adnan@nu.edu.pk, Iqbal.sajid@nu.edu.pk

2Department of Electronic Engineering, Sudan University of Science and Technology, Sudan; salahedingasim@sustech.edu

*Correspondence Authors: m.adnan@nu.edu.pk

Abstract: This study employs retrospective empirical approach to assess the innovative methods and techniques in the European union perspective with the aim to transition from smart grid to super smart grid. The European union has embarked on a significant modernization of its electricity infrastructure. This transformation is centralized on the idea of the development of a super smart grid, which leverages on emerging technologies to enhance grid intelligence and interconnectivity all across the Europe. This study critically analyses the technical, regulatory and economic steps taken by the EU to facilitate this transition. It put forward the challenges due to the integration of RES, such as intermittency, cybersecurity threats, as well as the need for advance demand-supply balance for the network. It addresses the market hurdles for grid expansion and modernization, and also include the need for updated regulatory frameworks. This study also explores the cutting-edge solutions like blockchain based decentralized energy transactions, which offers a secure platform for an efficient, and autonomous energy balance. The development of a super smart grid makes use of artificial intelligence for predictive analytics, demand forecasting, and real-time decision-making. The metaverse and digital twin formulates simulation and optimization of energy systems. Furthermore, this study put forwards a comprehensive overview of the potential challenges and issues encountered in super smart grids, emphasizing the role of regulatory reforms and innovations in technology for achieving a sustainable, intelligent and interconnected European power grid.

Keywords: Super Smart Grid, Energy, Metaverse, Digital Twin, Blockchain, Artificial Intelligence

# 1. Introduction

In numerous nations, the electricity sector has experienced a remarkable shift towards the fostering of Renewable Energy Systems (RESs) [1], marking an initial phase of systemic transformation and disruptive innovation [2]. Regulation should not exclusively prioritize the cost-efficient utilization of current infrastructure and investments in reinforcement and replacement; innovation should also be a factor to consider. This commencement of transition is mainly about the technical and economic feasibility of RESs, which enables their penetration into the electricity systems. In this case, the new narrative also goes further than just substitution of fossil fuel resources, but rather it is the consideration of how the whole electricity systems should function In this regard, the composition of RESs is complemented with finely balanced supply, demand [3], along with storage. This ensure high power quality and mitigating congestion [4] in power transmission and distribution along with energy storage. These complex combinations of factors present new demands for electricity grids’ operational models and require that all parts of the supply chain from generation to usage be reviewed in a holistic manner.

The energy and power sector is expected to play an important role in establishing a low carbon economy [5]. Across all European Union (EU) decarbonisation scenarios, renewable energy sources (RES) are integral to the solution [6,7]. However, existing grids present significant barriers to the future uptake of RES. The EU has acknowledged the need to modernize the grids by advocating the development of a "super smart grid." This initiative will interconnect European grids on the continental level and connect the grid with information and communication technology (ICT). [8]. To achieve this goal, the EU has started research and development project and established a unified legislative framework to facilitate the indispensable grid modernization across Europe [9].

Our study aims to provide both an overview and a critical assessment of the actions undertaken by the EU to transform the grids into the foundation of a decarbonized electricity system. It proposes that for an aggressive and effective deployment of super smart grids, the existing regulatory framework must be re-evaluated, and it is imperative to incorporate the enabling technologies.

## 1.1. Challenges of Conventional Grid

Even the modern power systems are prone to challenges and achieving the national goal to accommodate the RE generation is still a problem for policymakers and regulators. These challenges include maintaining stable power due to intermittency, cyber-attacks, supply-demand balance, lessen the peak-demand, uninterrupted supply and the management of digital devices [10,11]. Hence, the need is to take a step further to develop a more resilient and reliable network with vitality metrics and simulations to prove their authenticity [12]. According to [13] the foremost obstacle in achieving the required grid expansion do not stem from technical or financial constraints but are primarily associated with the absence of apt regulatory frameworks and constrained public acceptance. Moreover, to address this issue substantial modifications are required in the existing regulatory process rather than small amendments or just improving the current regulations.

In response to these challenges, the smart grids’ concept has emerged as a strategic framework, which could enhance the efficiency and resilience of the electricity systems [14].Super Smart Grids works to ease the integration of decentralized electricity generation technologies and also focus on system operation and transmission across national and international grids.

Smart grids being highly adaptable and versatile can be modified according to specific applications. However, a consensus exists regarding its fundamental attributes, which serve as foundational pillars in grasping its transformative potential [15–17]. Conceptually, the smart grid are a refined iteration of a power distribution system, augmented with renewable energy sources, power electronic devices and advanced automation. More recently, there has been a conceptual evolution of smart grids as clusters of interconnected microgrids and being labelled as cyber-physical systems  [18].

Smart grids with bi-directional flow of energy and automacy exhibits both transformative and disruptive characteristics. The now emerging ownership models with decentralised generation require novel ways to balance demand and supply. The ownership can reside to single person or energy communities, which can influence the incumbent institutions and firms of the traditional grid, thereby disrupting the energy markets and offering support to newly emerged prosumers. This sort of disruption requires transition to upgraded technologies, such as those support direct trading, system balancing and improved demand response [19–22] .

# 2. Motivation and Methodology

This review paper presents transformation pathway from smart grid to super smart grid and how it would help interconnecting all the EU member countries and allow the energy exchange between them with ease. We have also explored in detail the opportunities and challenges encountered during this transition. Super smart grid is fundamentally backed by high computation-based automation and advanced enabling technologies such as metaverse and artificial intelligence, which are discussed in this review.

In summary, this review paper answers the question, “What are the necessary steps for the transition from smart grids to super smart grids?”. Also, “Why smart grids require more upgradation?”. Another interesting question that this study address is “What is the role of enabling technologies amidst this transition?”. This question is answered by extensively reviewing the available literature body for systematic, infrastructural and techno-economic affairs. The critical point is the influence of external factors such as regulatory affairs and policy actions that has significant impact on this transition. The discussion on the role of enabling technologies in super smart grids provides a holistic view about the dynamics of elements at the core of this transition. The flow of methodology is shown in Figure 1.

The literature review indicates that most research has focused on smart grids, while the innovation of super smart grids has not yet been examined from a comprehensive socio-technical system perspective. This review is designed to identify key themes and technological advancements, as well as to understand the regulatory frameworks and economic implications associated with the transition to super smart grids. By synthesizing existing knowledge, the literature review lays a solid foundation for further analysis. A comparison with other studies is presented in .

Explored Database: The prime resource for the qualitative analysis relates to academic journals and conferences as well as review papers. It also includes industrial reports, technical standards, official documents and governmental reports, think tanks and scientific research organizations. It also includes some archives of news articles, technical blogs and website. Official documents are sourced from the European Commission's database, along with publications of the International Energy Agency (IEA). Scopus, IEEE Explore, Springer, Google Scholar, Web of Science and Wiley are used for searching scientific papers. These databases were explored with Boolean search operators for key words “Virtual Power Plants”, “innovation”, “Energy Transaction”, “regulatory affairs”, “policy”, “Technologies”, “AI”, “IoT”, “Metaverse”, and “Blockchain” etc. in combination with “Super Smart Grid”, “Smart Grid”, and “European Countries”.

Figure 1. Flow Chart for this review paper.

Inclusion-exclusion criteria: In the initial search, more than 500 journal publications, reports, website links and articles were found. The paper theme and parameters served as the key exclusion criteria. A few resources referring to power electronics and grid integration techniques related to renewable resources were excluded. Other excluded articles discussed microgrids and energy markets, which were not in the scope of this article. Moreover, some blog posts and website pages excluded because of the questionable validity of their origin.

This review paper is structured as follows. Section 3 presents the European power grid transition from smart grid to super smart grid. It also includes the novel techniques for energy transaction among various stakeholders in a super smart grid. Section 4 explains the regulatory framework and policy interventions pertinent to super smart grid. In Section 5, State of the art enabling technologies are discussed in detail. This section also presents the challenges associated with the uptake of super smart grid. Finally, the study concludes with Section 6 with discussion on emerging challenges with super smart grid and potential solutions as well as proposing suggestions for future research.

Table 1. Comparison of this study with available literature

|  | Technical Aspects |  | Social Aspects | Enabling Technologies |  |  |  |  | Socio-technical |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference | Super Smart Grid | Energy Transaction | Regulations and Policies | Metaverse and Digital Twin | Artificial Intelligence | Big Data | IoT, 5G and Edge Computing | Blockchain | Challenges and Solutions |
| [23] |  |  |  |  |  |  |  |  |  |
| [24] |  |  |  |  |  |  |  |  |  |
| [25] |  |  |  |  |  |  |  |  |  |
| [26] |  |  |  |  |  |  |  |  |  |
| [18] |  |  |  |  |  |  |  |  |  |
| [27] |  |  |  |  |  |  |  |  |  |
| [28] |  |  |  |  |  |  |  |  |  |
| This Study |  |  |  |  |  |  |  |  |  |

# 3. European Power Grid

European power grid is the most intact and resilient grids around the world, with fully integrated energy markets. This modernized infrastructure ensures the reliable and inexpensive clean energy for the consumers. However, European Union (EU) is expected to experience a rise in electricity demand by 60% till year 2030.  This high demand is attributed to increasing share of renewable energy (RE) generation, electric vehicles, heating systems, and growing demand for hydrogen production as well as storage. In realistic terms, the grid needs to be more decentralised, flexible and digitalised by the end of the decade. According to European Green Deal [29,30],[31] upgradation of the conventional grid and increasing the cross-border transmission capacity requires an investment of 584 billion euros , while ENTSO-E projects an investment of 700 billion euros till year 2050 [32].

To meet the target of achieving 50-55% energy from RE resources, European Commission (EC) needs to make significant extensions throughout the grid, both at national level as well as transnational level. In this context, the SUSPLAN project [33] assess the barriers and strategies for RE penetration in nine different European regions. It also evaluates cost and benefit of the modernized infrastructure through economic optimization under various scenarios of RE penetration. With reference to EU’s Grid Action Plan [34,35], establishing the cross-border linkages for energy transaction is a priority across Europe, with an extensive transmission grid as its backbone.

According to the EU Wind Power Action Plan [30,36], offshore wind will have a substantial role in advancing the EU's climate and energy objectives in the coming years. Member States have collaboratively established ambitious targets for offshore RE production by 2050, delineating intermediate milestones for 2030 and 2040 within each of the EU's five maritime basins. To achieve the newly established EU objective of having at least 42.5% renewable energy by 2030, with a goal to achieve 45%, necessitates a substantial augmentation in wind installed capacity. This entails a projected surge from 204 GW (in year 2022) to surpass 500 GW (by year 2030) just for wind energy.

As of 2022, the cumulative offshore installed capacity in the EU-27 reached 16.3 GW. To address the disparity between the 111 GW pledged by Member States and the existing capacity, the installation rate of about 12 GW annual is required – a magnitude that exceeds the recent 1.2 GW installed last year by a factor of ten [36–38]. However, this ambitious target comes with an array of challenges among which are highly constrained grid, unfair trade practices and irregular energy markets.

Currently, almost 40% of the EU power grid is more than 40 years old infrastructure and does not support such large penetration of RE. The conventional grid topology means concentrated power generation plants having fixed points on the grid, supplying the large load centres only, before stepping down energy at consumer level. With the emergence of distributed RE, this topology has become irrelevant and unrealistic. European countries have different energy generation and load patterns.

The future smart grid should be able to allow integration of all regional grids and maintain a balance between such a “system of systems”. An intelligent grid should be able to balance regional surplus and deficits at all times. By 2050, achieving a 100% RE in Europe is economically viable and technically attainable through the implementation of cost-effective, flexible generation methods, enhanced energy storage solutions, and increased connectivity in transmission networks [39].

## 3.1. Smart to Super Smart Grid Transformation

The transformation of conventional grids to smart grids involves substantial challenges related to policy intervention, technological upgradation and social aspect. It is essentially a convergence of ICT and the evolving energy systems [40–42]. The smart grid (SG) with European perspective is an electricity network with intelligent integration of all stakeholders – generators, consumers, and operators. They all work together to achieve the common goal of an economic, secure and sustainable electricity system.

A SG conjoins innovative technology and services in a system with inherent intelligence of control, monitoring, communication and self-healing [43–45]. Markets and commercial aspect is also a crucial part of this new landscape, backed up with regulatory framework, environmental consideration, standardization usage and societal requirement [46]. A Super Smart Grid would have the capability to connect EU member countries through a single grid, without the limits of national boundaries and ensure ease in energy exchange between countries.

The present-day electricity grid confronts multifaceted challenges and issues such as privacy concerns between energy providers and consumers [47], cybersecurity threats originating from potential cyber-attacks [48], and the need to align with national objectives for integrating green energy sources. Moreover, the grid contends with heightened intricacy in maintaining grid stability in the event of intermittent power supply, as well as necessary action to mitigating peak demand surges to optimize energy utilization and reserve adequacy.

The incessant demand for uninterrupted power supply further amplifies the complexity. Additionally, the proliferation of digitally controlled devices introduces a transformative shift in electrical load dynamics. Addressing these challenges mandates the evolution of a self-regulating, intelligent and seamlessly integrated electric grid, facilitated by modern information and communication technology (ICT) techniques for efficient data manipulation and sharing. In this evolving landscape, Smart grids offer a responsive solution to these pressing demands, thereby making foundation for a resilient and sustainable electricity network [49].

The study examines surplus power generation and the residual load scenario in 30 European countries in case of 100% RE penetration. For individual countries, approximately 24% of the total electricity consumption in a year requires balancing. However, this lessens to 15% when all countries are interconnected with unrestricted interconnectors, thereby getting benefit of the maximum achievable transmission. The unconstrained interconnectors exhibit a Net Transfer Capacity about 11.5 times greater than current values. Nonetheless, constrained interconnector capacities, at 5.7 times the current value, deliver 98% of the maximum feasible transmission benefit [50].

A research study employing "LUT Energy System Transition model" simulates two distinct pathways toward achieving a 100% RE power sector in Europe by 2050 [39]. The first scenario models regions independently, while the Area scenario incorporates transmission interconnections between regions. The modelling framework accounts for current power plant capacities and ages, along with anticipated increases in future electricity demands. The optimization results suggest that the levelized cost of electricity could potentially decrease from the 69 €/MWh to 56 €/MWh in the regional scenario and 51 €/MWh in the area scenario, enabled by the adoption of cost-effective, flexible renewable energy generation and energy storage solutions. The second scenario supports the idea of the transition from smart to super smart grid.

SG’s are imperative to take maximum advantage of most intermittent RE resources (RER) due to their capability to manage and monitor the delivery the energy in real-time by means of a communication devices for automatic reconfiguration of RER. One aspect of SG deployment is to reduce the peak demand, thus making way to incorporate new loads such as, electric vehicles. The bidirectional flow of data provides consumers with insights on electricity consumed and price. Now, in case of transition from smart grids to super-grids, serving as large-scale transmission networks between wide geographical areas, make use of high-voltage direct current (HVDC) technology. One example of the super-grid is the project under the Desertec program, which links RER from North Africa and Europe [51]. Table 2 provides a comparison of attributes in case of transition from conventional to super grid [51–53].

Table 2. Difference in attributes of Conventional, smart and super smart grid.

| Attributes | Conventional Grid | Smart Grid | Smart Super Grid |
| --- | --- | --- | --- |
| Energy Generation | Central | Distributed | Distributed |
| Grid Topology | Radial/Ring | Dispersed Network | Dispersed Network |
| Provision of Distributed RE | Constrained | Flexible | Unconstrained |
| Self-Monitoring and Self-Healing | No | Yes | Yes |
| Flow of Energy and Information | One-way | Two-way | Two-way |
| System Testing | Manuel | Remote | Autonomous Reconfiguration |
| Carbon Emissions | Abundant | Environment Friendly | Environment Friendly |
| Advanced Metering Infrastructure | No | Yes | Yes |
| Accounts Demand Response | No | Yes | Yes |
| Ease of Data Collection | No | Real Time | Real Time |
| Energy Markets | Limited Scope | Multilateral | Multilateral |
| Energy Trading | Constrained | Possible | Possible with Ease |
| Energy trading Geographical spread | Constrained | Within Restricted Area | Wide Area |
| Cyber Security | No provision | Required | Necessary |
| Overall Efficiency | Low | High | Very High |

A review on smart grid technology [49,54], put forwards possible way to manage the large data and optimization of resources through cloud computing. The scalable and adaptable features of a cloud infrastructure can address the security and latency issues of a SG. It can also support with ease the dynamic applications such as demand response, load balancing and energy demand forecasting. However, such a platform needs to be highly responsive in case of scheduling latency-sensitive applications as well as must have privacy preservation. Another way to manage this complex network is through multi-agent systems (MAS). MAS [55–58] is one of the pioneering distributed architecture for a digital market, based on Game Theory [59–62] and microeconomics [63]. Later on, it is further evolved into various models like Decision Making Assistant (DMA)[64], Problem Formulator and Attributes Evaluator (PROFATE), the Electricity Market Multi-Agent System (EMMAS) [56,65,66] and Power-Matcher (cluster based) [67]. Figure 2. 7-Layer architecture of a Smart Grid given in [27].

Figure 2. 7-Layer architecture of a Smart Grid [27]

Smart grids are now evolving by employing efficient energy management systems, better automation with internet of things, capability to balance grid with electric vehicles and providing support for big data [26]. In the futuristic super smart grid environment, strategic energy demand management can be done through fuzzy logic based controller [68]. Ease of energy storage is one of the unique features of a SG, that makes energy readily available on the grid, thereby minimizing losses. It adds to the system efficiency but also compensates the intermittency and variability of the distributed resources.

In event of supply-demand imbalance, the energy storage can balance the difference by either charging or discharging itself [69–72]. Electric vehicles, pumped hydro, hydrogen storage are the modern means of energy storage. Till year 2021, EU has made huge funding contribution in this transformational process and invested 135 million EUR in France, 155 million EUR in Germany, 157.2 million EUR in Spain. This large investment covers the projects related to DG and storage integration, smart network and DSM [73]. Table 3 Compares the indicators of SG development in the EU countries [18,74] [75–78].

Table 3. Comparing indicators of SG development in EU countries.

| Parameter | Netherlands | Germany | Denmark | Norway | France |
| --- | --- | --- | --- | --- | --- |
| Population (millions) | 17.53 | 83.13 | ‎5.85‎ | 5.40 | 67.5 |
| Total investment in SG projects (EUR/capita) | 10 | 4.5 | ‎16‎ | 6 | 4 |
| Primary energy consumption (exajoules) | 3.54 | 12.3 | ‎0.68‎ | 1.9 | 8.39 |
| Total electricity generation (GWh) | 117,440 | 250,385 | ‎32,793‎ | 39,412 | 530,418 |
| RE generation (%) | 34.8 | 41.80 | ‎78.14‎ | 98.80 | 23.25 |
| Electricity demand (MWh per capita) | 6.62 | 6.16 | ‎6.23‎ | 25.17 | 7.19 |
| PV penetration rate (%) | 11.8 | 10.9 | ‎5.0‎ | 0.1 | 3.6 |
| Wind power capacity (%) | 14.7 | 25 | ‎39.1‎ | 7.49 | 6.76 |
| Offshore power capacity (%) | 5 | 3 | ‎12‎ | 0.01 | 0.01 |
| Number of EVs per 1,000 population | 21.7 | 15.7 | ‎24.7‎ | 117.3 | 11.6 |
| Total charging station (per 1000 EV) | 200 | 38 | ‎20‎ | 30 | 68 |
| Operational battery storage (MW) | 37 | 570 | ‎2‎ | 6 | 19 |
| Effective electrochemical storage by population (W/capita) | 2.11 | 9.02 | ‎0.341‎ | 1.19 | 0.28 |
| Smart metering rolling out (%) | 85.2 | 15 | ‎80‎ | 98 | 80–90 |
| CO2 emissions per capita (tonnes) | 8.06 | 8.09 | ‎5.05‎ | 7.57 | 4.74 |
| Electricity market flexibility for DMS | Low | medium | low | low | medium |
| Level of concentration System operator | Medium | low | low | low | very high |
| Companies/organisations involved in the SG projects (count) | 476 | 835 | ‎430‎ | 245 | 680 |
| Collaborations with other EU countries for SG projects (per 1000 population) (count) | 0.31 | 0.42 | ‎3.02‎ | 0.87 | 0.41 |

## 3.2. Way Forward to Futuristic Super Smart Grids

The emergence of a Super Smart Grid (SSG) which interconnects European electricity grids at the continental scale and connects them with intelligence through ICT and advanced computation, constitutes a pivotal strategy in the European Union for the seamless integration of RES [79,80]. The SSG is a self-reliance and intelligent system with advanced control methods, most modern sensors and latest communication devices coupled with advanced technologies. It can balance supply-demand over a more wide area than a smart grid, Figure 3 is a visual representation of SSG.

The distinct attribute of super smart grids is wide area coverage and distributed generation (DG). The abundant use of renewable resource based DGs comes with critical issues i.e., transient stability and load-flow balancing of the grid [28]. A conceptual sustainable grids network is devised, linking Denmark and Norway, aimed at resolving transient stability concerns within SSGs. A comprehensive probabilistic model is crafted to bolster system stability. Findings clearly demonstrate that integrating Unified Power Flow Controller [81] proves highly effective in strengthening resilience of power system networks as well as transient stability.

Figure 3. Scalability of Super Smart Grid

Energy management [82] with computational capabilities is a pivotal part of the super smart grid environment. It enhances the grid observability; improve performance, reliability, security as well as better control over assets. It also surpasses the smart grid in terms of benefits for the improved environmental factors. While smart grids integrates renewable resources, micro-grid and demand response [83–86], the super smart grids manages the ever-increasing size and complexity of smart grids paired with advanced communication system thereby increasing the reliability [87,88] for the prosumers.

In true nature, SSG are large-scale smart grids with enhanced automation, even higher computation and complex control strategies. Connecting national and regional level smart grids, thereby making SSG could bring together various stakeholders to facilitate this transition by making collaborating platforms. Through which they can promote collaborative initiatives, share best practices, and exchange the lessons learned. This fosters innovation and progress in smart grid technology on both local and national levels [78,89].

Three principal systems of any SG are investigated in [90–93], the smart infrastructure system, the smart management system, and the smart protection system. Within the smart infrastructure system, the study delve into the constituent subsystems including smart energy, smart information, and smart communication. Within the smart management system, diverse management objectives such as enhancing energy efficiency, demand profiling, load forecasting, maximizing utility, cost reduction, and emission control, along with corresponding management strategies were analyzed [94][72,82,84,95]. Within the smart protection system, various mechanisms aimed at enhancing SG reliability, as well as the privacy and security concerns associated in SG operations were reviewed.

SSGs accelerate the unbundling process in the power sector, grow energy efficiency, and save money in electricity consumption, benefiting economic development and energy security in Europe [96]. SSG and SG allow electricity buying and selling (exchange) generated by large scale concentrated solar power plants, and can reduce climate policy costs by up to 34%, and 24%, for Western Europe and Eastern Europe respectively [97], calculated through WITCH model.

The Dual-Primal Distributed (DPDA) is an efficient algorithm with a network-independent step-size that can manage energy in smart grids. This results in faster convergence and a low gradient information exchange [98]. To ensure the security of energy transaction while energy trading in decentralized SG was explored in [99], without relying on third parties. By leveraging blockchain technology, multi-signatures and anonymous encrypted messaging streams, it presents a proof-of-concept for a decentralized power transaction system. This approach enables stakeholders to anonymously negotiate power prices and securely execute energy exchange, addressing concerns related to the security and privacy of load consumption and energy trading data.

Utilizing mean-variance portfolio optimization, [100] investigate optimal scenarios for allocating new renewable capacity nationally to guide energy decision-makers. The findings tells that the current capacity of distributed generation in Europe is sub-optimal. This suggests that a potential increase of approximately 31% in mean power supply or a reduction of about 37.5% in daily variability with a more efficient spatial distribution. Strategic planning at the European level could significantly improve this distribution imbalance. However, the optimal deployment of resources depends on the objective. This means that maximizing average output favours countries those have abundant resources.

The primary barriers to grid expansion in the European Union are neither financial nor technical, rather lack of appropriate regulatory frameworks and public acceptance, requiring major changes in the overall regulatory process [13]. This study examines excess power generation and the residual load across 30 European countries with 100% variable renewable energy penetration, assessing the benefit of power transaction between them. Using extensive weather data, hourly mismatches between demand and distributed generation are modelled. The various countries require balancing for approximately 24% of their total yearly electricity consumption. Upon networking all countries with unconstrained interconnectors, this reduces to 15%, representing the maximum achievable transmission benefit. Constrained interconnector capacities, just 5.7 times more than current values, provide 98% of this maximum benefit. This generates the need of further investigation into various constrained transmission capacity layouts for a fully renewable energy based European electricity system [50] [23,101].

SSGs relied on smart grid technologies, which are rapidly progressing in both the U.S. and Europe, with the U.S. smart grid initiative focusing on timely consumer information and control, while Europe is unified in research and development. The Smart Grid Architecture Model (SGAM) [102,103] effectively aids in designing, developing, and validating complex power and energy systems, with potential for future adaptations and extensions. A multi-layered SSG architecture is devised in this paper after reviewing all the attributes of SSG mentioned in available literature body in Figure 4.

Figure 4. Multi-layered Super Smart Grid Architecture.

## 3.3. Innovative Ways for Energy Transactions

Energy Transaction or Energy Exchange are terms used interchangeably in the literature. Transactive Energy (TE) is a system where coordinated accomplices utilize automation tools to communicate and exchange energy, adhering to grid reliability indexes and constraints. The growing number of prosumers highlights the necessity of establishing an electricity trading mechanism to facilitate peer-to-peer energy exchange. Within an electric power system, it employs market based financial transactions and control functionalities to facilitate energy exchange (trading and sharing) among prosumers, conventional and renewable power producers, storage systems, and active consumers. Transactive Energy Systems (TES) transform energy into a tradable commodity, allowing customers to trade surplus energy either in real-time or on a deferred basis [104–106].

It is ideally based on a system of control mechanisms and economics that allows the dynamic balance of demand and supply [107] through the entire grid infrastructure and allow energy give and take at multiple points across the network. This model requires equitable grid access for both demand and supply side energy resources in both bulk and retail power networks., as well as open and transparent pricing of energy services [108]. The purpose of such a system would be to combine all available resources within a specific geographic footprint in order to fulfil load at the lowest possible cost. Virtual Power Plants (VPPs) stands as a significant stride towards advancing future Sustainable Smart Grids (SSGs).

In centralized energy exchange, a central authority manages and controls transactions between prosumers. This ensures transparency and imposing penalties for contract violations. However, this central control poses significant risks to the security and privacy of participants. It adds to the time, cost, and computational burdens [1,109–111]. To address these issues, authors propose Blockchain-based decentralized energy transactions in Smart Grids (SG) [112], and Peer-to-Peer (P2P) trading [113,114] without a central authority. This approach includes models like the SolarCoin, which is a digital currency for transactions among prosumers or energy communities. This might enhance security, efficiency, and autonomy in energy trading [5,105,115,116].

Decentralized energy markets typically prioritize resilience and safety but also often at the expense of privacy. TRANSAX model enables efficient energy trading by matching possible exchange, aiding system operators in planning their energy needs. TRANSAX [109] ensures participant privacy through anonymous trading activities. It enforce safety constraints like maintaining energy flow within line capacities [117]. Prosumers and active users can be managed through Distributed Ledger Technology (DLT), particularly blockchain for energy exchange. It enables secure smart contracts between them, safeguarded by cryptographic hashes [118]. [119] highlights the 3-layer, 5 layer and 7 layer architecture as well as energy market, control and network management for a flexible power system.

In a super smart grid environment, the energy transactions would be immense and would require high computation. A possible solution is the study [120], which proposes a transactive energy system of two stages to balance distribution networks, accommodating various participant behaviours. In the first stage, an operator manages the network by purchasing energy and setting dynamic locational marginal prices (DLMPs) or utilizing storage. In the second stage, prosumers make decisions based on DLMPs and preferences. They either using grid energy or engaging in peer-to-peer (P2P) transactions. This results in reduced import costs and grid energy usage. The model converges to optimal solutions efficiently with the increase in participant numbers. Figure 5 presents the distinct features of transactive energy network.

Figure 5. Distinct features of Transactive Energy.

4. Regulatory Framework and Policy Interventions

Studies on renewable technologies analyse both transformational and systemic approaches such as [121] studies systematic problems faced by European countries in developing renewable technologies and diffusing these technologies at a slow rate. Systematic failure is attributed to market structure, where dominant characters control the market. Systematic failures as well as markets are reviewed in [122] [13,123] although it lacks to discuss transformational failures.

The development of a SG necessitates a precise regulatory framework having an important aspect of market liberalization. it is an imperative step to keep the energy prices affordable for all stakeholder as well as increase system flexibility by ensuring interoperability between various prosumers. This gives prosumers a choice to opt for suitable energy provider based on their need and financial situation [124].

For smart grid transition, the case study of Netherlands can be taken as reference, which sustains many socio-technological setbacks but continued to evolve. In Netherland, the Dutch national government’s motivation resulted in rapid adaptation of smart grid technology. Consequently, it caught the attention of policymakers and attracted the investment from firms and scientists for R&D. However, the lack of leadership, learning opportunities, regulatory frameworks [123] slowed the smart grid diffusion by stakeholders.

The energy system is subject to extensive regulation to ensure a secure electricity supply. These regulations are continually reviewed and adjusted. This is because of the need to address evolving external economic, technical, or social factors. For example, the Germany's electricity system regulation, particularly the EEG (German Renewable Energy Sources Act), has undergone multiple revisions in the past decade. These changes in regulations necessitate careful evaluations. This is done to define their effectiveness and identify any potential harmful consequences. For example, research on electricity market design in Germany indicates that the previous design of market encouraged participants to systematically under-provide energy which negatively impact system stability [125–127].

The action plan should have been to involve corporate sector and relevant stakeholders like energy communities into the policymaking. Also, upgradation of the grid to incorporate smart technologies, should be emphasized in the legislation. Turning smart grids into super smart grids can only be possible with the involvement of innovative entrepreneurs and technological start-ups in the process. They can be supported through legislation, transparent regulations and robust economic policy instruments [128].

In order to facilitate the innovation [122] by network companies, regulators found it necessary to review their regulatory frameworks. Conventionally, the principal focus of monopoly regulation was cost reduction, achieved in various countries by transitioning from cost plus to regulations based on incentives. However, to efficiently integrate increasing renewables and maximize opportunities presented by digitalization, additional incentives for innovation uptake became essential. Instances of such regulatory mechanisms include output regulation, incentivizing the investments with higher returns in innovative network, and fostering competition for innovation grants [129].

Among energy regulators, the regulatory experimentation remains relatively underutilized. Still, instead of replacing the traditional regulatory methods in fostering innovation, they enhance it. The insights gained from such experiments help bridge the information gap between regulators and innovators which help in the revision of existing regulations and also helps in the formulation of innovative ones.

In only a few Member States, regulatory frameworks for network companies include specific compensation for innovation. This can incentivize innovation by reducing financial risk for Distribution System Operators (DSOs). DSOs, subject to economic regulation, face the risk of not recovering costs if the projects fail. This is especially true for large-scale demonstrations that are subject to high risks and costs [78]. To accelerate smart grid development, it is essential to fund these projects through special mechanisms rather than relying solely on the regular regulatory framework.

To attract new market players and activities, regulators often utilize waivers as a key instrument. These waivers specify the exemptions for specific actions or types of stakeholders. They are explicitly granted by regulatory decisions and may be revisited once the innovation has matured sufficiently. Additionally, waivers can also be granted when regulators choose to overlook new activities or actors. For instance, in the case of wind and solar generators, in 2013, some European Union Member States waived financial obligations for balancing, as reported by the European Commission [130,131]. Figure 6 demonstrates the key action points for the uptake of SSG from the regulatory perspective.

Figure 6. key regulatory actions for the fast-track uplifting of super smart grids

# 5. Enabling Technologies in Super Smart Grid

This section discusses leveraging the key enabling technologies to transform the smart grids into super smart grids through innovation and improvements. In the modern world, cities generate nearly 80% of greenhouse gases and consume 75% of the total energy [132], leading to severe environmental impacts. Experts from both industry and academia recognize smart cities as the ultimate solution to the challenges posed by population growth, rapid urbanization, energy resource depletion, and environmental pollution. The foundation of smart cities are the Internet of Things (IoT) [133–135], which has evolved from conventional networks to connects billions of devices. Technological progress in wireless sensor networks (WSN), ubiquitous computing (UC), and machine-to-machine (M2M) communication have further enhanced the IoT, supporting the development of smart cities and SG [136].

Employing latest technologies has multifaceted advantages for energy industry; enhance the communication protocol for smart grids, provide virtual experience to customers and ensure supplier support. These cutting-edge technologies facilitates real-time system monitoring, remote monitoring and maintenance of assets, virtual site visits, and accurate energy accounting [25]. Furthermore, it also has social benefit as it can provide virtual environment for training of field engineers to manage power plants, thereby saving time and reducing risk [137]. The methods and techniques used by these technologies in SSG can support in optimizing resource allocation and evaluating strategic interactions. They can also extract patterns from large volumes of data, and modelling network/communication infrastructure. Moreover, it can enhance the functionality of the grid by realising it in virtual environment, which results in an immersive and real-time experience [138–140] [141,142]. Figure 7 shows the role of enabling technologies in SSG.

Figure 7. Role of Enabling Technologies in Super Smart Grid.

## 5.1. Metaverse and Digital Twin

The metaverse is a prime example of Human-Computer Interaction (HCI) and widely explored by researchers globally. It is a virtual environment, consists of immersive and intertwined digital spaces, providing users a platform to interact with computer [143][144]. Applying metaverse in SG can revolutionize and transform the grid into super smart grid. It has the potential to enhance the features of smart grids like reliability, better infrastructure, prosumer service, accessibility and economics [145]. The European Parliament expects that the metaverse market will reach 597.3 billion euros by 2030 [146]. Metaverse founds huge application in smart cities as it can help smooth integration of electric vehicles, ease of connectivity for Internet of Things [144,147], prevent cyber-physical attacks [148,149], lowers carbon emission by reducing the commute, and efficient energy management. It would be a next generation of Geographic Information System (GIS) [144], in which super smart grids would be perfectly laid out in 3D.

Digital twin as a fundamental building block of metaverse [150] is expected to reach 86.09 billion dollar market by 2028. In case of a super smart grid, it acts as a virtual representation of the whole electric network in the digital space of metaverse and uses simulation, real time data and artificial intelligence methods to help reach a decision. This technology replicates the reality and instantaneously synchronise the sensor collected real time data between digital space and physical world [151]. Like metaverse application in smart cities [152], employing it in smart grid can help in optimal resource allocation and analyse strategic interactions between elements of grid. It also extract trends from data, predict future energy supply demand and modelling of system components. In metaverse environment, grid forming components like generation sources, transmission and distribution network can be virtually modelled and analyzed. Modelling the contingency situation of grid is also an added advantage of metaverse. Figure 8. Metaverse and Digital Twin representation in SSG.

Figure 8. Metaverse and Digital Twin representation in SSG.

The creation of digital twin of energy infrastructure makes system testing more robust, reduced in cost and errors, and improved estimation of carbon emission. The metaverse provides the environment for optimizing and simulation of renewable energy resource for virtual power plants and battery storage system. Implementation of such technologies would greatly enhance the energy efficiency and management throughout the network and help prosumers as well as energy companies to make informed decisions.

## 5.2. Artificial Intelligence (AI)

AI founds its application in metaverse as a combination of computer vision, natural language processing (NLP) [153,154] and machine learning. The metaverse has roots in AI to generate the 3D animations, images, speech, smart contracts and to carry out virtual transactions [155]. In smart grids, AI can help generate virtual models; a prime example is virtual power plants (VPPs). Through machine learning and deep learning using artificial neural networks (ANN), AI empowers the system to make informed decisions based on pattern recognition and predictions. This assists in visualising novel scenarios which results in diverse, more accurate and adaptive solutions. Moreover, AI also facilitates prosumer interaction, data and grid network protection. For instance, in a study [156], authors proposed an energy management technique known as collaborative multi-agent deep reinforcement learning. It modelled the problem of energy optimization and have application in transmission power control.

Cutting-edge machine learning (ML) tools offer powerful capabilities for predicting market dynamics using publicly available data. ML techniques have been employed to forecast and balance the electricity market prices [157]. While these models excel at capturing non-linear effects and interactions for accurate predictions, they often lack interpretability. This makes them unsuitable for analyzing regulatory changes post hoc [158,159]. As a result, econometric analyses typically resort to linear models. However, explainable artificial intelligence (XAI) in energy systems is rapidly advancing, with diverse applications spanning from ensuring power grid stability to analyzing price trends. XAI models effectively differentiate between regulatory settings and enhance comprehension of their impacts on changes in the grid [124,125,160,161].

Federated Learning (FL) techniques facilitate collaborative model training across distributed devices ensuring data privacy without the necessity to transmit data to a centralized server. In Smart Grids (SG), FL mechanisms maintain data privacy [162]. This is done by conducting local training at individual stations, with only the results shared with the cloud server. Consumers receive suitable incentives based on the data obtained by the cloud server. Edge-cloud collaboration was proposed by [163,164] for privacy-preserving energy data sharing. Figure 9 shows application of artificial intelligence and machine learning algorithms with asset and ICT layers in a SG environment [23]. These enabling technologies in SG constitutes the basis of a super smart grid. Table 4. Summary of research papers within the domain of enabling technologies, SG and SSG.

Figure 9. Artificial intelligence applications in SG [23].

Grey: Artificial intelligence , Purple: communication layer, Blue: Asset layer, Red: ‎information layer

## 5.3. Big Data

As the resources are going to be scarce in future, it is crucial to implement solutions for its improved and controlled utilization. In this regard, integrated technological systems like Geographic Information Systems (GIS) and Enterprise Resource Planning (ERP) can help [165]. Such systems enable the identification of areas where energy gets wasted, improve resource distribution, control costs, and reduce consumption of natural resource. Enhancing grid reliability and efficiency through self-monitoring and feedback loops involves connecting smart meters and sensors. This can be done across generation, transmission, distribution systems, and load consumption [166]. These devices can collect real-time data on power generation, load consumption and line faults. The implementation of dynamic pricing models can lower the peak by charging higher rates during peak times and lower rates during off-peak periods. This helps to prevent outages due to high demand. Consumers receive near real-time information about their load demand. This enables them to manage consumption based on needs and cost [167]. This approach requires extensive data collection from all parts of the power system and real-time big data analytics to process this data. It send control information, thereby enhancing the overall performance of the electric power system [168].

## 5.4. IoT, 5G and Edge Computing

The metaverse has the capacity to derive valuable insights from the data acquired from real-world. This is done while prioritizing prosumer safety and inclusivity. This demands a robust IoT infrastructure due to its role in linking the physical and digital realms. IoT helps in the interconnection of devices which enable seamless data exchange and integration [136], which is crucial for the metaverse's functionality and complexity. IoT not only makes the technological foundations of the metaverse but also shapes its user experience and potential for innovation [80]. The integration of AI and IoT tools along with virtual reality helps in better understanding the energy efficiency and consumption of a system.

The applications and functionalities of the metaverse are fundamentally enhanced through 5G technology. Its ability to ensure latency, reliability and throughput essentially serves the core needs of a metaverse ecosystem. More than that, 5G represents a substantial advancement over traditional broadband in terms of lowering energy consumption and raising data rates by almost 90%. Additionally, it cuts latency by roughly 20 times and improves reliability by about 95%  [134,169,170]. Such improvements are an indispensable part to improving user experience and meeting the functional requirements for building metaverse applications. Additionally, edge computing has emerged as yet another vital supporting technology for the metaverse, which enables users to have faster response times and thus feel more engaged in the virtual world.

The vast amount of data requires efficient communication network for supporting metaverse. It should be highly responsive, high bandwidth connectivity [171] and have latency.  The Internet of Things (IoT) [172] plays a crucial role in smart grids by enabling efficient data transfer and interaction among utilities and smart devices. IoT devices, such as smart energy meters can help to identify power shortages and collect load patterns. It also helps in improving fault detection, grid restoration, and overall efficiency. This data collection leads to better customer load management, reduced energy outages, and lower consumer bills. IoT also enhances the resilience of power transmission network against natural disasters. IoT can reduces losses through enhanced sensing and advanced communication technologies. However, IoT-enabled smart grids face significant security challenges, including impersonation and unauthorized access. Other security threats include data tempering, eavesdropping, malicious code, cyber-attacks and privacy issues [173–175].

## 5.5. Block-chain

Block-chain technology has profound application in super smart grids as it simplifies the complex multi-level system [176][177]. It is a secure, decentralised, trustworthy and traceable digital ledger for keeping transactions that can ensure safeguarding the digital assets in a virtual space [178][179,180]. Blockchain works in a synchronise virtual space in a decentralized environment and allows independent nodes in synchronization. Just as monetary trading, blockchain can also work in energy trading like real-time trading with crypto currency as means of virtual purchase [181,182]. Similarly, Non fungible tokens (NFTs) can create a system of digital economy within this realm [182]. Blockchain technology has proven effective in optimizing big data applications, data sharing and data circulation. Figure 10 shows the blockchain working [24] in a potential SSG.

The future Metaverse will generate vast amounts of data because of which the current centralized data storage mechanisms are inadequate. The solution is to use Blockchain which offers promising solutions for data storage and processing in the Metaverse. Its features of non-tampering and non-traceability ensure data authenticity and high quality. This provides a foundation for the Metaverse's development, a unique virtual economic systems [177,183]. Blockchain based Distributed Ledger Technology (DLT) enables decentralized data management. DLT have the features of security of digital assets and verifies user identities. At the same time, it  establishes a trustworthy environment for transactions [184]. DLT is extensively cited in the research literature as an efficient and suitable blockchain. A comprehensive comparison between various blockchain algorithms are given in [108].

Figure 10. Role of Blockchain in Super Smart Grid [24].

To manage a crowdsourced energy system, an optimised blockchain architecture is presented in [185] for seamless peer-to-peer energy transferring as well as to operate islanded microgrids. This operation algorithm is prototyped through IBM Hyperledger Fabric, an efficient blockchain implementation. Blockchain technology plays a crucial role in virtual power plants (VPPs) by facilitating real-time energy trading through smart contracts. These contracts enable secure, transparent transactions and reflect supply and demand information, allowing for efficient two-way selection under information symmetry. Blockchain helps set fair electricity prices, reduces trust costs, improves trading efficiency, enhances the and scheduling of distributed energy resources and their coordinated control within VPPs [116,186,187].

Table 4. Summary of research papers within the domain of enabling technologies, SG and SSG.

| Reference | Year | Scope of research |
| --- | --- | --- |
| [188] | 2018 | Survey privacy preserving schemes, leaking privacy attack, countermeasures, and game theory approach. Also reviewed SG communications, standards, applications and security. |
| [189] | 2019 | Presented a review of game theory based incentive mechanisms in telecommunication within blockchain networks designed to motivate participants using consensus mechanisms. |
| [190] | 2019 | Presented a review on the application of big data analytics and machine learning in next generation of SG along with their application to load forecasting |
| [191] | 2019 | Explores blockchain applications in the SG, detailing its use in peer-to-peer energy trading, data privacy and security, energy distribution control, and equipment maintenance, while discussing commercial implementations and integration challenges. |
| [192] | 2020 | Examines smart grid communication needs, evaluates IoT protocols, identifies their shortcomings, and proposes cost-effective, high-penetration solutions while emphasizing security and reliability. |
| [193] | 2021 | Reviews cyber-physical security in SG, categorizing attacks, presenting defence strategies, and discussing future security challenges and opportunities. |
| [194] | 2021 | Explores the role of business intelligence and data analytics in managing smart grid networks with RE, highlighting their importance in operational and strategic decisions for energy companies. |
| [141] | 2022 | Surveys the fundamentals, security, and privacy of the Metaverse, exploring its architecture, key characteristics, threats, challenges, and current countermeasures, while highlighting future research directions. |
| [195] | 2022 | Explores the sociotechnical drivers and barriers to the development of a global Super-grid, a vision for a renewable-based electricity system with a vast, advanced transmission network. |
| [138] | 2022 | Explores the edge-enabled Metaverse, addressing communication, networking, and computation challenges, and examines blockchain solutions to enhance interoperability, economic transactions, and resource management. |
| [196] | 2023 | An overview of the Metaverse, its technologies, applications, challenges, and the ethical and social implications, with a focus on its transformative impact on e-commerce and other sectors. |
| [197] | 2024 | Outlines the framework of the electric digital twin grid and its applications in serving producers, prosumers, consumers, and national development. |
| [198] | 2024 | presents strategies for integrating Green Cloud Computing techniques into metaverse platforms, aiming to create sustainable and energy-efficient cloud architectures to support immersive virtual worlds. |

# 6.  Challenges in Super Smart Grid

The acceleration of super smart grid technologies requires advanced sensor technology, communication, and control strategies. However, this integration also increases the possibility of grids to cyber-attacks. These attacks can affect crucial data required for grid operations. This potentially leads to significant malfunctions. Injecting incorrect data into the system can induce grid instability and provide financial incentives for attackers [23,199–202].

SSGs are prone to cyber-physical attacks. The study delineates three key security levels for smart grids: confidentiality, availability and integrity. Various attack classification strategies have been explored, elucidating their impacts across different communication layers such as the physical, MAC, network, transport, and application layers. Detailed examinations of cyber-attacks on data integrity and confidentiality are also provided in [48,203].

Security mechanisms outlined in [204] offer protection for numerous information objects within the SG. Vulnerabilities of these objects are simulated, demonstrating their effects using defined performance metrics. Additionally, the framework for sensors and actuators necessary for Industry 4.0 services and management, is discussed in [205] that could enhance system robustness and reliability.

On the generation side, the cyber-attacks can disturb automatic voltage regulator (AVR) set-points and governor settings. This could cause disruption in the system frequency and active power control. Attacks on transmission control can disrupt operation of FACTS device. These operations include state estimation algorithms, and wide area monitoring systems (WAMS). Similarly, attacks on distribution side can cause load shedding and demand side management strategies. Once these effects are understood, supporting methodologies are implemented to detect and mitigate cyber-attacks. Emerging research challenges in smart grid cybersecurity, as discussed in [206] encompass risk modelling, mitigation algorithms and coordinated attack defence. Other research challenges in this domain include AMI security, trust management, attack attribution, and validation of incoming data.

Data management in the metaverse has to face multiple challenges [196] due to the diversity in the data formats. The various data types include structured data, location information and multimedia. Each of these requires distinct management and storage methods. Current methods such as data compression, data streaming, and cloud storage have certain limitations. These limitations are due to the processing power, data size, interoperability issues, network bandwidth [25,207].

To understand the metaverse potential, new requirements must be met: scalable infrastructure, enhanced compression algorithms, efficient data management, and seamless integration of various data types. Technical challenges like user experience optimization, real-time rendering and data synchronization also need resolution. Additionally, decisions regarding the treatment of physical and virtual data—whether separate or unified—impact data organization and performance [178]. Integrating diverse data sources and developing innovative solutions beyond conventional heterogeneous databases are crucial. For context-aware scenarios, the development of context-aware algorithms, novel operators and new approximation operators is essential [208]. Table 5. Super Smart Grid challenges and solutions.

Table 5. Super Smart Grid challenges and solutions

| Parameters | Grid Challenges | Solution via Enabling Technologies |
| --- | --- | --- |
| System Monitoring | Remote monitoring and maintenance of assets | Immersive and real time monitoring and maintenance using metaverse |
| Energy management solution (EMS) | High computation | Through virtual power plants, simulation and optimization help in EMS |
| Contingency | Ability to detect gaps and loopholes in the event of contingency | Virtual visualization of system helps in pre-planning and detecting loopholes |
| Interoperability [209][210] | Variety of software, hardware, and network protocols as well as user inputs | Maintain standards and regulations, pre-testing system virtually |
| Network and communication requirements [171,211] | High-bandwidth connectivity requirements and responsiveness | Upgradation of network infrastructure with automation |
| Security and Privacy [212,213] | Grid and user data requires high security measures and protection | Creation of a doable privacy policy for metaverse that ensure user-to-user privacy. Employing enhanced multi-factor authentication method [214]. |
| Data Management [215] | Large volume of data need to be processed and integrate. Also, heterogeneous storage need increased | Design algorithms which are context-aware and novel approximation operators. Use pf AI, IoT and fast computing methods. |
| Governance and Law [216] | Traditional institutions and physical premises required for resolving conflicts. Well crafted Energy and Power policies require that address every problem. | A decentralized governance system for virtual space having transparent conflict resolution system. Policies are crafted with the support of generative AI. |
| Monetarily Transactions | Traceable transactions with high security measures | Block-chain ensures immutability of transactions and storage. Transactions would be virtual, encrypted and anonymous. |

It is very challenging to enforce laws in the metaverse due to its virtual nature. Another reasons include the use of anonymous and encrypted cryptocurrency transactions [145,182]. This complicates accountability for data theft. In Metaverse no physical entity is present for handling complains. That is the reason behind its lack of consumer protection and traditional regulation. New legislation is needed to address virtual crimes. Even for this challenge, there exists ambiguity in determining jurisdiction and enforcing laws. This is due to the intangible nature of virtual assets and variety of legal frameworks. Additionally, the metaverse introduces challenges including privacy breaches and unauthorized access to virtual infrastructure. Other challenges include data manipulation, and the need for comprehensive security measures to safeguard both physical and cyber modules. Addressing these issues is essential for ensuring user safety in the metaverse.

Super smart grids comprising of extensive energy internet of things (EIOT) has to face critical challenges like data security, privacy as well as latency issues [217] due to high computation volume. For this purpose, authentication schemes were employed with the help of two-factor authentication (2FA), ID based single factor authentication scheme and Public Key Infrastructure (PKI) [218,219] [220,221]. Realising super smart grids in metaverse, high latency will create problem with the validity of data such as payment information. One way to manage the unbalanced computational capability (between client and server) is to use low-latency authentication and key exchange protocol LLAKEP [222].

Data uncertainty, a significant concern in designing super smart grids. It encompasses issues such as consistency, completeness, and accuracy. However, there are certain challenges like missing or inconsistent values, noise and unsynchronised data. Other issues include sensor inaccuracies and factors that alter the real-time decisions [24]. Safeguarding data availability and public trust are the two significant challenges among others in the adoption of incentive-based smart grids [223–225]. Data analytics and data mining can be the foremost options to resolve such data uncertainties. Other challenges include data security,  privacy [201,225,226], integrity , authenticity, latency, throughput, anomaly issues and real-time pricing [227–231][63].

In Metaverse the need is to overcome the challenge of integrating virtual and real spaces while optimizing the energy utilisation in the network [232]. For this purpose, key issues include estimating energy consumption with its optimal use. Addressing these challenges such as in Green Metaverse Networking [233], is essential for the power system. It will help to integrate artificial intelligence and the Metaverse. However, there is still room for much research work in exploring and developing solutions for the future power grid.

# 7. Future Research Direction

The transition into a grid reliant on transactive demand signals has quite a few problems. One such issue is the retail tariffs. Retail tariffs these days have various elements such as the costs incurred during energy generation, cost of energy transmission and distribution, cost of operating electricity markets, and costs incurred during the operations of negotiation parties involved in retailing. In most cases, retail tariffs also include the costs incurred for other purposes such as cost of federal regulations, green energy incentive programs and various other forms of energy efficiency programs. Moving towards a tariff unaffected by transactive demand signals would pose challenges due to the legal and regulatory constraints present in the changing economic ground realities of energy consumption and production.

SSGs are largely interconnected, and even minor load variation could make a huge impact on system stability. This load variation can create the voltage fluctuations and frequency in the whole system. Moreover, in case of load fluctuation, the incorporation of abundant inverter fed DG reduces the inertia of the system. Hence, the need is to devise a decoupled control strategy inclusive of inertial dynamics of the system for both voltage and frequency deviations. A study should be done on the inclusion of non-linear devices in sophisticated inertial system in super smart grid. Reliability concerns of Renewable Energy Resources (RERs) should be addressed. The unpredictable discrepancies between demand and generation arise challenges in Sustainable Smart Grids (SSGs). Achieving load flow equilibrium and transient stability becomes particularly complex in SSG. It happens especially in scenarios when unexpected outages happen such as three-phase faults (L-L-L), which makes power quality disturbances [28].

The proliferation of the metaverse also poses potential issues relevant to the enforcement of jurisdiction when doing cross-border commercial transactions, the lack of regulation of the activities occurring and the protection of consumers operating in the virtual sphere. These are critical issues that policymakers must prioritize in order to enable the regulation and enforcement of business activity and establish the relevant legal frameworks required to govern conflict resolution in regard to virtual assets. It is imperative for all stakeholders to carefully deliberate on these issues moving forward.

As SSGs rely on the incorporation of various technologies such as HVAC, FACTS devices and HVDC etc. [234], their interoperability is a significant research challenge. Offering the power infrastructure that SSGs need requires tackling the heterogeneity of the many components which are interconnected with multiple standards and technologies. The literature is void of optimal grid operation in event of energy trading in a SSG environment when multiple nodes are involved. Also, the evolution of energy markets in SSG should be studied in detail for real time price signalling, energy volatility and consumer behaviour.

In order to be able to implement, operate and control SSG transactive energy systems, a suitable legislative framework has to be created. Market participants would need to have boundaries of privacy and data integrity, energy transaction mechanisms, contracts with customer participants, rules of market price clearing that would be public. The behaviour of transactive energy systems should be more than just their real time balancing, but rather day-ahead optimisation and ancillary services like frequency control and voltage control. Additionally, attention is focused on implementing cooperative control strategies and security protocols within the SSGs power infrastructure. Optimal deployment of information and communication infrastructure, considering investment and maintenance costs, bandwidth, and coverage, is crucial for efficient energy transactions and data sharing

Considerable research is currently underway to advance smart grid technologies. However, numerous opportunities exist for future exploration across various domains within the smart grid field. These include algorithms for forecasting, power flow optimization, communication protocols, micro-grid integration, demand and energy management systems, interoperability standards, economic aspects, scalability, data security, and above all, the automation of electricity generation, transmission and distribution systems.

Additionally, defining interoperability standards for various distributed ledger technologies and enhancing network security with strong cryptographic measures are vital to ensuring customer privacy, data integrity and protection against cyber-attacks. These recommendations will support the transition from conventional power systems to decentralized smart grids and then all the way to super smart grid, improving scalability, resilience, reliability, and sustainability.

# Conclusion

This study contributes to the acceleration of the development of super smart grids by employing a new approach and exploratory super smart grid innovation from the perspectives of policy and governance. The study’s findings can assist policymakers in developing unified strategies to address previous shortcomings in policies aimed at fostering the transition to super smart grids. The qualitative analysis constitutes an important part of this review, especially while assessing the regulatory frameworks and policies across Europe. This analysis investigates the effects of market liberalization, the influence of metaverse and blockchain technologies on decentralized energy transactions. It also gives insight on how 5G and IoT technologies assist in super smart grid operations. The paper also examines the socio-economic consequences of these transitions, as well as their implications for the process of policy formulation through stakeholder engagement.

Energy transactions in super smart grids permit dynamic pricing which allows consumers to adjust their energy consumption in order to balance supply and demand. They enable distributed generation and p2p trading, improve grid performance through distribution management systems, and facilitate integration of renewables by flattening their peaks. They also improve the security of energy supply with the use of microgrids, support decarbonisation by minimising resource wastage and enable electric cars to be used as mobile batteries.

Finally, the paper conducts a thorough technological assessment, evaluating solutions such as AI for predictive analytics and the application of the metaverse for simulating and optimizing energy systems. This review has multi-faceted approach and covers the possible opportunities as well as the problems which these technologies encounter in achieving better grid management, safety and strength. In this comprehensive approach, the research seeks to integrate all insights, helping to understand the current contemplation and possible outcomes of moving towards super smart grids in Europe, while reinforcing the importance of technology, regulatory and economy interdependencies.

# References

[1]	Amin U, Hossain MJ, Tushar W, Mahmud K. Energy Trading in Local Electricity Market with Renewables - A Contract Theoretic Approach. IEEE Trans Ind Informatics 2021;17:3717–30. https://doi.org/10.1109/TII.2020.3018123.

[2]	Wilson C, Tyfield D. Critical perspectives on disruptive innovation and energy transformation. Energy Res Soc Sci 2018;37:211–5. https://doi.org/10.1016/J.ERSS.2017.10.032.

[3]	Sun S, Dong M, Liang B. Distributed Real-Time Power Balancing in Renewable-Integrated Power Grids With Storage and Flexible Loads. IEEE Trans Smart Grid 2016;7:2337–49. https://doi.org/10.1109/TSG.2015.2445794.

[4]	Bajaj M, Singh AK. Grid integrated renewable DG systems: A review of power quality challenges and state‐of‐the‐art mitigation techniques. Int J Energy Res 2020;44:26–69. https://doi.org/10.1002/ER.4847.

[5]	Xiaohu Z. Carbon Trading Based Low-Carbon Economic Dispatching for Power Grid Integrated With Wind Power System. Power Syst Technol 2013.

[6]	Gerbaulet C, Von Hirschhausen C, Kemfert C, Lorenz C, Gei PY. Scenarios for decarbonizing the European electricity sector. 2017 14th Int Conf Eur Energy Mark 2017. https://doi.org/10.1109/EEM.2017.7982017.

[7]	Zappa W, Junginger M, van den Broek M. Is a 100% renewable European power system feasible by 2050? Appl Energy 2019;233–234:1027–50. https://doi.org/10.1016/J.APENERGY.2018.08.109.

[8]	Nair NKC, Zhang L. SmartGrid: Future networks for New Zealand power systems incorporating distributed generation. Energy Policy 2009;37:3418–27. https://doi.org/10.1016/J.ENPOL.2009.03.025.

[9]	Yiwe Z. Direction of Research and Innovation in the Development of European Power Grid. Power Syst Clean Energy 2014.

[10]	Basit MA, Dilshad S, Badar R, Sami ur Rehman SM. Limitations, challenges, and solution approaches in grid‐connected renewable energy systems. Int J Energy Res 2020;44:4132–62. https://doi.org/10.1002/ER.5033.

[11]	Christopher AV, Rex S, Kumar.J A. Smart grid challenges and signal processing based solutions - A Literature survey. Int J Eng Trends Technol 2016;37:189–96. https://doi.org/10.14445/22315381/IJETT-V37P232.

[12]	Amani AM, Jalili M. Power Grids as Complex Networks: Resilience and Reliability Analysis. IEEE Access 2021;9:119010–31. https://doi.org/10.1109/ACCESS.2021.3107492.

[13]	Battaglini A, Komendantova N, Brtnik P, Patt A. Perception of barriers for expansion of electricity grids in the European Union. Energy Policy 2012;47:254–9. https://doi.org/10.1016/J.ENPOL.2012.04.065.

[14]	Kolokotsa D. The role of smart grids in the building sector. Energy Build 2016;116:703–8. https://doi.org/10.1016/J.ENBUILD.2015.12.033.

[15]	Salvadori F, Gehrke CS, De Oliveira AC, De Campos M, Sausen PS. Smart Grid Infrastructure Using a Hybrid Network Architecture. IEEE Trans Smart Grid 2013;4:1630–9. https://doi.org/10.1109/TSG.2013.2265264.

[16]	Yu X, Xue Y. Smart Grids: A Cyber–Physical Systems Perspective. Proc IEEE 2016;104:1058–70. https://doi.org/10.1109/JPROC.2015.2503119.

[17]	Dileep G. A survey on smart grid technologies and applications. Renew Energy 2020;146:2589–625. https://doi.org/10.1016/J.RENENE.2019.08.092.

[18]	Norouzi F, Hoppe T, Kamp LM, Manktelow C, Bauer P. Diagnosis of the implementation of smart grid innovation in The Netherlands and corrective actions. Renew Sustain Energy Rev 2023;175:113185. https://doi.org/10.1016/J.RSER.2023.113185.

[19]	Pereira GI, Specht JM, Silva PP, Madlener R. Technology, business model, and market design adaptation toward smart electricity distribution: Insights for policy making. Energy Policy 2018;121:426–40. https://doi.org/10.1016/j.enpol.2018.06.018.

[20]	Shomali A, Pinkse J. The consequences of smart grids for the business model of electricity firms. J Clean Prod 2016;112:3830–41. https://doi.org/10.1016/J.JCLEPRO.2015.07.078.

[21]	Gui EM, Diesendorf M, MacGill I. Distributed energy infrastructure paradigm: Community microgrids in a new institutional economics context. Renew Sustain Energy Rev 2017;72:1355–65. https://doi.org/10.1016/j.rser.2016.10.047.

[22]	Lindberg MB, Markard J, Andersen AD. Policies, actors and sustainability transition pathways: A study of the EU’s energy policy mix. Res Policy 2019;48:103668. https://doi.org/10.1016/J.RESPOL.2018.09.003.

[23]	Panda DK, Das S. Smart grid architecture model for control, optimization and data analytics of future power networks with more renewable energy. J Clean Prod 2021;301:126877. https://doi.org/10.1016/J.JCLEPRO.2021.126877.

[24]	Bhattacharya S, Chengoden R, Srivastava G, Alazab M, Javed AR, Victor N, et al. Incentive Mechanisms for Smart Grid: State of the Art, Challenges, Open Issues, Future Directions. Big Data Cogn Comput 2022, Vol 6, Page 47 2022;6:47. https://doi.org/10.3390/BDCC6020047.

[25]	Yaqoob I, Salah K, Jayaraman R, Omar M. Metaverse applications in smart cities: Enabling technologies, opportunities, challenges, and future directions. Internet of Things 2023;23:100884. https://doi.org/10.1016/J.IOT.2023.100884.

[26]	Majeed Butt O, Zulqarnain M, Majeed Butt T. Recent advancement in smart grid technology: Future prospects in the electrical power network. Ain Shams Eng J 2021;12:687–95. https://doi.org/10.1016/J.ASEJ.2020.05.004.

[27]	Zia MF, Benbouzid M, Elbouchikhi E, Muyeen SM, Techato K, Guerrero JM. Microgrid transactive energy: Review, architectures, distributed ledger technologies, and market analysis. IEEE Access 2020;8:19410–32. https://doi.org/10.1109/ACCESS.2020.2968402.

[28]	Adnan M, Ghadi Y, Ahmed I, Ali M. Transmission Network Planning in Super Smart Grids: A Survey. IEEE Access 2023;11:77163–227. https://doi.org/10.1109/ACCESS.2023.3296152.

[29]	The European Green Deal - European Commission n.d. https://commission.europa.eu/strategy-and-policy/priorities-2019-2024/european-green-deal_en (accessed May 26, 2024).

[30]	An EU Wind Power Action Plan to keep wind power a European success story - European Commission n.d. https://commission.europa.eu/news/eu-wind-power-action-plan-keep-wind-power-european-success-story-2023-10-24_en (accessed May 26, 2024).

[31]	Vela Almeida D, Kolinjivadi V, Ferrando T, Roy B, Herrera H, Vecchione Gonçalves M, et al. The “Greening” of Empire: The European Green Deal as the EU first agenda. Polit Geogr 2023;105:102925. https://doi.org/10.1016/J.POLGEO.2023.102925.

[32]	ENTSO-E’s Recommendations for the Announced EU Grid Action Plan n.d. https://www.entsoe.eu/news/2023/11/16/entso-es-recommendations-for-the-announced-eu-grid-action-plan/ (accessed May 26, 2024).

[33]	Boie I, Fernandes C, Frías P, Klobasa M. Efficient strategies for the integration of renewable energy into future energy infrastructures in Europe – An analysis based on transnational modeling and case studies for nine European regions. Energy Policy 2014;67:170–85. https://doi.org/10.1016/J.ENPOL.2013.11.014.

[34]	European Comission. Questions and Answers on the EU Action Plan for Grids. 2023.

[35]	Commission E. Actions to accelerate the roll-out of electricity grids. 2023.

[36]	Immediate actions for the European wind power industry n.d. https://ec.europa.eu/commission/presscorner/detail/e%2Bn/ip_23_5185 (accessed May 26, 2024).

[37]	EU Commission announces electricity grid action plan | Reuters n.d. https://www.reuters.com/business/energy/eu-commission-announces-electricity-grid-action-plan-2023-11-28/ (accessed May 31, 2024).

[38]	EU Grid Action Plan will help renewables, but urgent action needed on excessive connection queues | WindEurope n.d. https://windeurope.org/newsroom/press-releases/eu-grid-action-plan-will-help-renewables-but-urgent-action-needed-on-excessive-connection-queues/ (accessed May 31, 2024).

[39]	Child M, Kemfert C, Bogdanov D, Breyer C. Flexible electricity generation, grid exchange and storage for the transition to a 100% renewable energy system in Europe. Renew Energy 2019;139:80–101. https://doi.org/10.1016/J.RENENE.2019.02.077.

[40]	Agrell PJ, Bogetoft P, Mikkers M. Smart-grid Investments, Regulation and Organization. Energy Policy 2013;52:656–66. https://doi.org/10.1016/J.ENPOL.2012.10.026.

[41]	Shaukat N, Ali SM, Mehmood CA, Khan B, Jawad M, Farid U, et al. A survey on consumers empowerment, communication technologies, and renewable generation penetration within Smart Grid. Renew Sustain Energy Rev 2018;81:1453–75. https://doi.org/10.1016/J.RSER.2017.05.208.

[42]	Salkuti SR. Challenges, issues and opportunities for the development of smart grid. Int J Electr Comput Eng 2020;10:1179–86. https://doi.org/10.11591/IJECE.V10I2.PP1179-1186.

[43]	González LP, Fensel A, Berbís JMG, Popa A, Seco A de A. A Survey on Energy Efficiency in Smart Homes and Smart Grids. Energies 2021;14. https://doi.org/10.3390/EN14217273.

[44]	Ponce-Jara MA, Ruiz E, Gil R, Sancristóbal E, Pérez-Molina C, Castro M. Smart Grid: Assessment of the past and present in developed and developing countries. Energy Strateg Rev 2017;18:38–52. https://doi.org/10.1016/J.ESR.2017.09.011.

[45]	IqtiyaniIlham N, Hasanuzzaman M, Hosenuzzaman M. European smart grid prospects, policies, and challenges. Renew Sustain Energy Rev 2017;67:776–90. https://doi.org/10.1016/J.RSER.2016.09.014.

[46]	Union PO of the E. European SmartGrids Technology Platform : vision and strategy for Europe’s electricity networks of the future. 2006.

[47]	Vigurs C, Maidment C, Fell M, Shipworth D. Customer Privacy Concerns as a Barrier to Sharing Data about Energy Use in Smart Local Energy Systems: A Rapid Realist Review. Energies 2021;14. https://doi.org/10.3390/EN14051285.

[48]	Mrabet Z El, Kaabouch N, Ghazi H El, Ghazi H El. Cyber-security in smart grid: Survey and challenges. ArXiv 2018;abs/1809.02609:469–82. https://doi.org/10.1016/J.COMPELECENG.2018.01.015.

[49]	Ardito L, Procaccianti G, Menga G, Morisio M. Smart Grid Technologies in Europe: An Overview. Energies 2013, Vol 6, Pages 251-281 2013;6:251–81. https://doi.org/10.3390/EN6010251.

[50]	Rodríguez RA, Becker S, Andresen GB, Heide D, Greiner M. Transmission needs across a fully renewable European power system. Renew Energy 2014;63:467–76. https://doi.org/10.1016/J.RENENE.2013.10.005.

[51]	Smart Grids and Supergrids. Lect Notes Energy 2013;3:335–52. https://doi.org/10.1007/978-1-4471-4385-7_16.

[52]	Ohanu CP, Rufai SA, Oluchi UC. A comprehensive review of recent developments in smart grid through renewable energy resources integration. Heliyon 2024;10. https://doi.org/10.1016/j.heliyon.2024.e25705.

[53]	Kataray T, Nitesh B, Yarram B, Sinha S, Cuce E, Shaik S, et al. Integration of smart grid with renewable energy sources: Opportunities and challenges – A comprehensive review. Sustain Energy Technol Assessments 2023;58. https://doi.org/10.1016/j.seta.2023.103363.

[54]	Babatunde OM, Munda JL, Hamam Y. Power system flexibility: A review. Energy Reports 2020;6:101–6. https://doi.org/10.1016/J.EGYR.2019.11.048.

[55]	Ghorbani MJ, Choudhry MA, Feliachi A. A multiagent design for power distribution systems automation. IEEE Trans Smart Grid 2015;7:329–39.

[56]	Yu C, Zhang M, Ren F, Tan G. Multiagent learning of coordination in loosely coupled multiagent systems. IEEE Trans Cybern 2015;45:2853–67.

[57]	Kalyaev IA, Dyachenko AA, Usachev LZ, Kapustyan SG. Multiagent Management of Smart Internet Production. 2018 Glob. Smart Ind. Conf., IEEE; 2018, p. 1–8.

[58]	Li C, Savaghebi M, Vasquez JC, Guerrero JM. Multiagent based distributed control for operation cost minimization of droop controlled AC microgrid using incremental cost consensus. 2015 17th Eur. Conf. Power Electron. Appl. (EPE’15 ECCE-Europe), IEEE; 2015, p. 1–9.

[59]	Wang Y, Ai X, Tan Z, Yan L, Liu S. Interactive dispatch modes and bidding strategy of multiple virtual power plants based on demand response and game theory. IEEE Trans Smart Grid 2015;7:510–9.

[60]	Nagata T, Okamoto K. Electric power interchange between micro-grids by using multi-agent approach. 2014 IEEE Int. Conf. Autom. Sci. Eng., IEEE; 2014, p. 50–5.

[61]	Hernández L, Baladron C, Aguiar JM, Carro B, Sanchez-Esguevillas A, Lloret J, et al. A multi-agent system architecture for smart grid management and forecasting of energy demand in virtual power plants. IEEE Commun Mag 2013;51:106–13.

[62]	Esfahani MM, Hariri A, Mohammed OA. A multiagent-based game-theoretic and optimization approach for market operation of multimicrogrid systems. IEEE Trans Ind Informatics 2018;15:280–92.

[63]	Samadi P, Mohsenian-Rad A-H, Schober R, Wong VWS, Jatskevich J. Optimal real-time pricing algorithm based on utility maximization for smart grid. IeeexploreIeeeOrgP Samadi, AH Mohsenian-Rad, R Schober, VWS Wong, J Jatskevich2010 First IEEE Int Conf Smart Grid Commun 2010•ieeexploreIeeeOrg n.d.

[64]	Peik-Herfeh M, Seifi H, Sheikh-El-Eslami MK. Decision making of a virtual power plant under uncertainties for bidding in a day-ahead market using point estimate method. Int J Electr Power Energy Syst 2013;44:88–98.

[65]	Adhikaree A, Makani H, Yun J, Qiao W, Kim T. Internet of Things-enabled multiagent system for residential DC microgrids. 2017 IEEE Int. Conf. Electro Inf. Technol., IEEE; 2017, p. 100–4.

[66]	Elmitwally A, Elsaid M, Elgamal M, Chen Z. A fuzzy-multiagent service restoration scheme for distribution system with distributed generation. IEEE Trans Sustain Energy 2015;6:810–21.

[67]	Amina M, Kodogiannis VS. Load forecasting using fuzzy wavelet neural networks. IEEE Int Conf Fuzzy Syst 2011:1033–40. https://doi.org/10.1109/FUZZY.2011.6007492.

[68]	Motamedi A, Zareipour H, Rosehart WD. Electricity price and demand forecasting in smart grids. IEEE Trans Smart Grid 2012;3:664–74. https://doi.org/10.1109/TSG.2011.2171046.

[69]	Warmer C, Kok K, Karnouskos S, Weidlich A, Nestle D, Selzam P, et al. Web services for integration of smart houses in the smart grid 2009.

[70]	Tabors RD, Parker G, Caramanis MC. Development of the smart grid: Missing elements in the policy process. Proc Annu Hawaii Int Conf Syst Sci 2010. https://doi.org/10.1109/HICSS.2010.148.

[71]	Grijalva S, Tariq MU. Prosumer-based smart grid architecture enables a flat, sustainable electricity industry. IEEE PES Innov Smart Grid Technol Conf 2011. https://doi.org/10.1109/ISGT.2011.5759167.

[72]	Ringelstein J, Nestle D. APPLICATION OF BIDIRECTIONAL ENERGY MANAGEMENT INTERFACES FOR DISTRIBUTION GRID SERVICES. IET Conf Publ 2009. https://doi.org/10.1049/CP.2009.0990.

[73]	VASILJEVSKA J, GANGALE F, COVRIG L, MENGOLINI AM. Smart Grids and Beyond – An EU research and innovation perspective n.d. https://doi.org/10.2760/705655.

[74]	International Renewable Energy Agency IRENA. Renewable Capacity Statistics 2021. 2021.

[75]	Global EV Outlook 2021 – Analysis - IEA n.d. https://www.iea.org/reports/global-ev-outlook-2021 (accessed May 27, 2024).

[76]	Renewable Energy Agency I. INNOVATION LANDSCAPE FOR A RENEWABLE-POWERED FUTURE: SOLUTIONS TO INTEGRATE VARIABLE RENEWABLES 2019.

[77]	Annual patents filed in sustainable energy, 2021 n.d. https://ourworldindata.org/grapher/patents-for-renewables-by-country (accessed May 27, 2024).

[78]	GANGALE F, VASILJEVSKA J, COVRIG C-F, MENGOLINI AM, FULLI G. Smart grid projects outlook 2017: facts, figures and trends in Europe n.d. https://doi.org/10.2760/15583.

[79]	de Sépibus J. The Integration of Electricity from Renewable Energy Sources in the European Union Electricity Market – The case for “Smart Grids.” Sépibus, Joëlle (July 2013) Integr Electr from Renew Energy Sources Eur Union Electr Mark – Case “Smart Grids” (NCCR Trade Work Pap 38) 2013. https://doi.org/10.7892/BORIS.49795.

[80]	Encarnação BC da. A Smart Future? The EU Digital Agenda Between Broadband, the Grid and Energy Efficiency 2013. https://doi.org/10.7892/BORIS.49795.

[81]	Alsuwian T, Basit A, Amin AA, Adnan M, Ali M. An Optimal Control Approach for Enhancing Transients Stability and Resilience in Super Smart Grids. Electron 2022, Vol 11, Page 3236 2022;11:3236. https://doi.org/10.3390/ELECTRONICS11193236.

[82]	Cecati C, Citro C, Siano P. Combined Operations of Renewable Energy Systems and Responsive Demand in a Smart Grid. IEEE Trans Sustain Energy 2011;2:468–76. https://doi.org/10.1109/TSTE.2011.2161624.

[83]	Mnatsakanyan A, Kennedy SW. A novel demand response model with an application for a virtual power plant. IEEE Trans Smart Grid 2014;6:230–7.

[84]	Gelazanskas L, Gamage KAA. Demand side management in smart grid: A review and proposals for future direction. Sustain Cities Soc 2014;11:22–30. https://doi.org/10.1016/j.scs.2013.11.001.

[85]	Alzahrani A, Ferdowsi M, Shamsi P, Dagli CH. Modeling and Simulation of Microgrid. Procedia Comput Sci 2017;114:392–400. https://doi.org/10.1016/j.procs.2017.09.053.

[86]	Porsinger T, Janik P, Leonowicz Z, Gono R. Modelling and optimization in microgrids. Energies 2017;10:1–22. https://doi.org/10.3390/en10040523.

[87]	Cortes CA, Contreras SF, Shahidehpour M. Microgrid topology planning for enhancing the reliability of active distribution networks. IEEE Trans Smart Grid 2018;9:6369–77.

[88]	Bie Z, Zhang P, Li G, Hua B, Meehan M, Wang X. Reliability evaluation of active distribution systems including microgrids. IEEE Trans Power Syst 2012;27:2342–50.

[89]	Arefifar SA, Yasser A-RM, El-Fouly THM. Optimum microgrid design for enhancing reliability and supply-security. IEEE Trans Smart Grid 2013;4:1567–75.

[90]	Fang X, Misra S, Xue G, Yang D. Smart grid - The new and improved power grid: A survey. IEEE Commun Surv Tutorials 2012;14:944–80. https://doi.org/10.1109/SURV.2011.101911.00087.

[91]	Xu G, Yu W, Griffith D, Golmie N, Moulema P. Toward Integrating Distributed Energy Resources and Storage Devices in Smart Grid. IEEE Internet Things J 2017;4:192–204. https://doi.org/10.1109/JIOT.2016.2640563.

[92]	Keyhani A. Design of smart power grid renewable energy systems. John Wiley & Sons; 2016.

[93]	Morais H, Sousa T, Faria P, Vale Z. Reactive power management strategies in future smart grids. 2013 IEEE Power Energy Soc. Gen. Meet., IEEE; 2013, p. 1–5.

[94]	Yu X, Cecati C, Dillon T, Simes MG. The new frontier of smart grids. IEEE Ind Electron Mag 2011;5:49–63. https://doi.org/10.1109/MIE.2011.942176.

[95]	Jurado S, Nebot À, Mugica F, Avellana N. Hybrid methodologies for electricity load forecasting: Entropy-based feature selection with machine learning and soft computing techniques. Energy 2015;86:276–91. https://doi.org/10.1016/J.ENERGY.2015.04.039.

[96]	Ludynia A. Smart Grids in the Process of Building a Competitive Economy and Energy Security in Europe. Found Manag 2012;4:37–50. https://doi.org/10.2478/FMAN-2013-0009.

[97]	Ricci EC. An Integrated Assessment of Super & Smart Grids. SSRN Electron J 2013. https://doi.org/10.2139/SSRN.2340415.

[98]	Guo L, Shi X, Zhou Y, Cao J. A Distributed Algorithm for Optimal Energy Management With Network-Independent Stepsize. Int Conf Inf Softw Technol 2023:424–31. https://doi.org/10.1109/ICIST59754.2023.10367147.

[99]	Aitzhan NZ, Svetinovic D. Security and Privacy in Decentralized Energy Trading Through Multi-Signatures, Blockchain and Anonymous Messaging Streams. IEEE Trans Dependable Secur Comput 2018;15:840–52. https://doi.org/10.1109/TDSC.2016.2616861.

[100]	Santos-Alamillos FJ, Brayshaw DJ, Methven J, Thomaidis NS, Ruiz-Arias JA, Pozo-Vázquez D. Exploring the meteorological potential for planning a high performance European electricity super-grid: optimal power capacity distribution among countries. Environ Res Lett 2017;12:114030. https://doi.org/10.1088/1748-9326/AA8F18.

[101]	IEC SRD 63200 - SGAM basics - SyC Smart Energy n.d. https://syc-se.iec.ch/deliveries/sgam-basics/ (accessed May 27, 2024).

[102]	Uslar M, Rohjans S, Neureiter C, Andrén FP, Velasquez J, Steinbrink C, et al. Applying the Smart Grid Architecture Model for Designing and Validating System-of-Systems in the Power and Energy Domain: A European Perspective. Energies 2019, Vol 12, Page 258 2019;12:258. https://doi.org/10.3390/EN12020258.

[103]	Simoes MG, Roche R, Kyriakides E, Suryanarayanan S, Blunier B, McBee KD, et al. A comparison of smart grid technologies and progresses in Europe and the U.S. IEEE Trans Ind Appl 2012;48:1154–62. https://doi.org/10.1109/TIA.2012.2199730.

[104]	Ron Melton and Jason Fuller. Transactive energy: Envisioning the future. 2016.

[105]	Safak Bayram I, Shakir MZ, Abdallah M, Member S, Qaraqe K. A Survey on Energy Trading in Smart Grid 2014.

[106]	Shabanzadeh M, Sheikh-El-Eslami MK, Haghifam MR. Risk-based medium-term trading strategy for a virtual power plant with first-order stochastic dominance constraints. IET Gener Transm Distrib 2017. https://doi.org/10.1049/iet-gtd.2016.1072.

[107]	Chen S, Liu CC. From demand response to transactive energy: state of the art. J Mod Power Syst Clean Energy 2017;5:10–9. https://doi.org/10.1007/S40565-016-0256-X/TABLES/2.

[108]	Huang Q, Amin W, Umer K, Gooi HB, Eddy FYS, Afzal M, et al. A review of transactive energy systems: Concept and implementation. Energy Reports 2021;7:7804–24. https://doi.org/10.1016/J.EGYR.2021.05.037.

[109]	Laszka A, Eisele S, Dubey A, Karsai G, Kvaternik K. TRANSAX: A Blockchain-Based Decentralized Forward-Trading Energy Exchanged for Transactive Microgrids. Proc Int Conf Parallel Distrib Syst - ICPADS 2018;2018-December:918–27. https://doi.org/10.1109/PADSW.2018.8645001.

[110]	Yahaya AS, Javaid N, Javed MU, Shafiq M, Khan WZ, Aalsalem MY. Blockchain-Based Energy Trading and Load Balancing Using Contract Theory and Reputation in a Smart Community. IEEE Access 2020;8:222168–86. https://doi.org/10.1109/ACCESS.2020.3041931.

[111]	Masaud TM, Warner J, El-Saadany EF. A Blockchain-Enabled Decentralized Energy Trading Mechanism for Islanded Networked Microgrids. IEEE Access 2020;8:211291–302. https://doi.org/10.1109/ACCESS.2020.3038824.

[112]	Ahl A, Yarime M, Tanaka K, Sagawa D. Review of blockchain-based distributed energy: Implications for institutional development. Renew Sustain Energy Rev 2019;107:200–11. https://doi.org/10.1016/J.RSER.2019.03.002.

[113]	Phadke A, Antonio Medrano F, Brahmbhatt J, Ustymenko S. A Framework for an Optimized Smart Energy System. 2022 Int Symp Electr Electron Inf Eng 2022:240–6. https://doi.org/10.1109/ISEEIE55684.2022.00049.

[114]	UNFCCC. ME SOLshare: Peer-to-Peer Smart Village Grids | Bangladesh. United Nations Framew Conv Clim Chang 2020.

[115]	Rehman S, Khan B, Arif J, Ullah Z, Aljuhani AJ, Alhindi A, et al. Bi-Directional Mutual Energy Trade between Smart Grid and Energy Districts Using Renewable Energy Credits. Sensors (Basel) 2021;21. https://doi.org/10.3390/S21093088.

[116]	Shabanzadeh M, Sheikh-El-Eslami MK, Haghifam MR. An interactive cooperation model for neighboring virtual power plants. Appl Energy 2017. https://doi.org/10.1016/j.apenergy.2017.05.066.

[117]	Eisele S, Eghtesad T, Campanelli K, Agrawal P, Laszka A, Dubey A. Safe and private forward-trading platform for transactive microgrids. ACM Trans Cyber-Physical Syst 2021;5. https://doi.org/10.1145/3403711.

[118]	Siano P, De Marco G, Rolan A, Loia V. A Survey and Evaluation of the Potentials of Distributed Ledger Technology for Peer-to-Peer Transactive Energy Exchanges in Local Energy Markets. IEEE Syst J 2019;13:3454–66. https://doi.org/10.1109/JSYST.2019.2903172.

[119]	Gupta N, Prusty BR, Alrumayh O, Almutairi A, Alharbi T. The Role of Transactive Energy in the Future Energy Industry: A Critical Review. Energies 2022, Vol 15, Page 8047 2022;15:8047. https://doi.org/10.3390/EN15218047.

[120]	Zomorodi Moghadam A, Javidi MH. Designing a two-stage transactive energy system for future distribution networks in the presence of prosumers’ P2P transactions. Electr Power Syst Res 2022;211:108202. https://doi.org/10.1016/J.EPSR.2022.108202.

[121]	Negro SO, Alkemade F, Hekkert MP. Why does renewable energy diffuse so slowly? A review of innovation system problems. Renew Sustain Energy Rev 2012;16:3836–46. https://doi.org/10.1016/J.RSER.2012.03.043.

[122]	Wieczorek AJ, Hekkert MP. Systemic instruments for systemic innovation problems: a framework for policy makers and innovation scholars. Sci Public Policy 2012;39:74–87. https://doi.org/10.1093/SCIPOL/SCR008.

[123]	Shafie-khah M, Moghaddam MP, Sheikh-El-Eslami MK, Rahmani-Andebili M. Modeling of interactions between market regulations and behavior of plug-in electric vehicle aggregators in a virtual power market environment. Energy 2012;40:139–50.

[124]	Pütz S, Kruse J, Witthaut D, Hagenmeyer V, Schäfer B. Regulatory Changes in German and Austrian Power Systems Explored with Explainable Artificial Intelligence. E-Energy 2023 Companion - Proc 14th ACM Int Conf Futur Energy Syst 2023:26–31. https://doi.org/10.1145/3599733.3600247.

[125]	Kruse J, Schäfer B, Witthaut D. Revealing drivers and risks for power grid frequency stability with explainable AI. Patterns 2021;2. https://doi.org/10.1016/J.PATTER.2021.100365.

[126]	Ehrhart KM, Ocker F. Design and regulation of balancing power auctions: an integrated market model approach. J Regul Econ 2021;60:55–73. https://doi.org/10.1007/S11149-021-09430-7.

[127]	Eicke A, Ruhnau O, Hirth L. Electricity balancing as a market equilibrium: An instrument-based estimation of supply and demand for imbalance energy. Energy Econ 2021;102. https://doi.org/10.1016/J.ENECO.2021.105455.

[128]	Schittekatte T, Meeus L, Jamasb T, Llorca M. Regulatory experimentation in energy: Three pioneer countries and lessons for the green transition. Energy Policy 2021;156:112382. https://doi.org/10.1016/J.ENPOL.2021.112382.

[129]	Jamasb T;, Llorca M;, Meeus L;, Schittekatte T. Energy Network Innovation for Green Transition Economic Issues and Regulatory Options n.d. https://doi.org/10.22439/csei.pb.008.

[130]	Union PO of the E. Support schemes for renewable energy n.d. https://energy.ec.europa.eu/topics/renewable-energy/financing/support-schemes-renewable-energy_en (accessed May 31, 2024).

[131]	Union PO of the E. European Commission guidance for the design of renewables support schemes [SWD(2013) 439] - European Commission 2024. https://energy.ec.europa.eu/publications/european-commission-guidance-design-renewables-support-schemes-swd2013-439_en (accessed May 31, 2024).

[132]	United Nations. Urban Energy | UN-Habitat n.d. https://unhabitat.org/topic/urban-energy (accessed May 27, 2024).

[133]	Salam A. Internet of things in sustainable energy systems. Internet of Things, 2020, p. 183–216. https://doi.org/10.1007/978-3-030-35291-2_6.

[134]	Hossein Motlagh N, Mohammadrezaei M, Hunt J, Zakeri B. Internet of Things (IoT) and the Energy Sector. Energies 2020;13:494. https://doi.org/10.3390/en13020494.

[135]	Ren J, Wang Z, Luo Z, Liu F. Smart Grid and Electric Power Informatization. J. Phys. Conf. Ser., IOP Publishing; 2019. https://doi.org/10.1088/1742-6596/1187/2/022017.

[136]	Silva BN, Khan M, Han K. Towards sustainable smart cities: A review of trends, architectures, components, and open challenges in smart cities. Sustain Cities Soc 2018;38:697–713. https://doi.org/10.1016/J.SCS.2018.01.053.

[137]	What role does the metaverse play in the future of energy? - Good New Energy n.d. https://goodnewenergy.enagas.es/en/innovative/what-paper-does-the-metaverse-occupy-in-the-future-of-the-energy/ (accessed May 20, 2024).

[138]	Xu M, Ng WC, Yang W, Lim B, Kang J, Xiong Z, et al. A full dive into realizing the edge-enabled metaverse: Visions, enabling technologies, and challenges. IeeexploreIeeeOrgM Xu, WC Ng, WYB Lim, J Kang, Z Xiong, D Niyato, Q Yang, X Shen, C MiaoIEEE Commun Surv Tutorials, 2022•ieeexploreIeeeOrg n.d.

[139]	The next economy: creating new ways to shop, work, and play - Fast Company n.d. https://www.fastcompany.com/90771640/the-next-economy-creating-new-ways-to-shop-work-and-play (accessed May 20, 2024).

[140]	Virtual power plants: Metaverse for the power sector - Opinion News | The Financial Express n.d. https://www.financialexpress.com/opinion/virtual-power-plants-metaverse-for-the-power-sector/2511907/ (accessed May 20, 2024).

[141]	Wang Y, Su Z, Zhang N, Xing R, Liu D, Luan TH, et al. A Survey on Metaverse: Fundamentals, Security, and Privacy. IEEE Commun Surv Tutorials 2023;25:319–52. https://doi.org/10.1109/COMST.2022.3202047.

[142]	Sparkes M. What is a metaverse. New Sci 2021;251:18. https://doi.org/10.1016/S0262-4079(21)01450-0.

[143]	Mckinsey. Value creation in the metaverse | McKinsey n.d. https://www.mckinsey.com/capabilities/growth-marketing-and-sales/our-insights/value-creation-in-the-Metaverse (accessed May 20, 2024).

[144]	Allam Z, Sharifi A, Bibri SE, Jones DS, Krogstie J. The Metaverse as a Virtual Form of Smart Cities: Opportunities and Challenges for Environmental, Economic, and Social Sustainability in Urban Futures. Smart Cities 2022, Vol 5, Pages 771-801 2022;5:771–801. https://doi.org/10.3390/SMARTCITIES5030040.

[145]	Vidal-Tomás D. The illusion of the metaverse and meta-economy. Int Rev Financ Anal 2023;86:102560. https://doi.org/10.1016/J.IRFA.2023.102560.

[146]	POLONA C, André MT, Maria N. Metaverse: Opportunities, risks and policy implications 2022.

[147]	Zainab HE, Bawanay NZ. Digital Twin, Metaverse and Smart Cities in a Race to the Future. 2023 24th Int Arab Conf Inf Technol ACIT 2023 2023. https://doi.org/10.1109/ACIT58888.2023.10453803.

[148]	Liu Y, Shen Y, Guo C, Tian Y, Wang X, Zhu Y, et al. MetaSensing in Metaverses: See There, Be There, and Know There. IEEE Intell Syst 2022;37:7–12. https://doi.org/10.1109/MIS.2022.3221844.

[149]	Smart Cities – Cities of the Future | Microsoft Industry n.d. https://www.microsoft.com/en-us/industry/government/resources/smart-cities (accessed May 20, 2024).

[150]	Lv Z, Xie S, Li Y, Shamim Hossain M, El Saddik A. Building the metaverse using digital twins at all scales,states, and relations. Virtual Real Intell Hardw 2022;4:459–70. https://doi.org/10.1016/J.VRIH.2022.06.005.

[151]	Virtual smart cities using Metaverse, digital twins - Bizness Transform n.d. https://www.biznesstransform.com/virtual-smart-cities-using-metaverse-digital-twins/ (accessed May 13, 2024).

[152]	Angelidou M. Smart cities: A conjuncture of four forces. Cities 2015;47:95–106. https://doi.org/10.1016/J.CITIES.2015.05.004.

[153]	Huynh-The T, Pham QV, Pham XQ, Nguyen TT, Han Z, Kim DS. Artificial intelligence for the metaverse: A survey. Eng Appl Artif Intell 2023;117:105581. https://doi.org/10.1016/J.ENGAPPAI.2022.105581.

[154]	The Role of AI in Shaping the Metaverse · Neil Sahota n.d. https://www.neilsahota.com/the-role-of-ai-in-shaping-the-metaverse/ (accessed May 13, 2024).

[155]	AI: The driving force behind the metaverse? - ITU n.d. https://www.itu.int/hub/2022/06/ai-driving-force-metaverse/ (accessed May 13, 2024).

[156]	Yang H, Zhong W De, Chen C, Alphones A, Xie X. Deep-reinforcement-learning-based energy-efficient resource management for social and cognitive internet of things. IEEE Internet Things J 2020;7:5677–89. https://doi.org/10.1109/JIOT.2020.2980586.

[157]	Merten M, Rücker F, Schoeneberger I, Sauer DU. Automatic frequency restoration reserve market prediction: Methodology and comparison of various approaches. Appl Energy 2020;268. https://doi.org/10.1016/J.APENERGY.2020.114978.

[158]	Lago J, Marcjasz G, De Schutter B, Weron R. Forecasting day-ahead electricity prices: A review of state-of-the-art algorithms, best practices and an open-access benchmark. Appl Energy 2020;293. https://doi.org/10.1016/J.APENERGY.2021.116983.

[159]	Yang W, Sun S, Hao Y, Wang S. A novel machine learning-based electricity price forecasting model based on optimal model selection strategy. Energy 2022;238. https://doi.org/10.1016/J.ENERGY.2021.121989.

[160]	Barredo Arrieta A, Díaz-Rodríguez N, Del Ser J, Bennetot A, Tabik S, Barbado A, et al. Explainable Artificial Intelligence (XAI): Concepts, Taxonomies, Opportunities and Challenges toward Responsible AI. Inf Fusion 2019;58:82–115. https://doi.org/10.1016/J.INFFUS.2019.12.012.

[161]	Trebbien J, Gorjão LR, Praktiknjo A, Schäfer B, Witthaut D. Understanding electricity prices beyond the merit order principle using explainable AI. Energy AI 2022. https://doi.org/10.48550/ARXIV.2212.04805.

[162]	Sun Z, Feng J, Yin L, Zhang Z, … RL-, Continua M&, et al. Fed-DFE: A Decentralized Function Encryption-Based Privacy-Preserving Scheme for Federated Learning. CdnTechscienceCnZ Sun, J Feng, L Yin, Z Zhang, R Li, Y Hu, C NaComputers, Mater Contin 2022•cdnTechscienceCn n.d. https://doi.org/10.32604/cmc.2022.022290.

[163]	Su Z, Wang Y, Luan TH, Zhang N, Li F, Chen T, et al. Secure and Efficient Federated Learning for Smart Grid with Edge-Cloud Collaboration. IEEE Trans Ind Informatics 2022;18:1333–44. https://doi.org/10.1109/TII.2021.3095506.

[164]	Boudko S, Abie H, Nigussie E, WINSYS RS-, 2021  undefined. Towards Federated Learning-based Collaborative Adaptive Cybersecurity for Multi-microgrids. ScitepressOrgS Boudko, H Abie, E Nigussie, R SavolaWINSYS, 2021•scitepressOrg n.d. https://doi.org/10.5220/0010580700830090.

[165]	Al Nuaimi E, Al Neyadi H, Mohamed N, Al-Jaroodi J. Applications of big data to smart cities. J Internet Serv Appl 2015;6:1–15. https://doi.org/10.1186/S13174-015-0041-5/FIGURES/3.

[166]	Yin J, Sharma P, Gorton I, Akyol B. Large-scale data challenges in future power grids. Proc - 2013 IEEE 7th Int Symp Serv Syst Eng SOSE 2013 2013:324–8. https://doi.org/10.1109/SOSE.2013.71.

[167]	Grid Modernization and the Smart Grid | Department of Energy n.d. https://www.energy.gov/oe/grid-modernization-and-smart-grid (accessed May 26, 2024).

[168]	Mohamed N, Al-Jaroodi J. Real-time big data analytics: Applications and challenges. Proc 2014 Int Conf High Perform Comput Simulation, HPCS 2014 2014:305–10. https://doi.org/10.1109/HPCSIM.2014.6903700.

[169]	Indium. Role of IoT in Metaverse n.d. https://www.indiumsoftware.com/blog/ (accessed May 13, 2024).

[170]	5 key technologies that power the Metaverse - Excellarate n.d. https://www.encora.com/insights/5-key-technologies-that-power-the-metaverse?excellarate-is-now-encora (accessed May 13, 2024).

[171]	Network Requirements for the Metaverse, Are We Ready? | iceConnect n.d. https://www.iceconnect.com/blog/it-support/metaverse-network-requirements/ (accessed May 20, 2024).

[172]	Chukwuemeka CM, Uchechi MI, Obinna CO, Modesta E, Chimezie UB, Nnamdi AI. Optimizing the Sustainability of Renewable Energy: A Review on the Impart of Internet of Things. Asian J Res Comput Sci 2020. https://doi.org/10.9734/ajrcos/2020/v5i430147.

[173]	Ciavarella S, Joo J, Smart SS-IT on, 2016  undefined. Managing contingencies in smart grids via the internet of things. IeeexploreIeeeOrgS Ciavarella, JY Joo, S SilvestriIEEE Trans Smart Grid, 2016•ieeexploreIeeeOrg n.d.

[174]	Haider H, See O, Energy WE-R and S, 2016  undefined. A review of residential demand response of smart grid. ElsevierHT Haider, OH See, W ElmenreichRenewable Sustain Energy Rev 2016•Elsevier n.d.

[175]	Science CB-PC, 2014  undefined. Security issues and challenges for the IoT-based smart grid. ElsevierC BekaraProcedia Comput Sci 2014•Elsevier n.d.

[176]	Andoni M, Robu V, Flynn D, Abram S, Geach D, Jenkins D, et al. Blockchain technology in the energy sector: A systematic review of challenges and opportunities 2018. https://doi.org/10.1016/j.rser.2018.10.014.

[177]	Shi F, Ning H, Zhang X, Li R, Tian Q, Zhang S, et al. A new technology perspective of the Metaverse: Its essence, framework and challenges. Digit Commun Networks 2023. https://doi.org/10.1016/J.DCAN.2023.02.017.

[178]	Huynh-The T, Gadekallu TR, Wang W, Yenduri G, Ranaweera P, Pham QV, et al. Blockchain for the metaverse: A Review. Futur Gener Comput Syst 2023;143:401–19. https://doi.org/10.1016/J.FUTURE.2023.02.008.

[179]	Ryu J, Son S, Lee J, Park Y, Park Y. Design of Secure Mutual Authentication Scheme for Metaverse Environments Using Blockchain. IEEE Access 2022;10:98944–58. https://doi.org/10.1109/ACCESS.2022.3206457.

[180]	Chen J, Xiao H, Hu M, Chen CM. A blockchain-based signature exchange protocol for metaverse. Futur Gener Comput Syst 2023;142:237–47. https://doi.org/10.1016/J.FUTURE.2022.12.031.

[181]	Maksymyuk T, Gazda J, Bugár G, … VG-I, 2022  undefined. Blockchain-empowered service management for the decentralized metaverse of things. IeeexploreIeeeOrgT Maksymyuk, J Gazda, G Bugár, V Gazda, M Liyanage, M DohlerIEEE Access, 2022•ieeexploreIeeeOrg n.d.

[182]	Vidal-Tomás D. The new crypto niche: NFTs, play-to-earn, and metaverse tokens. Financ Res Lett 2022;47:102742. https://doi.org/10.1016/J.FRL.2022.102742.

[183]	Jeon H, Youn H, Ko S, the TK-A in, 2022  undefined. Blockchain and AI Meet in the Metaverse. BooksGoogleComHJ Jeon, HC Youn, SM Ko, TH KimAdvances Converg Blockchain Artif Intell 2022•booksGoogleCom n.d.

[184]	Kuru K. MetaOmniCity: Toward Immersive Urban Metaverse Cyberspaces Using Smart City Digital Twins. IEEE Access 2023;11:43844–68. https://doi.org/10.1109/ACCESS.2023.3272890.

[185]	Wang S, Taha AF, Wang J, Kvaternik K, Hahn A. Energy Crowdsourcing and Peer-to-Peer Energy Trading in Blockchain-Enabled Smart Grids. IEEE Trans Syst Man, Cybern Syst 2019;49:1612–23. https://doi.org/10.1109/TSMC.2019.2916565.

[186]	Lu J, Wu S, Cheng H, Xiang Z. Smart contract for distributed energy trading in virtual power plants based on blockchain. Comput Intell 2021;37:1445–55. https://doi.org/10.1111/COIN.12388.

[187]	Kolenc M, Nemček P, Gutschi C, Suljanović N, Zajc M. Performance evaluation of a virtual power plant communication system providing ancillary services. Electr Power Syst Res 2017. https://doi.org/10.1016/j.epsr.2017.04.010.

[188]	Ferrag MA, Maglaras LA, Janicke H, Jiang J, Shu L. A systematic review of data protection and privacy preservation schemes for smart grid communications. Sustain Cities Soc 2018;38:806–35. https://doi.org/10.1016/J.SCS.2017.12.041.

[189]	Wang W, Hoang DT, Hu P, Xiong Z, Niyato D, Wang P, et al. A Survey on Consensus Mechanisms and Mining Strategy Management in Blockchain Networks. IEEE Access 2018;7:22328–70. https://doi.org/10.1109/ACCESS.2019.2896108.

[190]	Hossain E, Khan I, Un-Noor F, Sikander SS, Sunny MSH. Application of Big Data and Machine Learning in Smart Grid, and Associated Security Concerns: A Review. IEEE Access 2019;7:13960–88. https://doi.org/10.1109/ACCESS.2019.2894819.

[191]	Alladi T, Chamola V, Rodrigues JJPC, Kozlov SA. Blockchain in Smart Grids: A Review on Different Use Cases. Sensors 2019, Vol 19, Page 4862 2019;19:4862. https://doi.org/10.3390/S19224862.

[192]	Tightiz L, Yang H. A Comprehensive Review on IoT Protocols’ Features in Smart Grid Communication. Energies 2020, Vol 13, Page 2762 2020;13:2762. https://doi.org/10.3390/EN13112762.

[193]	Zhang H, Liu B, Wu H. Smart Grid Cyber-Physical Attack and Defense: A Review. IEEE Access 2021;9:29641–59. https://doi.org/10.1109/ACCESS.2021.3058628.

[194]	Colmenares-Quintero RF, Quiroga-Parra DJ, Rojas N, Stansfield KE, Colmenares-Quintero JC. Big Data analytics in Smart Grids for renewable energy networks: Systematic review of information and communication technology tools. Cogent Eng 2021;8. https://doi.org/10.1080/23311916.2021.1935410.

[195]	Hojckova K, Ahlborg H, Sandén BA. A global super-grid: sociotechnical drivers and barriers. Energy Sustain Soc 2022;12:1–16. https://doi.org/10.1186/S13705-022-00368-Y/FIGURES/2.

[196]	De Felice F, De Luca C, Chiara S Di, Petrillo A. Physical and digital worlds: implications and opportunities of the metaverse. Procedia Comput Sci 2023;217:1744–54. https://doi.org/10.1016/J.PROCS.2022.12.374.

[197]	Sifat MMH, Choudhury SM, Das SK, Ahamed MH, Muyeen SM, Hasan MM, et al. Towards electric digital twin grid: Technology and framework review. Energy AI 2023;11:100213. https://doi.org/10.1016/J.EGYAI.2022.100213.

[198]	Kasture S, Maurya S, Verma R, Pathak N, Joshi P, Rakesh N. Green Cloud Computing for the Metaverse: Powering the Future of the Internet with Renewable Energy 2024:1–6. https://doi.org/10.1109/ICRITO61523.2024.10522167.

[199]	Ruland KC, Sassmannshausen J, Waedt K, Zivic N. Smart Grid Security – ein Überblick über Standards und Richtlinien. Elektrotechnik Und Informationstechnik 2017;134:19–25. https://doi.org/10.1007/S00502-017-0472-8/METRICS.

[200]	Langer L, Skopik F, Smith P, Kammerstetter M. From old to new: Assessing cybersecurity risks for an evolving smart grid. Comput Secur 2016;62:165–76. https://doi.org/10.1016/J.COSE.2016.07.008.

[201]	Leszczyna R. Cybersecurity and privacy in standards for smart grids – A comprehensive survey. Comput Stand Interfaces 2018;56:62–73. https://doi.org/10.1016/J.CSI.2017.09.005.

[202]	Neureiter C, Engel D, Trefke J, Santodomingo R, Rohjans S, Uslar M. Towards consistent smart grid architecture tool support: From use cases to visualization. IEEE PES Innov Smart Grid Technol Conf Eur 2015;2015-January. https://doi.org/10.1109/ISGTEUROPE.2014.7028834.

[203]	Wang W, Lu Z. Cyber security in the Smart Grid: Survey and challenges. Comput Networks 2013;57:1344–71. https://doi.org/10.1016/J.COMNET.2012.12.017.

[204]	Hahn A, Govindarasu M. Cyber attack exposure evaluation framework for the smart grid. IEEE Trans Smart Grid 2011;2:835–43. https://doi.org/10.1109/TSG.2011.2163829.

[205]	Batista NC, Melício R, Mendes VMF. Services enabler architecture for smart grid and smart living services providers under industry 4.0. Energy Build 2017;141:16–27. https://doi.org/10.1016/J.ENBUILD.2017.02.039.

[206]	Sridhar S, Hahn A, Govindarasu M. Cyber-physical system security for the electric power grid. Proc IEEE 2012;100:210–24. https://doi.org/10.1109/JPROC.2011.2165269.

[207]	Fu Y, Li C, Yu FR, Luan TH, Zhao P, Liu S. A Survey of Blockchain and Intelligent Networking for the Metaverse. IEEE Internet Things J 2023;10:3587–610. https://doi.org/10.1109/JIOT.2022.3222521.

[208]	Al-Ghaili AM, Kasim H, Al-Hada NM, Hassan Z Bin, Othman M, Tharik JH, et al. A Review of Metaverse’s Definitions, Architecture, Applications, Challenges, Issues, Solutions, and Future Trends. IEEE Access 2022;10:125835–66. https://doi.org/10.1109/ACCESS.2022.3225638.

[209]	Jaimini U, Zhang T, Computing GB-II, 2022  undefined. iMetaverseKG: Industrial Metaverse Knowledge Graph to Promote Interoperability in Design and Engineering Applications. IeeexploreIeeeOrgU Jaimini, T Zhang, GO Brikis, A ShethIEEE Internet Comput 2022•ieeexploreIeeeOrg n.d.

[210]	Interoperability in the Metaverse | World Economic Forum n.d. https://www.weforum.org/publications/interoperability-in-the-metaverse/ (accessed May 31, 2024).

[211]	The metaverse and communication service providers | Nokia n.d. https://www.nokia.com/blog/the-metaverse-and-communication-service-providers/ (accessed May 20, 2024).

[212]	Oleksy T, Wnuk A, Piskorska M. Migration to the metaverse and its predictors: Attachment to virtual places and metaverse-related threat. Comput Human Behav 2023;141:107642. https://doi.org/10.1016/J.CHB.2022.107642.

[213]	Kuo SY, Tseng FH, Chou YH. Metaverse intrusion detection of wormhole attacks based on a novel statistical mechanism. Futur Gener Comput Syst 2023;143:179–90. https://doi.org/10.1016/J.FUTURE.2023.01.017.

[214]	Qamar S, Anwar Z, Afzal M. A systematic threat analysis and defense strategies for the metaverse and extended reality systems. Comput Secur 2023;128:103127. https://doi.org/10.1016/J.COSE.2023.103127.

[215]	Data Management For The Metaverse – ACM SIGMOD Blog n.d. https://wp.sigmod.org/?p=3481 (accessed May 20, 2024).

[216]	No country has jurisdiction over the metaverse (yet) | BSA Middle East Law Firm n.d. https://bsabh.com/knowledge-hub/news/no-country-has-jurisdiction-over-the-metaverse-yet (accessed May 20, 2024).

[217]	Chen Z, Gan W, Wu J, Lin H, Chen CM. Metaverse for smart cities: A survey. Internet Things Cyber-Physical Syst 2024;4:203–16. https://doi.org/10.1016/J.IOTCPS.2023.12.002.

[218]	Ometov A, Bezzateev S, Mäkitalo N, Andreev S, Mikkonen T, Koucheryavy Y. Multi-factor authentication: A survey. Cryptography 2018;2:1–31. https://doi.org/10.3390/CRYPTOGRAPHY2010001.

[219]	Petsas T, Tsirantonakis G, … EA-P of the, 2015  undefined. Two-factor authentication: is the world ready? Quantifying 2FA adoption. DlAcmOrgT Petsas, G Tsirantonakis, E Athanasopoulos, S IoannidisProceedings Eighth Eur Work Syst Secur 2015•dlAcmOrg 2015. https://doi.org/10.1145/2751323.2751327.

[220]	Wang D, He D, Wang P, on CC-IT, 2014  undefined. Anonymous two-factor authentication in distributed systems: Certain goals are beyond attainment. IeeexploreIeeeOrgD Wang, D He, P Wang, CH ChuIEEE Trans Dependable Secur Comput 2014•ieeexploreIeeeOrg 2014;12:428–42. https://doi.org/10.1109/TDSC.2014.2355850.

[221]	Lee L-H, Braud T, Zhou P, Wang L, Xu D, Lin Z, et al. All One Needs to Know about Metaverse: A Complete Survey on Technological Singularity, Virtual Ecosystem, and Research Agenda 2021.

[222]	Zhang X, Huang X, Yin H, Huang J, Chai S, Xing B, et al. LLAKEP: A Low-Latency Authentication and Key Exchange Protocol for Energy Internet of Things in the Metaverse Era. Math 2022, Vol 10, Page 2545 2022;10:2545. https://doi.org/10.3390/MATH10142545.

[223]	Connor PM, Axon CJ, Xenias D, Balta-Ozkan N. Sources of risk and uncertainty in UK smart grid deployment: An expert stakeholder analysis. Energy 2018;161:1–9. https://doi.org/10.1016/J.ENERGY.2018.07.115.

[224]	Vaccaro A, Pisica I, Lai LL, Zobaa AF. A review of enabling methodologies for information processing in smart grids. Int J Electr Power Energy Syst 2019;107:516–22. https://doi.org/10.1016/J.IJEPES.2018.11.034.

[225]	Hu J, Vasilakos A V. Energy Big Data Analytics and Security: Challenges and Opportunities. IEEE Trans Smart Grid 2016;7:2423–36. https://doi.org/10.1109/TSG.2016.2563461.

[226]	Tu C, He X, Shuai Z, Energy FJ-R and S, 2017  undefined. Big data issues in smart grid–A review. ElsevierC Tu, X He, Z Shuai, F JiangRenewable Sustain Energy Rev 2017•Elsevier 2017;79:1099–107. https://doi.org/10.1016/j.rser.2017.05.134.

[227]	Dalipi F, … SY-C on FI of T, 2016  undefined. Security and privacy considerations for iot application on smart grids: Survey and research challenges. IeeexploreIeeeOrgF Dalipi, SY Yayilgan2016 IEEE 4th Int Conf Futur Internet of, 2016•ieeexploreIeeeOrg n.d.

[228]	Shen H, Zhang M, Information JS-IT on, 2017  undefined. Efficient privacy-preserving cube-data aggregation scheme for smart grids. IeeexploreIeeeOrgH Shen, M Zhang, J ShenIEEE Trans Inf Forensics Secur 2017•ieeexploreIeeeOrg 2017;12:1369. https://doi.org/10.1109/TIFS.2017.2656475.

[229]	Tahir M, Khan A, Hameed A, Masoom Alam M, Alam M, Khurram Khan M, et al. Towards a set aggregation-based data integrity scheme for smart grids. SpringerM Tahir, A Khan, A Hameed, M Alam, MK Khan, F JabeenAnnals Telecommun 2017•Springer 2017;72:551–61. https://doi.org/10.1007/s12243-017-0602-7.

[230]	Khan MW, Zeeshan M, Farid A, Usman M. QoS-aware traffic scheduling framework in cognitive radio based smart grids using multi-objective optimization of latency and throughput. Ad Hoc Networks 2020;97. https://doi.org/10.1016/J.ADHOC.2019.102020.

[231]	Wan Yen S, Morris S, Ezra MAG, Jun Huat T. Effect of smart meter data collection frequency in an early detection of shorter-duration voltage anomalies in smart grids. Int J Electr Power Energy Syst 2019;109:1–8. https://doi.org/10.1016/J.IJEPES.2019.01.039.

[232]	Zhao J, Zhou X, Li Y, Qian L. Optimizing Utility-Energy Efficiency for the Metaverse over Wireless Networks under Physical Layer Security. Proc Twenty-Fourth Int Symp Theory, Algorithmic Found Protoc Des Mob Networks Mob Comput 2023;23:250–9. https://doi.org/10.1145/3565287.3610271.

[233]	Zhang S, Lim WYB, Ng WC, Xiong Z, Niyato D, Shen X, et al. Towards Green Metaverse Networking Technologies, Advancements and Future Directions. ArXiv n.d.;abs/2211.03057. https://doi.org/10.48550/ARXIV.2211.03057.

[234]	Feltes JW, Gemmell BD, Retzmann D. From smart grid to super grid: Solutions with HVDC and FACTS for grid access of renewable energy sources. IEEE Power Energy Soc Gen Meet 2011. https://doi.org/10.1109/PES.2011.6039346.