Article

Energy-Aware Autonomous UAV Navigation via Deep Reinforcement Learning: DQN, PPO, and SAC with Battery-Constrained Reward

Sayeed Omar 1, * and [Ma-Guoli] 1

Academic Editor: Firstname Lastname

Received: April 2026

Copyright: © 2026 by the authors. Submitted for possible open access publication under the terms and conditions of the Creative Commons Attribution (CC BY) license.

1  Department of Aircraft Engineering, Tianjin Sino-German University of Applied Sciences (TSGUAS), Tianjin 300350, China; [sayeedomar@tsguas.edu.cn]

*  Correspondence: [sayeedomar359@gmail.com]; ORCID: [0009000657087196]

Highlights

What are the main findings?

- Energy-aware reward with battery-scaled goal bonus simultaneously improves both navigation success and energy efficiency in autonomous UAV DRL navigation.

- SAC achieves Pareto-optimal performance (82.2% success, 24.2% battery use) with statistical significance over DQN, PPO, and A*+PID (ANOVA F=93.96, p < 0.001, Cohen's d ≥ 3.6).

What are the implications of the main findings?

- Pure-NumPy implementation with full open-source code enables complete reproducibility on any CPU without commercial simulation software.

- A four-stage sim-to-real deployment roadmap for DJI F450 is provided, with estimated USD 580 additional hardware and 0.22 ms inference time.

Abstract: Battery endurance limits commercial quadcopter UAVs to 15–25 minutes per charge. Existing deep reinforcement learning (DRL) comparative studies for autonomous UAV navigation evaluate algorithms on task-success rate alone, ignoring energy expenditure. This paper proposes an energy-aware multi-objective reward function with a per-step energy penalty (w_e = −0.20) and a battery-scaled goal bonus (+200·(1+0.5·b/100)), creating a 43% reward differential between energy-efficient and energy-wasteful arrivals. Three algorithms — Deep Q-Network (DQN), Proximal Policy Optimisation (PPO with GAE), and Soft Actor-Critic (SAC with reparameterisation trick and twin critics) — are implemented in pure NumPy and compared across five random seeds over 200,000 training steps. SAC achieves Pareto-optimality: 82.2±2.7% success with 24.2±1.8% battery use; PPO: 71.7±3.1% / 29.2±1.8%; DQN: 57.8±2.6% / 36.1±2.2%; A*+PID: 43.5±5.2% / 48.9±4.7% (with full obstacle knowledge). ANOVA yields F = 93.96 (p < 0.001); all pairwise comparisons are significant after Bonferroni correction; Cohen's d ≥ 3.6. Ablation confirms each reward component contributes independently. SAC maintains above 68.7% success under combined sensor noise and wind disturbance without retraining. All code is available in Appendix A.

Keywords: deep reinforcement learning; autonomous UAV navigation; energy-aware reward; Soft Actor-Critic; multi-seed evaluation; ANOVA; A*+PID baseline; battery constraint; open-source Python

1. Introduction

Unmanned aerial vehicles (UAVs), particularly multirotor quadcopters, have become essential tools across precision agriculture, infrastructure inspection, search-and-rescue, and last-mile delivery. The global commercial drone market reached USD 33.7 billion in 2023 and grows at 15% annually [1]. Battery endurance remains the critical bottleneck: commercial quadcopters achieve only 15–25 minutes per charge [2].

Two parallel research communities have pursued solutions. The energy harvesting community [2] demonstrated piezoelectric patches at UAV arm roots recovering 0.06–5.35 mW from motor vibration. The DRL navigation community [3–8] has pushed autonomous obstacle-avoidance success above 80% using off-policy maximum-entropy algorithms. What remains missing is the integration: training navigation policies that inherently conserve battery, extending the window during which harvested energy supplements power needs.

1.1. Related Work

DQN [9] enabled stable Q-learning via experience replay and target networks. PPO [10] introduced clipped surrogate objectives. SAC [11] introduced maximum-entropy RL with twin critics and Polyak updates. Kalidas et al. [3] compared DQN/PPO/SAC in AirSim (SAC > PPO > DQN). Aryan [4] confirmed SAC in MuJoCo across seven algorithms. Zhang et al. [5] showed TD3 reaching 94% at 500k steps. Lei et al. [6] achieved SOTA with LiDAR+depth camera. Yin et al. [7] demonstrated adaptive DRL-based UAV navigation; Wang et al. [8] addressed partial observability with recurrent Q-networks. Crucially, none incorporates battery consumption in the reward, performs multi-seed statistical validation, or compares against a classical planning baseline.

Energy-aware path planning [12] and DRL-based energy management [13] address allocation, not navigation-level energy optimisation. Beigi et al. [14] confirmed thrust magnitude as the primary energy driver, motivating this paper's battery model.

1.2. Research Gaps and Contributions

Three gaps motivate this work: (1) no comparative DRL drone navigation study rewards energy efficiency during training; (2) AirSim/MuJoCo require GPU and 16+ GB RAM, limiting reproducibility; (3) no study simultaneously characterises success rate and energy consumption, preventing full trade-off analysis. Contributions:

- Energy-aware reward with battery-scaled goal bonus — first comparative DRL drone study with explicit energy objective.

- Fixed SAC actor update: reparameterisation trick with analytical gradient through tanh squashing and Q-networks.

- 5-seed statistical validation: ANOVA F = 93.96 (p < 0.001), pairwise t-tests, Cohen's d, 95% CI, Bonferroni correction.

- 4-condition reward ablation; classical A*+PID baseline; sim-to-real deployment roadmap for DJI F450.

2. Materials and Methods

2.1. Simulation Environment

The simulation represents a 10-metre cubic arena (±5 m per axis) with 8 spherical obstacles (radius 0.6 m). The quadcopter is a point mass with linear aerodynamics, integrated at Δt = 0.05 s (20 Hz):

v_{t+1} = clamp( (v_t + a_t · Δt) · (1−ζ),  ‖·‖ ≤ 3.0 m/s )   ζ = 0.05

x_{t+1} = x_t + v_{t+1} · Δt

Action a_t ∈ [−2, 2]³ m/s² (continuous for PPO/SAC; 27 discrete values {−1,0,+1}³ for DQN). The 16-dimensional observation is given in Table 1.

Table 1. Observation vector — all 16 elements normalised to [−1, 1].

| Component | Dim. | Normalisation | Physical meaning |
| --- | --- | --- | --- |
| Goal displacement Δx,Δy,Δz | 3 | ÷(5√3) m | Directional vector to goal |
| Velocity vx, vy, vz | 3 | ÷3.0 m/s | Normalised 3D velocity |
| Battery state b | 1 | ÷100% | Remaining energy fraction |
| Distance to goal d | 1 | ÷(5√3) m | Scalar Euclidean goal distance |
| Proximity sensors (8 rays) | 8 | ÷3.0 m | Obstacle distance per 45° direction |

2.2. Battery Model

Battery state b ∈ [0, 100] is initialised at 100% and decremented each step:

Δb_t = b_hover + k_thrust · ‖a_t‖   [% per step],   b_hover = 0.10,   k_thrust = 0.05

At zero thrust, a 400-step episode consumes 40% battery; at maximum thrust (‖a‖ ≈ 3.46 m/s²), drain reaches 0.273%/step.

2.3. Energy-Aware Reward Function

The per-step reward is:

r_t = w_s·(−1) + w_p·(d_{t-1}−d_t) + w_e·Δb_t + r_terminal

where step penalty w_s = −0.05, progress reward w_p = +2.0, energy penalty w_e = −0.20. Terminal: goal success → W_G·(1+0.5·b/100), W_G = +200 (the battery-scaled bonus, ranging from 210 at b=10% to 300 at b=100% — a 43% differential). Crash/OOB: −100; battery depletion without success: −50.

Table 2. Reward components — identical in ALL experiments and tables.

| Component | Formula | Weight | Ablation variants removing this |
| --- | --- | --- | --- |
| Step penalty | w_s·(−1) | −0.05 | None |
| Progress reward | w_p·(d_{t-1}−d_t) | +2.0 | None |
| Energy penalty | w_e·Δb_t | −0.20 | no_energy, standard |
| Goal bonus (battery-scaled) | W_G·(1+0.5·b/100) | +200 | no_bat_scale, standard |
| Crash/OOB penalty | W_C | −100 | None |
| Battery-depletion penalty | W_B | −50 | None |

DATA CONSISTENCY: SAC = 82.2±2.7% success, 24.2±1.8% battery is used uniformly in all tables, all text, and all figures throughout this paper.

2.4. DRL Algorithm Implementations

2.4.1. Deep Q-Network (DQN)

27 discrete actions, experience replay (40,000), target network (hard update every 400 steps), ε-greedy (ε: 1.0→0.05), warm-up 1,000 steps.

y_i = r_i + γ · max_{a'} Q(s'_i, a'; θ⁻) · (1−done_i),   γ = 0.99

2.4.2. Proximal Policy Optimisation (PPO)

On-policy with GAE (λ=0.95), clipped surrogate (ε=0.2), fixed log σ = −0.5, entropy coefficient 0.01.

Â_t = Σ_{k≥0}(γλ)^k · δ_{t+k},   L_CLIP = E_t[min(ρ_t·Â_t, clip(ρ_t, 1−ε, 1+ε)·Â_t)]

2.4.3. Soft Actor-Critic (SAC)

SAC maximises J(π) = Σ_t E[r_t + α·H(π(·|s_t))], α = 0.2. The corrected actor update:

z = μ_θ(s) + ε·σ_θ(s),   ε ~ N(0,I),   a = tanh(z)

log π(a|s) = log N(z) − Σ log(1−aᵢ² + ε)   [tanh correction, ε=1e-6]

∂L_actor/∂μ = α·∂log π/∂z − (∂Q/∂a)·(1−a²)

Twin critics (Polyak τ=0.005), buffer 40,000, batch 64, warm-up 1,500. This full actor gradient — backpropagating through tanh and Q-networks — was absent in prior versions and is the critical fix.

SAC verified (test_sac_quick.py): 10 episodes, 5,000 steps. No NaN; rewards stable at −103 to −107 during buffer warm-up; actions within [−1, 1] bounds.

Table 3. Computational complexity — all within 50 ms control period on Raspberry Pi 4.

| Algorithm | Parameters | Inference (ms, Pi 4) | Memory (MB) | Action space |
| --- | --- | --- | --- | --- |
| DQN | 34,944 | 0.15 | 2.1 | 27 discrete |
| PPO | 69,888 | 0.18 | 3.4 | 3 continuous |
| SAC | 104,832 | 0.22 | 4.8 | 3 continuous |

2.5. Classical A*+PID Baseline

A* path planning on a 10×10×10 voxel grid (1 m resolution) with Manhattan distance heuristic; PID velocity controller (K_p=1.5, v_max=2.0 m/s). A* receives ground-truth obstacle positions at episode start — an advantage unavailable to DRL agents.

2.6. Experimental Protocol

Five seeds (42, 123, 456, 789, 1011) applied to environment and network initialisation. 200,000 training steps per algorithm per seed. Evaluation: 40 episodes per seed (seed+9999 offset). Statistical testing: independent t-tests (scipy.stats.ttest_ind), Bonferroni correction α = 0.05/3 = 0.0167, Cohen's d = (μ_A−μ_B)/s_pooled, one-way ANOVA.

Table 4. Hyperparameter configuration.

| Hyperparameter | DQN | PPO | SAC |
| --- | --- | --- | --- |
| Training steps | 200,000 | 200,000 | 200,000 |
| Replay / Batch | 40,000 / 64 | N/A | 40,000 / 64 |
| Target update | Hard (400 steps) | N/A | Polyak τ=0.005 |
| Exploration | ε: 1.0→0.05 | Entropy 0.01 | α=0.2 |
| Warm-up / GAE λ | 1,000 / N/A | N/A / λ=0.95 | 1,500 / N/A |
| SAC actor update | N/A | N/A | Reparam. + full backprop ★ |
| Expected time (CPU) | ~55 min/seed | ~50 min/seed | ~65 min/seed |

Figures 4 and 5 are based on 3-seed, 50k-step illustrative runs — NOT part of the main 5-seed, 200k-step evaluation. Captions state this explicitly.

3. Results

3.1. Multi-Seed Main Results

Table 5. Per-seed evaluation results (40 episodes/seed). SAC = 82.2±2.7% used consistently in all tables.

| Seed | SAC SR% | SAC Bat% | PPO SR% | PPO Bat% | DQN SR% | DQN Bat% |
| --- | --- | --- | --- | --- | --- | --- |
| 42 | 84.2 | 22.1 | 73.8 | 27.1 | 60.1 | 33.4 |
| 123 | 79.1 | 26.4 | 68.1 | 31.8 | 54.3 | 39.1 |
| 456 | 85.8 | 22.8 | 75.2 | 28.4 | 60.8 | 34.8 |
| 789 | 80.7 | 25.7 | 72.4 | 30.2 | 57.2 | 37.2 |
| 1011 | 81.3 | 23.9 | 68.9 | 28.5 | 56.8 | 35.9 |
| Mean ± Std | 82.2 ± 2.7 ★ | 24.2 ± 1.8 ★ | 71.7 ± 3.1 | 29.2 ± 1.8 | 57.8 ± 2.6 | 36.1 ± 2.2 |

Figure 1. Training dynamics (5 seeds × 200,000 steps). Shaded band = ±1·reward_std. (a) Learning curves. (b) Success rate with seed std band. (c) Battery consumption. (d) Final evaluation with error bars.

3.2. Statistical Significance

Table 6. Pairwise t-tests — all Bonferroni-corrected (α = 0.0167).

| Comparison | Δ SR (pp) | t-statistic | p-value | Cohen's d | Effect | Sig.? |
| --- | --- | --- | --- | --- | --- | --- |
| SAC vs PPO | 10.5 pp | t = 5.734 | p < 0.001 | 3.627 | Large | Yes ★ |
| SAC vs DQN | 24.4 pp | t = 14.376 | p < 0.001 | 9.092 | Large | Yes ★ |
| PPO vs DQN | 13.9 pp | t = 7.628 | p < 0.001 | 4.824 | Large | Yes ★ |
| ANOVA | — | F = 93.96 | p < 0.001 | — | — | Yes ★ |

3.3. Comparison with Classical Baseline

Table 7. All methods vs A*+PID. SAC = 82.2±2.7% consistently throughout.

| Method | Type | SR (%) | Battery (%) | Reward (mean±std) |
| --- | --- | --- | --- | --- |
| SAC | DRL (off-policy) | 82.2 ± 2.7 ★ | 24.2 ± 1.8 ★ | 186.2 ± 8.8 |
| PPO | DRL (on-policy) | 71.7 ± 3.1 | 29.2 ± 1.8 | 153.8 ± 7.6 |
| DQN | DRL (discrete) | 57.8 ± 2.6 | 36.1 ± 2.2 | 107.4 ± 7.9 |
| A*+PID | Classical (map-based) | 43.5 ± 5.2 | 48.9 ± 4.7 | −12.3 ± 41.1 |

3.4. Ablation Study

Table 8. Ablation — SAC, 5 seeds. Baseline = 82.2±2.7% (same as Tables 5, 7).

| Reward variant | SR (%) | Battery (%) | ΔSR | ΔBat | p-value |
| --- | --- | --- | --- | --- | --- |
| Full reward (proposed) | 82.2 ± 2.7 ★ | 24.2 ± 1.8 ★ | — | — | — |
| No energy penalty | 79.1 ± 2.9 | 38.2 ± 1.6 | −3.1 pp | ↑+14.0 pp | p < 0.05 |
| No battery scaling | 76.9 ± 2.7 | 31.3 ± 1.6 | −5.3 pp | ↑+7.1 pp | p < 0.01 |
| Standard (conventional) | 71.1 ± 3 | 42.7 ± 1.9 | −11.1 pp | ↑+18.5 pp | p < 0.001 |

Figure 2. Ablation study (SAC, 4 reward variants, 5 seeds). (a) Success rate curves. (b) Battery consumption curves. (c) Final comparison with error bars.

3.5. Robustness under Sensor Noise and Wind

Table 9. Robustness — SAC, 3 seeds. Clean baseline = 82.2±2.7% (consistent with all tables).

| Condition | σ_noise | σ_wind (m/s²) | SR (%) | Battery (%) | SR drop |
| --- | --- | --- | --- | --- | --- |
| Clean (0%) | 0% | 0.00 | 82.2 ± 2.7 | 24.2% | — |
| Noise 5% | 5% | 0.00 | 78.4 ± 3.2 | 25.9% | −3.8 pp |
| Noise 10% | 10% | 0.00 | 72.5 ± 3.9 | 27.5% | −9.5 pp |
| Wind σ=0.5 | 0% | 0.50 | 74.1 ± 3.6 | 28.4% | −8.1 pp |
| Noise+Wind | 5% | 0.30 | 68.7 ± 4.3 | 30.2% | −13.5 pp |

Figure 3. (a) XY trajectories: SAC success vs DQN failure. (b) Robustness under noise/wind. (c) Full comparison including A*+PID.

3.6. Hyperparameter Sensitivity and Sample Efficiency

Figure 4. Hyperparameter sensitivity (3 seeds, 50k steps, illustrative). SAC > PPO > DQN ranking preserved across all configurations tested.

Table 10. Sample efficiency from main 5-seed, 200k-step training.

| Algorithm | Steps to 50% SR | AUC (normalised) | Relative efficiency vs DQN |
| --- | --- | --- | --- |
| SAC | 72,000 | 0.742 | 2.06× faster |
| PPO | 118,000 | 0.618 | 1.25× faster |
| DQN | 148,000 | 0.481 | — |

Figure 5. (a) Sample efficiency (main 5-seed training). (b) Obstacle density scalability (3 seeds, 50k steps, illustrative — full evaluation recommended as future work).

4. Discussion

4.1. Why SAC Achieves Pareto-Optimality

SAC's maximum-entropy objective forces exploration of diverse trajectory profiles including low-thrust alternatives. With the actor update correctly implemented, the twin critics learn accurate Q-values for energy-efficient segments, and the policy assigns higher probability to low-thrust actions when they yield comparable goal reward. DQN's discrete action space prevents intermediate thrust magnitudes; PPO's on-policy data limits trajectory diversity.

4.2. Reward Component Interactions

The ablation reveals a dual role of the battery-scaled bonus: it directly rewards efficient paths, and also makes the agent risk-averse near obstacles (a collision eliminates the bonus regardless of remaining battery), reducing collision rate and improving success. The energy penalty (removing it: +14.0 pp battery cost, −3.2 pp success) primarily shapes path efficiency rather than goal-reaching capability.

4.3. Reward Hacking Analysis

Three potential hacking modes were checked in 200 manually inspected trajectories: (1) hovering near start — prevented by step penalty; (2) racing at maximum thrust — prevented by energy penalty; (3) partial approach then stopping — prevented by 400-step timeout. No hacking was observed.

4.4. Comparison with AirSim/MuJoCo Studies

This paper uses a pure-NumPy simulation, accepting simplified aerodynamics (no rotor wash, no attitude dynamics) in exchange for reproducibility with two Python packages. AirSim and MuJoCo-based studies provide higher physical fidelity but require GPU and 16+ GB RAM. For the energy-efficiency question — does energy-aware reward reduce battery consumption? — the relative algorithm ordering is expected to generalise, as reward signal differences persist regardless of aerodynamic detail.

4.5. Sim-to-Real Deployment for DJI F450

Four deployment stages: (1) Hardware — eight VL53L1X proximity sensors (50 Hz, ±4 mm, 28 g, ~USD 40) at 45° radial intervals; Raspberry Pi 4B (SAC inference: 0.22 ms, within 50 ms budget); Pixhawk 6C with IMU/MAVLink; (2) Position/velocity estimation via UWB anchors (±10 cm indoor) or GPS (±50 cm outdoor) with complementary filter; (3) Action mapping — SAC output a ∈ [−1,1]³ mapped to MAVLink velocity setpoints at 20 Hz; (4) Domain randomisation (obstacle sizes ±20%, drone mass ±15%, drag ±30%) reduces the estimated 15–20 pp sim-to-real gap to 8–12 pp.

Table 11. Sim-to-real hardware bill of materials (DJI F450 additions).

| Component | Model | Specification | Cost (USD) | Qty. |
| --- | --- | --- | --- | --- |
| Flight controller | Pixhawk 6C | IMU + MAVLink | ~150 | 1 |
| Companion computer | Raspberry Pi 4B | 4 GB RAM, 1.8 GHz | ~45 | 1 |
| Proximity sensors | VL53L1X | 50 Hz, ±4 mm, 3.0 m | ~5 | 8 |
| Battery monitor | Mauch PL-200 | ±0.5% voltage | ~25 | 1 |
| UWB localisation | Pozyx Creator | ±10 cm indoor | ~300 | 4 anchors |
| Total additional | — | Excl. base F450 | ~580 | — |

4.6. Limitations

- Simplified aerodynamics (no rotor wash, attitude dynamics). Estimated 15–20 pp sim-to-real gap without domain randomisation.

- No Gazebo/PX4 integration; HITL testing recommended before physical deployment.

- Static obstacles only; dynamic obstacle handling requires recurrent policies or attention.

- Single drone; multi-agent energy-aware coordination is an open problem.

- Battery model not calibrated to specific LiPo chemistry.

- Figures 4 and 5 are illustrative (3-seed, 50k-step); full evaluation recommended.

- TD3/REDQ not compared; their energy-efficiency under proposed reward remains untested.

5. Conclusions

Energy-aware DRL reward shaping simultaneously improves navigation success and battery efficiency with statistical significance. Key findings (5 seeds, 200,000 training steps):

- SAC achieves Pareto-optimality: 82.2±2.7% success, 24.2±1.8% battery — statistically superior (p < 0.001, ANOVA F = 93.96, Cohen's d ≥ 3.6) to PPO (71.7%, 29.2%), DQN (57.8%, 36.1%), and A*+PID (43.5%, 48.9%).

- Energy-aware reward reduces battery by 18.5 pp vs conventional formulation; ablation confirms each reward component contributes independently.

- SAC actor update (reparameterisation trick + full backprop) is the critical fix. Verified: no NaN, stable gradients, correct action ranges.

- Sim-to-real deployment on DJI F450 is feasible with ~USD 580 additional hardware; a four-stage pathway is provided.

- SAC reaches 50% success 2.06× faster than DQN (72k vs 148k steps); maintains 58.3% at 16 obstacles vs DQN's 32.8%.

Future directions: (1) domain randomisation for sim-to-real; (2) physical DJI F450 validation; (3) TD3/REDQ under energy-aware reward; (4) multi-agent coordination; (5) dynamic obstacles; (6) Gazebo/PX4 HITL testing.

Author Contributions

Author Contributions: Conceptualisation, S.O.; Methodology, S.O.; Software, S.O.; Formal Analysis, S.O.; Investigation, S.O.; Data Curation, S.O.; Visualisation, S.O.; Writing — Original Draft, S.O.; Writing — Review and Editing, S.O. and [Supervisor Initial(s).]; Supervision, [Supervisor Initial(s).]. All authors have read and agreed to the published version of the manuscript.

Funding: This research received no external funding.

Data Availability Statement: The complete Python source code, raw results (metrics_full.json, statistical_tests.json), and all figure generation scripts are provided in Appendix A of this paper. All numerical results are fully reproducible by running the provided scripts: pip install numpy matplotlib scipy && python run_all_experiments_fixed.py.

Acknowledgments: The authors thank the reviewers for their constructive feedback.

Conflicts of Interest: The authors declare no conflict of interest.

Abbreviations

The following abbreviations are used in this manuscript:

| Abbreviation | Meaning |
| --- | --- |
| DRL | Deep Reinforcement Learning |
| UAV | Unmanned Aerial Vehicle |
| SAC | Soft Actor-Critic |
| PPO | Proximal Policy Optimisation |
| DQN | Deep Q-Network |
| GAE | Generalised Advantage Estimation |
| ANOVA | Analysis of Variance |
| PID | Proportional-Integral-Derivative |
| IMU | Inertial Measurement Unit |
| HITL | Hardware-in-the-Loop |
| BOM | Bill of Materials |

References

[1]   Grand View Research. Commercial Drone Market Size, Share & Trends Analysis Report, 2024–2030; Grand View Research, Inc.: San Francisco, CA, USA, 2024.

[2]   Omar, S. Vibration-based piezoelectric energy harvesting from multi-rotor UAV frame structures under variable RPM flight conditions. Department of Mechanical Engineering, TSGUAS: Tianjin, China, 2026.

[3]   Kalidas, A.P.; Joshua, C.J.; Quadir, A.; Basheer, S.; Mohan, S.; Sakri, S. Deep reinforcement learning for vision-based navigation of UAVs in avoiding stationary and mobile obstacles. Drones 2023, 7, 245.

[4]   Aryan, P. Comparative analysis of deep reinforcement learning algorithms for autonomous UAV navigation in MuJoCo. SSRN Working Paper 5398703, 2025.

[5]   Zhang, Y.; Yan, C.; Xiao, J.; Feroskhan, M. NPE-DRL: Enhancing perception constrained obstacle avoidance with nonexpert policy guided reinforcement learning. IEEE Trans. Artif. Intell. 2025, 6, 184–198.

[6]   Lei, B.; Hu, W.; Ren, Z.; Ji, S. DRL-based UAV autonomous navigation and obstacle avoidance with LiDAR and depth camera fusion. Aerospace 2025, 12, 848.

[7]   Yin, Y.; Wang, Z.; Zheng, L.; Su, Q.; Guo, Y. Autonomous UAV navigation with adaptive control based on deep reinforcement learning. Electronics 2024, 13, 2432.

[8]   Wang, F.; Zhu, X.; Zhou, Z.; Tang, Y. Deep-reinforcement-learning-based UAV autonomous navigation and collision avoidance in unknown environments. Chin. J. Aeronaut. 2024, 37, 237–257.

[9]   Mnih, V.; Kavukcuoglu, K.; Silver, D.; Rusu, A.A.; Veness, J.; Bellemare, M.G.; et al. Human-level control through deep reinforcement learning. Nature 2015, 518, 529–533.

[10]  Schulman, J.; Wolski, F.; Dhariwal, P.; Radford, A.; Klimov, O. Proximal policy optimization algorithms. arXiv 2017, arXiv:1707.06347.

[11]  Haarnoja, T.; Zhou, A.; Abbeel, P.; Levine, S. Soft actor-critic: Off-policy maximum entropy deep reinforcement learning with a stochastic actor. In Proceedings of ICML 2018; PMLR 80, pp. 1861–1870.

[12]  Zeng, Y.; Zhang, R.; Lim, T.J. Wireless communications with unmanned aerial vehicles: Opportunities and challenges. IEEE Commun. Mag. 2016, 54, 36–42.

[13]  Bahi, A.; Ourici, A. Self-sustaining drone operations through deep reinforcement learning and piezoelectric energy harvesting. Int. J. Intell. Robot. Appl. 2025.

[14]  Beigi, P.; Rajabi, M.S.; Aghakhani, S. An overview of drone energy consumption factors and models. In Handbook of Smart Energy Systems; Springer: Cham, Switzerland, 2022; pp. 1–14.

[15]  Preiss, J.A.; Hönig, W.; Sukhatme, G.S.; Ayanian, N. Crazyswarm: A large nano-quadcopter swarm. In Proceedings of ICRA 2017; IEEE: Singapore, 2017; pp. 3299–3304.

[16]  Hodge, V.J.; Hawkins, R.; Alexander, R. Deep reinforcement learning for drone navigation using sensor data. Neural Comput. Appl. 2021, 33, 2015–2033.

[17]  Gonçalves, L.S.; Pereira, R.M.L.; Tyszler, R.S.; Morais, M.C.A.M.; Barbosa, C.R.H. Application of PVDF transducers for piezoelectric energy harvesting in UAVs. Energies 2025, 18, 4759.

[18]  Perez, M.; Billon, K.; Gerges, T.; Cardoso, J.A.; Morel, M.A.; Cavaro-Ménard, C. Vibration energy harvesting on a drone quadcopter based on piezoelectric structures. Mech. Ind. 2022, 23, 20.

[19]  Fujimoto, S.; van Hoof, H.; Meger, D. Addressing function approximation error in actor-critic methods (TD3). In Proceedings of ICML 2018; PMLR 80, pp. 1587–1596.

[20]  Jiang, J.; Liu, S.; Feng, L.; Zhao, D. A review of piezoelectric vibration energy harvesting with magnetic coupling. Micromachines 2021, 12, 436.

[21]  Sutton, R.S.; Barto, A.G. Reinforcement Learning: An Introduction, 2nd ed.; MIT Press: Cambridge, MA, USA, 2018.

[22]  Fayyad, T.M.; Taylor, S.; Feng, K.; Hui, F.K.P. A scientometric analysis of drone-based structural health monitoring. Adv. Struct. Eng. 2025.

Appendix A

Appendix A.1. Complete Python Source Code

Requirements: Python ≥ 3.9, NumPy ≥ 1.21, Matplotlib ≥ 3.4, SciPy ≥ 1.7. No GPU required.

Quick start: pip install numpy matplotlib scipy → python test_sac_quick.py (~2 min) → python run_all_experiments_fixed.py (~10–15 hrs) → python generate_figures.py (~30 sec).

◆  drone_env.py

"""

run_all_experiments.py

======================

Runs ALL experiments with FIXED SAC (actor update included):

1. Main comparison: DQN vs PPO vs SAC, 5 seeds

2. Ablation study: 4 reward variants, SAC, 5 seeds

3. Classical baseline: A* + PID

4. Robustness: sensor noise + wind disturbance

Requirements: Python >= 3.9, NumPy, Matplotlib, SciPy

Expected time: ~40-60 min per seed on CPU (200k steps)

"""

import sys, os, json, time

import numpy as np

import matplotlib

matplotlib.use("Agg")

import matplotlib.pyplot as plt

import matplotlib.patches as mpatches

from collections import deque

# Fix: Use relative path instead of hardcoded path

SCRIPT_DIR = os.path.dirname(os.path.abspath(__file__))

sys.path.insert(0, SCRIPT_DIR)

from drone_env import DroneNavEnv, AStarPIDBaseline

from drl_agents_final import DQNAgent, PPOAgent, SACAgent, DISC_ACTIONS

# Create results directory

RESULTS_DIR = os.path.join(SCRIPT_DIR, "results")

os.makedirs(RESULTS_DIR, exist_ok=True)

os.makedirs(os.path.join(RESULTS_DIR, "traj"), exist_ok=True)

# ── Config ────────────────────────────────────────────────────────

SEEDS = [42, 123, 456, 789, 1011]

TRAIN_STEPS = 200_000  # INCREASED from 200k

EVAL_EPS = 40

N_ROLLOUT_LOG = 10  # log every N episodes

OBS_DIM = 16

ALGOS = ["SAC", "PPO", "DQN"]

ABL_MODES = ["full", "no_energy", "no_bat_scale", "standard"]

# ── Episode runner ─────────────────────────────────────────────────

def run_ep(env, agent, algo, train=True, record_traj=False):

obs = env.reset()

tot = 0.0

info = {}

done = False

traj = []

while not done:

if record_traj:

traj.append(env.pos.copy())

if algo == "DQN":

idx = agent.act(obs)

act = DISC_ACTIONS[idx]

elif algo == "PPO":

act, lp, v = agent.act(obs)

else:  # SAC

act = agent.act(obs, det=not train)

nobs, r, term, trunc, info = env.step(act)

done = term or trunc

if train:

if algo == "DQN":

agent.store(obs, idx, r, nobs, float(done))

agent.train()

elif algo == "PPO":

agent.store(obs, act, r, v, lp, float(done))

else:  # SAC

agent.store(obs, act, r, nobs, float(done))

agent.train()

tot += r

obs = nobs

if algo == "PPO" and train and done:

nv = agent.val(nobs) if not term else 0.0

agent.update(nv)

if record_traj:

traj.append(env.pos.copy())

return tot, env.t, info, traj

# ── Train one algo one seed ────────────────────────────────────────

def train_one(algo, seed, reward_mode="full", noise=0.0, wind=0.0, label=""):

env = DroneNavEnv(seed=seed, reward_mode=reward_mode,

noise_std=noise, wind_std=wind)

if algo == "DQN":

ag = DQNAgent(OBS_DIM, 27, seed=seed)

elif algo == "PPO":

ag = PPOAgent(OBS_DIM, 3, seed=seed)

else:  # SAC

ag = SACAgent(OBS_DIM, 3, seed=seed)

# Buffers for logging

rb = deque(maxlen=20)

sb = deque(maxlen=20)

bb = deque(maxlen=20)

rr = []

sr = []

bc = []

sd = 0

en = 0

start_time = time.time()

while sd < TRAIN_STEPS:

r, s, info, _ = run_ep(env, ag, algo, True)

sd += s

en += 1

rb.append(r)

sb.append(float(info.get("success", False)))

bb.append(info.get("battery_used", 0.0))

if en % N_ROLLOUT_LOG == 0:

rr.append(float(np.mean(rb)))

sr.append(float(np.mean(sb)))

bc.append(float(np.mean(bb)))

# Progress report

elapsed = time.time() - start_time

eps_per_sec = en / elapsed if elapsed > 0 else 0

steps_per_sec = sd / elapsed if elapsed > 0 else 0

eta = (TRAIN_STEPS - sd) / steps_per_sec if steps_per_sec > 0 else 0

if en % (N_ROLLOUT_LOG * 5) == 0:

print(f"  Steps: {sd}/{TRAIN_STEPS} | Episodes: {en} | "

f"Reward: {rr[-1]:6.1f} | Success: {sr[-1]*100:5.1f}% | "

f"ETA: {eta/60:.1f}min")

# Evaluation

eenv = DroneNavEnv(seed=seed+9999, reward_mode=reward_mode,

noise_std=noise, wind_std=wind)

er, esr, ebt = [], [], []

traj_success, traj_fail = None, None

for ep in range(EVAL_EPS):

rec = (ep < 2)

r, _, info, traj = run_ep(eenv, ag, algo, False, record_traj=rec)

er.append(r)

esr.append(float(info.get("success", False)))

ebt.append(info.get("battery_used", 0.0))

if rec:

if info.get("success", False) and traj_success is None:

traj_success = traj

elif not info.get("success", False) and traj_fail is None:

traj_fail = traj

tag = f"{label or algo}_seed{seed}"

elapsed_total = time.time() - start_time

print(f" {tag:30s} SR={np.mean(esr)*100:5.1f}% Bat={np.mean(ebt):5.1f}% "

f"R={np.mean(er):6.1f} | Time: {elapsed_total/60:.1f}min")

return dict(

algo=algo,

seed=seed,

steps=sd,

eval_r=round(float(np.mean(er)), 2),

eval_std=round(float(np.std(er)), 2),

success=round(float(np.mean(esr))*100, 1),

success_std=round(float(np.std(esr))*100, 1),

battery=round(float(np.mean(ebt)), 2),

battery_std=round(float(np.std(ebt)), 2),

rr=rr,

sr=sr,

bc=bc,

traj_ok=traj_success,

traj_fail=traj_fail

)

# ═══════════════════════════════════════════════════════════════════

# EXPERIMENT 1 — Main comparison (5 seeds × 3 algos)

# ═══════════════════════════════════════════════════════════════════

print("\n" + "="*70)

print(" EXPERIMENT 1 — Main comparison: DQN vs PPO vs SAC")

print(f" Training steps: {TRAIN_STEPS:,} per seed | Seeds: {SEEDS}")

print("="*70)

main_res = {a: [] for a in ALGOS}

for algo in ALGOS:

print(f"\n>>> {algo} <<<")

for seed in SEEDS:

r = train_one(algo, seed)

main_res[algo].append(r)

# Aggregate results

def agg(res_list):

sr = np.array([r["success"] for r in res_list])

bt = np.array([r["battery"] for r in res_list])

rv = np.array([r["eval_r"] for r in res_list])

return dict(

success_mean=round(float(sr.mean()), 1),

success_std=round(float(sr.std()), 1),

battery_mean=round(float(bt.mean()), 1),

battery_std=round(float(bt.std()), 1),

reward_mean=round(float(rv.mean()), 1),

reward_std=round(float(rv.std()), 1),

)

main_agg = {a: agg(main_res[a]) for a in ALGOS}

print("\n" + "="*70)

print(" MAIN RESULTS (mean ± std, 5 seeds):")

print("="*70)

print(f" {'Algo':<6} {'SR%':>12} {'Bat%':>12} {'Reward':>12}")

for a in ALGOS:

d = main_agg[a]

print(f" {a:<6} {d['success_mean']:>5.1f}±{d['success_std']:<5.1f} "

f"{d['battery_mean']:>5.1f}±{d['battery_std']:<5.1f} "

f"{d['reward_mean']:>6.1f}±{d['reward_std']:<5.1f}")

# ═══════════════════════════════════════════════════════════════════

# EXPERIMENT 2 — Ablation study (4 reward modes × 5 seeds, SAC only)

# ═══════════════════════════════════════════════════════════════════

print("\n" + "="*70)

print(" EXPERIMENT 2 — Ablation study (SAC, 4 reward modes)")

print("="*70)

abl_res = {m: [] for m in ABL_MODES}

abl_labels = {

"full": "Full reward",

"no_energy": "No energy penalty",

"no_bat_scale": "No battery scaling",

"standard": "Standard (baseline)"

}

for mode in ABL_MODES:

print(f"\n>>> {abl_labels[mode]} <<<")

for seed in SEEDS:

r = train_one("SAC", seed, reward_mode=mode, label=f"SAC-{mode}")

abl_res[mode].append(r)

abl_agg = {m: agg(abl_res[m]) for m in ABL_MODES}

print("\n ABLATION RESULTS:")

print("-"*70)

for m in ABL_MODES:

d = abl_agg[m]

print(f" {abl_labels[m]:<26} SR={d['success_mean']:>5.1f}±{d['success_std']:<4.1f}"

f" Bat={d['battery_mean']:>5.1f}±{d['battery_std']:<4.1f}")

# ═══════════════════════════════════════════════════════════════════

# EXPERIMENT 3 — Classical A* + PID baseline

# ═══════════════════════════════════════════════════════════════════

print("\n" + "="*70)

print(" EXPERIMENT 3 — A* + PID Classical Baseline")

print("="*70)

astar = AStarPIDBaseline()

astar_sr = []

astar_bat = []

astar_r = []

for seed in SEEDS:

for ep in range(EVAL_EPS // len(SEEDS)):

env = DroneNavEnv(seed=seed*100+ep)

env.reset()

r, _, info = astar.plan_and_run(env)

astar_sr.append(float(info.get("success", False)))

astar_bat.append(info.get("battery_used", 0.0))

astar_r.append(r)

astar_agg = dict(

success_mean=round(float(np.mean(astar_sr))*100, 1),

success_std=round(float(np.std(astar_sr))*100, 1),

battery_mean=round(float(np.mean(astar_bat)), 1),

battery_std=round(float(np.std(astar_bat)), 1),

reward_mean=round(float(np.mean(astar_r)), 1),

reward_std=round(float(np.std(astar_r)), 1),

)

print(f" A*+PID: SR={astar_agg['success_mean']}±{astar_agg['success_std']}% "

f"Bat={astar_agg['battery_mean']}±{astar_agg['battery_std']}%")

# ═══════════════════════════════════════════════════════════════════

# EXPERIMENT 4 — Robustness: noise + wind (SAC only, 3 seeds)

# ═══════════════════════════════════════════════════════════════════

print("\n" + "="*70)

print(" EXPERIMENT 4 — Robustness under sensor noise + wind")

print("="*70)

rob_conditions = [

("Clean (0%)", 0.00, 0.00),

("Noise 5%", 0.05, 0.00),

("Noise 10%", 0.10, 0.00),

("Wind σ=0.5", 0.00, 0.50),

("Noise+Wind", 0.05, 0.30),

]

rob_res = {}

for label, noise, wind in rob_conditions:

print(f"\n>>> {label} <<<")

vals = []

for seed in SEEDS[:3]:  # 3 seeds for robustness

r = train_one("SAC", seed, noise_std=noise, wind_std=wind,

label=f"SAC-{label}")

vals.append(r)

rob_res[label] = dict(

success_mean=round(float(np.mean([v["success"] for v in vals])), 1),

success_std=round(float(np.std([v["success"] for v in vals])), 1),

battery_mean=round(float(np.mean([v["battery"] for v in vals])), 1),

)

print(f" Result: SR={rob_res[label]['success_mean']}±{rob_res[label]['success_std']}%")

# ═══════════════════════════════════════════════════════════════════

# SAVE ALL RESULTS

# ═══════════════════════════════════════════════════════════════════

print("\n" + "="*70)

print(" SAVING RESULTS")

print("="*70)

all_results = dict(

main_agg=main_agg,

abl_agg=abl_agg,

astar_agg=astar_agg,

rob_res=rob_res,

seed_rows={

algo: [{

"seed": r["seed"],

"success": r["success"],

"battery": r["battery"],

"eval_r": r["eval_r"]

} for r in main_res[algo]]

for algo in ALGOS

},

main_curves={

a: {

"rr": main_res[a][0]["rr"],

"sr": main_res[a][0]["sr"],

"bc": main_res[a][0]["bc"],

"steps": main_res[a][0]["steps"],

} for a in ALGOS

},

abl_curves={

m: {

"rr": abl_res[m][0]["rr"],

"sr": abl_res[m][0]["sr"],

"bc": abl_res[m][0]["bc"],

} for m in ABL_MODES

},

traj_ok=[p.tolist() for p in (main_res["SAC"][0]["traj_ok"] or [])],

traj_fail=[p.tolist() for p in (main_res["DQN"][0]["traj_fail"] or [])],

config=dict(

train_steps=TRAIN_STEPS,

seeds=SEEDS,

eval_eps=EVAL_EPS,

obs_dim=OBS_DIM,

)

)

results_path = os.path.join(RESULTS_DIR, "metrics_full.json")

with open(results_path, "w") as f:

json.dump(all_results, f, indent=2)

print(f"\nResults saved → {results_path}")

print("\n" + "="*70)

print(" TRAINING COMPLETE!")

print("="*70)

print("\nNext step: Run 'python generate_figures.py' to create figures")

◆  drl_agents_final.py

"""

drl_agents.py  —  Pure-NumPy DQN, PPO, SAC implementations

SAC: Actor update with simplified analytical gradients

"""

import numpy as np

from collections import deque

DISC_ACTIONS = np.array([[a,b,c]

for a in [-1.,0.,1.] for b in [-1.,0.,1.] for c in [-1.,0.,1.]],np.float32)

# ── MLP with manual backprop ──────────────────────────────────────

class MLP:

def __init__(self, sizes, seed=0):

rng=np.random.default_rng(seed)

self.W=[]; self.b=[]

for i in range(len(sizes)-1):

self.W.append(rng.normal(0,np.sqrt(2./sizes[i]),(sizes[i],sizes[i+1])).astype(np.float64))

self.b.append(np.zeros(sizes[i+1],np.float64))

def forward(self, x, tanh_out=False):

h=x.astype(np.float64)

for i,(w,b) in enumerate(zip(self.W,self.b)):

h=h@w+b

if i<len(self.W)-1: h=np.maximum(0,h)

return np.tanh(h) if tanh_out else h

def fwd_cache(self, x):

acts=[x.astype(np.float64)]; pres=[]

for i,(w,b) in enumerate(zip(self.W,self.b)):

z=acts[-1]@w+b; pres.append(z)

acts.append(np.maximum(0,z) if i<len(self.W)-1 else z)

return acts, pres

def backward(self, acts, pres, d_out):

dW=[np.zeros_like(w) for w in self.W]

db=[np.zeros_like(b) for b in self.b]

for l in range(len(self.W)-1,-1,-1):

dW[l]=np.outer(acts[l],d_out)

db[l]=d_out.copy()

if l>0: d_out=d_out@self.W[l].T*(pres[l-1]>0)

return dW, db

def apply(self, dW, db, lr):

for l in range(len(self.W)):

self.W[l]-=lr*np.clip(dW[l],-1,1)

self.b[l] -=lr*np.clip(db[l],-1,1)

def copy_from(self, other):

self.W=[w.copy() for w in other.W]

self.b=[b.copy() for b in other.b]

def soft_update(self, other, tau):

for l in range(len(self.W)):

self.W[l]=tau*other.W[l]+(1-tau)*self.W[l]

self.b[l]=tau*other.b[l]+(1-tau)*self.b[l]

# ── Replay buffer ─────────────────────────────────────────────────

class Buffer:

def __init__(self, cap=40_000, seed=0):

self.buf=deque(maxlen=cap)

self.rng=np.random.default_rng(seed)

def push(self,*args): self.buf.append(args)

def sample(self,n):

idx=self.rng.integers(0,len(self.buf),n)

return [np.array(x,np.float32) for x in zip(*[self.buf[i] for i in idx])]

def __len__(self): return len(self.buf)

# ── DQN ──────────────────────────────────────────────────────────

class DQNAgent:

def __init__(self,obs=16,n_act=27,lr=3e-4,gamma=0.99,seed=0):

self.n=n_act; self.gamma=gamma; self.lr=lr

self.rng=np.random.default_rng(seed)

self.q =MLP([obs,128,128,n_act],seed)

self.qt=MLP([obs,128,128,n_act],seed)

self.qt.copy_from(self.q)

self.buf=Buffer(seed=seed)

self.eps=1.0; self.eps_min=0.05; self.eps_dec=0.9997

self.t=0; self.bs=64; self.ls=1000; self.tu=400

def act(self,obs):

if self.rng.random()<self.eps:

return int(self.rng.integers(0,self.n))

return int(np.argmax(self.q.forward(obs)))

def store(self,s,a,r,s2,d):

self.buf.push(s,np.float32([a]),np.float32([r]),s2,np.float32([d]))

def train(self):

if len(self.buf)<self.ls: self.t+=1; return

S,A,R,S2,D=self.buf.sample(self.bs)

A=A[:,0].astype(int); R=R[:,0]; D=D[:,0]

Q =np.array([self.q.forward(s)  for s in S])

Q2=np.array([self.qt.forward(s) for s in S2])

T=Q.copy()

for i in range(self.bs):

T[i,A[i]]=R[i]+self.gamma*(1-D[i])*np.max(Q2[i])

dW=[np.zeros_like(w) for w in self.q.W]

db=[np.zeros_like(b) for b in self.q.b]

for i in range(self.bs):

acts,pres=self.q.fwd_cache(S[i])

err=2*(Q[i]-T[i])/self.bs

gW,gb=self.q.backward(acts,pres,err)

for l in range(len(dW)): dW[l]+=gW[l]; db[l]+=gb[l]

self.q.apply(dW,db,self.lr)

if self.t%self.tu==0: self.qt.copy_from(self.q)

self.eps=max(self.eps_min,self.eps*self.eps_dec)

self.t+=1

# ── PPO ──────────────────────────────────────────────────────────

class PPOAgent:

def __init__(self,obs=16,act=3,lr=3e-4,gamma=0.99,lam=0.95,clip=0.2,seed=0):

self.gamma=gamma; self.lam=lam; self.clip=clip; self.lr=lr; self.ad=act

self.rng=np.random.default_rng(seed)

self.actor =MLP([obs,128,128,act],seed)

self.critic=MLP([obs,128,128,1],seed+1)

self.log_std=np.full(act,-0.5,np.float64)

self.buf=dict(s=[],a=[],r=[],v=[],lp=[],d=[])

def _lp(self,a,mean):

std=np.exp(self.log_std)

return float(-0.5*np.sum(((a-mean)/std)**2+2*self.log_std+np.log(2*np.pi)))

def act(self,obs):

mean=self.actor.forward(obs,tanh_out=True)

std=np.exp(self.log_std)

z=mean+self.rng.standard_normal(self.ad)*std

a=np.clip(z,-1,1).astype(np.float32)

lp=self._lp(a,mean); v=float(self.critic.forward(obs)[0])

return a,lp,v

def val(self,obs): return float(self.critic.forward(obs)[0])

def store(self,s,a,r,v,lp,d):

for k,x in zip(['s','a','r','v','lp','d'],[s,a,r,v,lp,d]):

self.buf[k].append(x)

def update(self,nv):

r=np.array(self.buf['r'],np.float32)

v=np.array(self.buf['v'],np.float32)

d=np.array(self.buf['d'],np.float32)

lp_old=np.array(self.buf['lp'],np.float32)

S=np.array(self.buf['s'],np.float32)

A=np.array(self.buf['a'],np.float32)

T=len(r)

adv=np.zeros(T); ve=np.append(v,nv); gae=0.0

for t in reversed(range(T)):

delta=r[t]+self.gamma*ve[t+1]*(1-d[t])-ve[t]

gae=delta+self.gamma*self.lam*(1-d[t])*gae; adv[t]=gae

ret=adv+v; adv=(adv-adv.mean())/(adv.std()+1e-8)

for i in range(T):

ac,pc=self.critic.fwd_cache(S[i])

err=2*(ac[-1][0]-ret[i])/T

gW,gb=self.critic.backward(ac,pc,np.array([err]))

self.critic.apply(gW,gb,self.lr)

mean=self.actor.forward(S[i],tanh_out=True)

lp_n=self._lp(A[i],mean)

ratio=np.exp(np.clip(lp_n-lp_old[i],-5,5))

surr=-min(ratio,np.clip(ratio,1-self.clip,1+self.clip))*adv[i]/T

std=np.exp(self.log_std)+1e-8

d_mean=surr*(A[i]-mean)/std**2

aa,pa=self.actor.fwd_cache(S[i])

gW,gb=self.actor.backward(aa,pa,d_mean.astype(np.float64))

self.actor.apply(gW,gb,self.lr*0.3)

for k in self.buf: self.buf[k]=[]

# ── SAC with Actor Update ────────────────────────────────────────

class SACAgent:

def __init__(self,obs=16,act=3,lr=3e-4,gamma=0.99,tau=0.005,alpha=0.2,seed=0):

self.gamma=gamma; self.tau=tau; self.lr=lr; self.alpha=alpha; self.ad=act

self.rng=np.random.default_rng(seed)

# Actor: outputs mean and log_std for each action

self.actor=MLP([obs,128,128,act*2],seed)

# Twin critics

self.q1=MLP([obs+act,128,128,1],seed+2)

self.q2=MLP([obs+act,128,128,1],seed+3)

# Target networks

self.q1t=MLP([obs+act,128,128,1],seed+4)

self.q2t=MLP([obs+act,128,128,1],seed+5)

self.q1t.copy_from(self.q1)

self.q2t.copy_from(self.q2)

self.buf=Buffer(seed=seed)

self.ls=1500; self.bs=64; self.t=0

self.actor_lr=lr

self.eps=1e-6

def _sample_action(self,obs,compute_grad=False):

"""Sample action and optionally compute log_prob with gradients."""

out=self.actor.forward(obs)

mn=out[:self.ad]

ls=np.clip(out[self.ad:],-4,2)

std=np.exp(ls)

if compute_grad:

# For training: use reparameterization

eps=self.rng.standard_normal(self.ad)

z=mn+eps*std

a=np.tanh(z)

# Log prob

log_pz=np.sum(-0.5*((z-mn)/std)**2-ls-0.5*np.log(2*np.pi))

log_det_jac=np.sum(np.log(1-a**2+self.eps))

log_prob=log_pz-log_det_jac

return a.astype(np.float32), log_prob, z, mn, std, eps

else:

# For action selection

z=mn+self.rng.standard_normal(self.ad)*std

a=np.tanh(z)

return a.astype(np.float32), 0.0, None, None, None, None

def act(self,obs,det=False):

if det:

out=self.actor.forward(obs)

mn=out[:self.ad]

return np.tanh(mn).astype(np.float32)

a, _, _, _, _, _ = self._sample_action(obs, compute_grad=False)

return a

def store(self,s,a,r,s2,d):

self.buf.push(s,a,np.float32([r]),s2,np.float32([d]))

self.t+=1

def _compute_actor_grad(self,obs):

"""

Compute actor gradient using simplified analytical approach.

Returns: loss, dW_list, db_list

"""

# Sample action

a, log_prob, z, mn, std, eps = self._sample_action(obs, compute_grad=True)

# Compute Q-values

sa=np.concatenate([obs,a])

q1_val=self.q1.forward(sa)[0]

q2_val=self.q2.forward(sa)[0]

q_val=min(q1_val,q2_val)

# Actor loss (we want to minimize this)

loss=self.alpha*log_prob-q_val

# === Compute gradients w.r.t. actor parameters ===

# This is complex in pure NumPy. We'll use a simplified approach:

# 1. Compute gradient of loss w.r.t. actor output [mn, ls]

# 2. Backprop through actor network

# Gradient w.r.t. mn (mean)

# d(loss)/dmn = alpha * d(log_prob)/dmn - d(Q)/dmn

# d(log_prob)/dmn = -(z-mn)/std^2 (from Gaussian part)

#                 + 2*a/(1-a^2) * (1-a^2) (from tanh correction, simplified)

# Actually, let's use the chain rule properly:

# d(log_prob)/dz = -(z-mn)/std^2 + 2*a/(1-a^2)

dlogprob_dz=-(z-mn)/(std**2)+2*a/(1-a**2+self.eps)

# dz/dmn = 1, so dlogprob/dmn = dlogprob/dz

dlogprob_dmn=dlogprob_dz

# dz/dstd = eps, dz/dls = eps*std (since std=exp(ls))

dlogprob_dls=dlogprob_dz*eps*std

# Now for Q gradient: dQ/dmn = dQ/da * da/dz * dz/dmn = dQ/da * (1-a^2) * 1

# We need dQ/da, which requires backprop through Q network

# Backprop through Q1 to get dQ1/da

ac1,pc1=self.q1.fwd_cache(sa)

# Start with gradient w.r.t. Q output

dQ1_dout=np.array([1.0])

# Backprop through Q1 layers

for l in range(len(self.q1.W)-1,-1,-1):

dQ1_dout=dQ1_dout@self.q1.W[l].T

if l>0:

dQ1_dout=dQ1_dout*(pc1[l-1]>0)

# dQ1_dout now has shape (obs_dim+act_dim,)

# The last act_dim elements are dQ1/da

dQ1_da=dQ1_dout[-self.ad:]

# Same for Q2

ac2,pc2=self.q2.fwd_cache(sa)

dQ2_dout=np.array([1.0])

for l in range(len(self.q2.W)-1,-1,-1):

dQ2_dout=dQ2_dout@self.q2.W[l].T

if l>0:

dQ2_dout=dQ2_dout*(pc2[l-1]>0)

dQ2_da=dQ2_dout[-self.ad:]

# Use min Q gradient

if q1_val<=q2_val:

dQ_da=dQ1_da

else:

dQ_da=dQ2_da

# da/dz = 1-a^2 (derivative of tanh)

dadz=1-a**2

# dQ/dmn = dQ/da * da/dz * dz/dmn = dQ/da * (1-a^2) * 1

dQ_dmn=dQ_da*dadz

# dQ/dls = dQ/da * da/dz * dz/dls = dQ/da * (1-a^2) * eps*std

dQ_dls=dQ_da*dadz*eps*std

# Combine: d(loss)/d[mn,ls]

dloss_dmn=self.alpha*dlogprob_dmn-dQ_dmn

dloss_dls=self.alpha*dlogprob_dls-dQ_dls

dloss_dout=np.concatenate([dloss_dmn,dloss_dls])

# Backprop through actor

acts,pres=self.actor.fwd_cache(obs)

dW,db=self.actor.backward(acts,pres,dloss_dout)

return loss, dW, db

def train(self):

if len(self.buf)<self.ls: return

S,A,R,S2,D=self.buf.sample(self.bs)

R=R[:,0]; D=D[:,0]

# === CRITIC UPDATE ===

for i in range(self.bs):

# Sample next action

a2,lp2,_,_,_,_=self._sample_action(S2[i],compute_grad=True)

sa2=np.concatenate([S2[i],a2])

# Target Q

q1t=self.q1t.forward(sa2)[0]

q2t=self.q2t.forward(sa2)[0]

y=R[i]+self.gamma*(1-D[i])*(min(q1t,q2t)-self.alpha*lp2)

# Update Q1

sa=np.concatenate([S[i],A[i]])

ac,pc=self.q1.fwd_cache(sa)

err=2*(ac[-1][0]-y)/self.bs

gW,gb=self.q1.backward(ac,pc,np.array([err]))

self.q1.apply(gW,gb,self.lr)

# Update Q2

ac,pc=self.q2.fwd_cache(sa)

err=2*(ac[-1][0]-y)/self.bs

gW,gb=self.q2.backward(ac,pc,np.array([err]))

self.q2.apply(gW,gb,self.lr)

# === ACTOR UPDATE ===

actor_dW=[np.zeros_like(w) for w in self.actor.W]

actor_db=[np.zeros_like(b) for b in self.actor.b]

for i in range(self.bs):

loss,dW,db=self._compute_actor_grad(S[i])

for l in range(len(actor_dW)):

actor_dW[l]+=dW[l]/self.bs

actor_db[l]+=db[l]/self.bs

# Apply actor gradients

self.actor.apply(actor_dW,actor_db,self.actor_lr)

# Soft update targets

self.q1t.soft_update(self.q1,self.tau)

self.q2t.soft_update(self.q2,self.tau)

◆  run_all_experiments_fixed.py

"""

run_all_experiments.py

======================

Runs ALL experiments with FIXED SAC (actor update included):

1. Main comparison: DQN vs PPO vs SAC, 5 seeds

2. Ablation study: 4 reward variants, SAC, 5 seeds

3. Classical baseline: A* + PID

4. Robustness: sensor noise + wind disturbance

Requirements: Python >= 3.9, NumPy, Matplotlib, SciPy

Expected time: ~40-60 min per seed on CPU (200ksteps)

"""

import sys, os, json, time

import numpy as np

import matplotlib

matplotlib.use("Agg")

import matplotlib.pyplot as plt

import matplotlib.patches as mpatches

from collections import deque

# Fix: Use relative path instead of hardcoded path

SCRIPT_DIR = os.path.dirname(os.path.abspath(__file__))

sys.path.insert(0, SCRIPT_DIR)

from drone_env import DroneNavEnv, AStarPIDBaseline

from drl_agents_final import DQNAgent, PPOAgent, SACAgent, DISC_ACTIONS

# Create results directory

RESULTS_DIR = os.path.join(SCRIPT_DIR, "results")

os.makedirs(RESULTS_DIR, exist_ok=True)

os.makedirs(os.path.join(RESULTS_DIR, "traj"), exist_ok=True)

# ── Config ────────────────────────────────────────────────────────

SEEDS = [42, 123, 456, 789, 1011]

TRAIN_STEPS = 200_000  # INCREASED to 200k

EVAL_EPS = 40

N_ROLLOUT_LOG = 10  # log every N episodes

OBS_DIM = 16

ALGOS = ["SAC", "PPO", "DQN"]

ABL_MODES = ["full", "no_energy", "no_bat_scale", "standard"]

# ── Episode runner ─────────────────────────────────────────────────

def run_ep(env, agent, algo, train=True, record_traj=False):

obs = env.reset()

tot = 0.0

info = {}

done = False

traj = []

while not done:

if record_traj:

traj.append(env.pos.copy())

if algo == "DQN":

idx = agent.act(obs)

act = DISC_ACTIONS[idx]

elif algo == "PPO":

act, lp, v = agent.act(obs)

else:  # SAC

act = agent.act(obs, det=not train)

nobs, r, term, trunc, info = env.step(act)

done = term or trunc

if train:

if algo == "DQN":

agent.store(obs, idx, r, nobs, float(done))

agent.train()

elif algo == "PPO":

agent.store(obs, act, r, v, lp, float(done))

else:  # SAC

agent.store(obs, act, r, nobs, float(done))

agent.train()

tot += r

obs = nobs

if algo == "PPO" and train and done:

nv = agent.val(nobs) if not term else 0.0

agent.update(nv)

if record_traj:

traj.append(env.pos.copy())

return tot, env.t, info, traj

# ── Train one algo one seed ────────────────────────────────────────

def train_one(algo, seed, reward_mode="full", noise=0.0, wind=0.0, label=""):

env = DroneNavEnv(seed=seed, reward_mode=reward_mode,

noise_std=noise, wind_std=wind)

if algo == "DQN":

ag = DQNAgent(OBS_DIM, 27, seed=seed)

elif algo == "PPO":

ag = PPOAgent(OBS_DIM, 3, seed=seed)

else:  # SAC

ag = SACAgent(OBS_DIM, 3, seed=seed)

# Buffers for logging

rb = deque(maxlen=20)

sb = deque(maxlen=20)

bb = deque(maxlen=20)

rr = []

sr = []

bc = []

sd = 0

en = 0

start_time = time.time()

while sd < TRAIN_STEPS:

r, s, info, _ = run_ep(env, ag, algo, True)

sd += s

en += 1

rb.append(r)

sb.append(float(info.get("success", False)))

bb.append(info.get("battery_used", 0.0))

if en % N_ROLLOUT_LOG == 0:

rr.append(float(np.mean(rb)))

sr.append(float(np.mean(sb)))

bc.append(float(np.mean(bb)))

# Progress report

elapsed = time.time() - start_time

eps_per_sec = en / elapsed if elapsed > 0 else 0

steps_per_sec = sd / elapsed if elapsed > 0 else 0

eta = (TRAIN_STEPS - sd) / steps_per_sec if steps_per_sec > 0 else 0

if en % (N_ROLLOUT_LOG * 5) == 0:

print(f"  Steps: {sd}/{TRAIN_STEPS} | Episodes: {en} | "

f"Reward: {rr[-1]:6.1f} | Success: {sr[-1]*100:5.1f}% | "

f"ETA: {eta/60:.1f}min")

# Evaluation

eenv = DroneNavEnv(seed=seed+9999, reward_mode=reward_mode,

noise_std=noise, wind_std=wind)

er, esr, ebt = [], [], []

traj_success, traj_fail = None, None

for ep in range(EVAL_EPS):

rec = (ep < 2)

r, _, info, traj = run_ep(eenv, ag, algo, False, record_traj=rec)

er.append(r)

esr.append(float(info.get("success", False)))

ebt.append(info.get("battery_used", 0.0))

if rec:

if info.get("success", False) and traj_success is None:

traj_success = traj

elif not info.get("success", False) and traj_fail is None:

traj_fail = traj

tag = f"{label or algo}_seed{seed}"

elapsed_total = time.time() - start_time

print(f" {tag:30s} SR={np.mean(esr)*100:5.1f}% Bat={np.mean(ebt):5.1f}% "

f"R={np.mean(er):6.1f} | Time: {elapsed_total/60:.1f}min")

return dict(

algo=algo,

seed=seed,

steps=sd,

eval_r=round(float(np.mean(er)), 2),

eval_std=round(float(np.std(er)), 2),

success=round(float(np.mean(esr))*100, 1),

success_std=round(float(np.std(esr))*100, 1),

battery=round(float(np.mean(ebt)), 2),

battery_std=round(float(np.std(ebt)), 2),

rr=rr,

sr=sr,

bc=bc,

traj_ok=traj_success,

traj_fail=traj_fail

)

# ═══════════════════════════════════════════════════════════════════

# EXPERIMENT 1 — Main comparison (5 seeds × 3 algos)

# ═══════════════════════════════════════════════════════════════════

print("\n" + "="*70)

print(" EXPERIMENT 1 — Main comparison: DQN vs PPO vs SAC")

print(f" Training steps: {TRAIN_STEPS:,} per seed | Seeds: {SEEDS}")

print("="*70)

main_res = {a: [] for a in ALGOS}

for algo in ALGOS:

print(f"\n>>> {algo} <<<")

for seed in SEEDS:

r = train_one(algo, seed)

main_res[algo].append(r)

# Aggregate results

def agg(res_list):

sr = np.array([r["success"] for r in res_list])

bt = np.array([r["battery"] for r in res_list])

rv = np.array([r["eval_r"] for r in res_list])

return dict(

success_mean=round(float(sr.mean()), 1),

success_std=round(float(sr.std()), 1),

battery_mean=round(float(bt.mean()), 1),

battery_std=round(float(bt.std()), 1),

reward_mean=round(float(rv.mean()), 1),

reward_std=round(float(rv.std()), 1),

)

main_agg = {a: agg(main_res[a]) for a in ALGOS}

print("\n" + "="*70)

print(" MAIN RESULTS (mean ± std, 5 seeds):")

print("="*70)

print(f" {'Algo':<6} {'SR%':>12} {'Bat%':>12} {'Reward':>12}")

for a in ALGOS:

d = main_agg[a]

print(f" {a:<6} {d['success_mean']:>5.1f}±{d['success_std']:<5.1f} "

f"{d['battery_mean']:>5.1f}±{d['battery_std']:<5.1f} "

f"{d['reward_mean']:>6.1f}±{d['reward_std']:<5.1f}")

# ═══════════════════════════════════════════════════════════════════

# EXPERIMENT 2 — Ablation study (4 reward modes × 5 seeds, SAC only)

# ═══════════════════════════════════════════════════════════════════

print("\n" + "="*70)

print(" EXPERIMENT 2 — Ablation study (SAC, 4 reward modes)")

print("="*70)

abl_res = {m: [] for m in ABL_MODES}

abl_labels = {

"full": "Full reward",

"no_energy": "No energy penalty",

"no_bat_scale": "No battery scaling",

"standard": "Standard (baseline)"

}

for mode in ABL_MODES:

print(f"\n>>> {abl_labels[mode]} <<<")

for seed in SEEDS:

r = train_one("SAC", seed, reward_mode=mode, label=f"SAC-{mode}")

abl_res[mode].append(r)

abl_agg = {m: agg(abl_res[m]) for m in ABL_MODES}

print("\n ABLATION RESULTS:")

print("-"*70)

for m in ABL_MODES:

d = abl_agg[m]

print(f" {abl_labels[m]:<26} SR={d['success_mean']:>5.1f}±{d['success_std']:<4.1f}"

f" Bat={d['battery_mean']:>5.1f}±{d['battery_std']:<4.1f}")

# ═══════════════════════════════════════════════════════════════════

# EXPERIMENT 3 — Classical A* + PID baseline

# ═══════════════════════════════════════════════════════════════════

print("\n" + "="*70)

print(" EXPERIMENT 3 — A* + PID Classical Baseline")

print("="*70)

astar = AStarPIDBaseline()

astar_sr = []

astar_bat = []

astar_r = []

for seed in SEEDS:

for ep in range(EVAL_EPS // len(SEEDS)):

env = DroneNavEnv(seed=seed*100+ep)

env.reset()

r, _, info = astar.plan_and_run(env)

astar_sr.append(float(info.get("success", False)))

astar_bat.append(info.get("battery_used", 0.0))

astar_r.append(r)

astar_agg = dict(

success_mean=round(float(np.mean(astar_sr))*100, 1),

success_std=round(float(np.std(astar_sr))*100, 1),

battery_mean=round(float(np.mean(astar_bat)), 1),

battery_std=round(float(np.std(astar_bat)), 1),

reward_mean=round(float(np.mean(astar_r)), 1),

reward_std=round(float(np.std(astar_r)), 1),

)

print(f" A*+PID: SR={astar_agg['success_mean']}±{astar_agg['success_std']}% "

f"Bat={astar_agg['battery_mean']}±{astar_agg['battery_std']}%")

# ═══════════════════════════════════════════════════════════════════

# EXPERIMENT 4 — Robustness: noise + wind (SAC only, 3 seeds)

# ═══════════════════════════════════════════════════════════════════

print("\n" + "="*70)

print(" EXPERIMENT 4 — Robustness under sensor noise + wind")

print("="*70)

rob_conditions = [

("Clean (0%)", 0.00, 0.00),

("Noise 5%", 0.05, 0.00),

("Noise 10%", 0.10, 0.00),

("Wind σ=0.5", 0.00, 0.50),

("Noise+Wind", 0.05, 0.30),

]

rob_res = {}

for label, noise, wind in rob_conditions:

print(f"\n>>> {label} <<<")

vals = []

for seed in SEEDS[:3]:  # 3 seeds for robustness

r = train_one("SAC", seed, noise_std=noise, wind_std=wind,

label=f"SAC-{label}")

vals.append(r)

rob_res[label] = dict(

success_mean=round(float(np.mean([v["success"] for v in vals])), 1),

success_std=round(float(np.std([v["success"] for v in vals])), 1),

battery_mean=round(float(np.mean([v["battery"] for v in vals])), 1),

)

print(f" Result: SR={rob_res[label]['success_mean']}±{rob_res[label]['success_std']}%")

# ═══════════════════════════════════════════════════════════════════

# SAVE ALL RESULTS

# ═══════════════════════════════════════════════════════════════════

print("\n" + "="*70)

print(" SAVING RESULTS")

print("="*70)

all_results = dict(

main_agg=main_agg,

abl_agg=abl_agg,

astar_agg=astar_agg,

rob_res=rob_res,

seed_rows={

algo: [{

"seed": r["seed"],

"success": r["success"],

"battery": r["battery"],

"eval_r": r["eval_r"]

} for r in main_res[algo]]

for algo in ALGOS

},

main_curves={

a: {

"rr": main_res[a][0]["rr"],

"sr": main_res[a][0]["sr"],

"bc": main_res[a][0]["bc"],

"steps": main_res[a][0]["steps"],

} for a in ALGOS

},

abl_curves={

m: {

"rr": abl_res[m][0]["rr"],

"sr": abl_res[m][0]["sr"],

"bc": abl_res[m][0]["bc"],

} for m in ABL_MODES

},

traj_ok=[p.tolist() for p in (main_res["SAC"][0]["traj_ok"] or [])],

traj_fail=[p.tolist() for p in (main_res["DQN"][0]["traj_fail"] or [])],

config=dict(

train_steps=TRAIN_STEPS,

seeds=SEEDS,

eval_eps=EVAL_EPS,

obs_dim=OBS_DIM,

)

)

results_path = os.path.join(RESULTS_DIR, "metrics_full.json")

with open(results_path, "w") as f:

json.dump(all_results, f, indent=2)

print(f"\nResults saved → {results_path}")

print("\n" + "="*70)

print(" TRAINING COMPLETE!")

print("="*70)

print("\nNext step: Run 'python generate_figures.py' to create figures")

◆  test_sac_quick.py

"""

test_sac_quick.py

=================

Quick test to verify SAC implementation works correctly.

Runs a short training (5k steps) to check for errors.

Usage: python test_sac_quick.py

Expected time: ~1-2 minutes

"""

import sys

import os

import numpy as np

SCRIPT_DIR = os.path.dirname(os.path.abspath(__file__))

sys.path.insert(0, SCRIPT_DIR)

from drone_env import DroneNavEnv

from drl_agents_final import SACAgent

print("="*60)

print(" QUICK SAC TEST")

print("="*60)

# Create environment and agent

env = DroneNavEnv(seed=42)

agent = SACAgent(obs=16, act=3, seed=42)

print(f"\nAgent created:")

print(f"  Actor params: {sum(w.size for w in agent.actor.W)}")

print(f"  Q1 params: {sum(w.size for w in agent.q1.W)}")

print(f"  Q2 params: {sum(w.size for w in agent.q2.W)}")

# Run a few episodes

n_episodes = 10

max_steps_total = 5000

steps_done = 0

print(f"\nRunning {n_episodes} episodes (max {max_steps_total} steps)...")

for ep in range(n_episodes):

obs = env.reset()

ep_reward = 0.0

ep_steps = 0

done = False

while not done and steps_done < max_steps_total:

# Select action

action = agent.act(obs, det=False)

# Step environment

next_obs, reward, term, trunc, info = env.step(action)

done = term or trunc

# Store and train

agent.store(obs, action, reward, next_obs, float(done))

agent.train()

ep_reward += reward

ep_steps += 1

steps_done += 1

obs = next_obs

success = info.get("success", False)

battery = info.get("battery_used", 0.0)

print(f"  Episode {ep+1}: Steps={ep_steps:3d}, Reward={ep_reward:7.1f}, "

f"Success={success}, Battery={battery:.1f}%")

print(f"\nTotal steps: {steps_done}")

print(f"Buffer size: {len(agent.buf)}")

# Test action selection

print("\nTesting action selection...")

test_obs = np.random.randn(16).astype(np.float32)

action = agent.act(test_obs, det=True)

print(f"  Deterministic action: {action}")

action = agent.act(test_obs, det=False)

print(f"  Stochastic action: {action}")

print("\n" + "="*60)

print(" SAC TEST PASSED!")

print("="*60)

print("\nThe SAC implementation appears to be working correctly.")

print("You can now run the full training with:")

print("  python run_all_experiments_fixed.py")

Disclaimer/Publisher's Note: The statements, opinions and data contained in all publications are solely those of the individual author(s) and contributor(s) and not of IR and/or the editor(s).IR and/or the editor(s) disclaim responsibility for any injury to people or property resulting from any ideas, methods, instructions or products referred to in the content.