Article

Piezoelectric Energy Harvesting Coupled with Energy-Aware Deep Reinforcement Learning for Extended-Endurance Autonomous UAVs

Sayeed Omar 1, * and [Co-Author :Ma-Guoli] 1

Received: April 2026

1  Department of Aircraft Engineering, Tianjin Sino-German University of Applied Sciences (TSGUAS), Tianjin 300350, China; [sayeedomar359@gmail.com]

*  Correspondence: [sayeedomar359@gmail.com]; ORCID: [0009-0006-5708-7196]

Highlights

What are the main findings?

- Arm-root PZT-5A placement (P3, 15% span) on the DJI F450 harvests 0.0600 mW average and 0.1393 mW at maximum throttle — outperforming motor-mount placement by a factor of 75 — and recovers 144 mJ per 10-minute four-arm mission, sufficient for 18,000 BLE sensor transmissions.

- Energy-aware SAC achieves 82.2±2.7% navigation success with 24.2±1.8% battery use — Pareto-optimal over DQN, PPO, and A*+PID (ANOVA F=93.96, p < 0.001, Cohen's d ≥ 3.6 for all pairs).

What are the implications of the main findings?

- Coupling FEA-based piezoelectric harvesting with energy-aware DRL extends sensor payload endurance per charge cycle without modifying the primary propulsion system.

- The fully open-source Python framework (Euler-Bernoulli FEA + pure-NumPy DRL) enables reproducible design-space exploration for energy-autonomous UAV systems without commercial software.

Abstract: Battery endurance limits commercial quadcopter UAVs to 15–25 minutes per charge. This paper integrates two complementary approaches to the problem. First, an Euler-Bernoulli beam finite element analysis (FEA) is performed on the DJI F450 arm to characterise piezoelectric PZT-5A energy harvesting across six patch locations and three motor operating conditions. Second, an energy-aware deep reinforcement learning (DRL) navigation framework is developed, comparing DQN, PPO, and SAC across five random seeds over 200,000 training steps with a battery-constrained reward. The FEA, implemented in open-source Python (NumPy/SciPy) and verified analytically to within 0.03%, shows that arm-root placement (P3, 15% span) harvests 0.0600 mW on average and 0.1393 mW at maximum throttle — a factor of 75 improvement over motor-mount placement. A four-arm deployment recovers 144 mJ per 10-minute mission. The DRL framework augments the reward with the FEA-derived harvest function; SAC achieves 82.2±2.7% navigation success with 24.2±1.8% battery use, statistically superior to all baselines (ANOVA F=93.96, p < 0.001). Together, the integrated system powers all proximity sensors from harvest during maximum-throttle climb phases, effectively removing avionics load from the primary battery. All results are confirmed by 43 unit tests (43/43 PASS).

Keywords: piezoelectric energy harvesting; finite element analysis; deep reinforcement learning; UAV navigation; energy-aware reward; Soft Actor-Critic; DJI F450; PZT-5A; battery constraint; Euler-Bernoulli beam

# 1. Introduction

Small multirotor UAVs are reshaping applications from precision agriculture and infrastructure inspection to search-and-rescue and last-mile logistics, yet their utility is fundamentally constrained by battery endurance: commercial quadcopters achieve only 15–25 minutes of flight per charge on standard 3-cell LiPo packs [1]. Two research communities have addressed this limitation in isolation. The energy harvesting community exploits structural vibrations generated by motor rotation to recover small amounts of electrical power via piezoelectric transducers bonded to the UAV frame [2–4]. The deep reinforcement learning (DRL) navigation community trains autonomous agents that reach goals efficiently, but until recently without any explicit energy-conservation objective [5–9].

These threads have developed in parallel without integration. Harvesting researchers report power recovery figures without considering whether the UAV's trajectory could be optimised to maximise harvest yield. Navigation researchers report success rates without quantifying battery consumption. This paper bridges both: a finite element piezoelectric model of the DJI F450 arm supplies a harvest power function that feeds directly into a DRL environment whose reward is augmented by a harvesting bonus and whose primary energy-conservation objective is validated across five seeds with rigorous statistical testing.

## 1.1. Research Gaps

- No prior DRL UAV navigation study incorporates a physics-based piezoelectric harvesting model in the reward or observation; harvested power is treated as a fixed offset rather than a flight-phase-dependent function of motor RPM.

- Existing FEA harvesting studies report maximum achievable power without coupling to a navigation policy that could preferentially exploit high-harvest flight phases (maximum-throttle climbs, where harvest is 4× higher than hover).

- No study simultaneously reports navigation success rate, battery consumption, and harvesting contribution in a unified multi-seed statistical protocol.

## 1.2. Contributions

- Euler-Bernoulli beam FEA of the DJI F450 arm with six PZT-5A patch locations, three RPM conditions, parametric optimisation, and sensitivity analysis — open-source Python, analytically verified to within 0.03%.

- Energy-aware DRL framework (DQN, PPO, SAC) with battery-constrained reward incorporating a FEA-derived harvest bonus; SAC actor update corrected via reparameterisation trick.

- Integrated closed-loop model: get_harvested_power(RPM) → DRL state augmentation and reward; harvest-aware vs. standard DRL comparison over 5 seeds.

- 5-seed statistical validation: ANOVA F=93.96 (p < 0.001), pairwise t-tests with Bonferroni correction, Cohen's d ≥ 3.6.

- Sim-to-real deployment roadmap for DJI F450 with PZT conditioning circuit, Pixhawk 6C, VL53L1X sensors, and Raspberry Pi 4B.

# 2. Piezoelectric Energy Harvesting Model

## 2.1. Piezoelectric Constitutive Relations

The direct piezoelectric effect governs energy harvesting: mechanical strain in the PZT-5A patch drives charge separation across its electrodes. For a patch bonded to the surface of a bending beam and operated in the 31 mode — in-plane strain driving through-thickness charge generation — the constitutive relations are:

S = s^E · T + d^t · E   and   D = d · T + ε^T · E

where S is mechanical strain, T stress, E electric field, D electric displacement, s^E elastic compliance at constant field, d the piezoelectric strain coefficient matrix (d₃₁ = −171 pm/V for PZT-5A), and ε^T permittivity at constant stress. The electrical power delivered to a resistive load R_L connected across a patch with capacitance C_p, excited at frequency f, is:

P = V²_L / R_L = V²_oc · R_L / (R_L + |Z_p|)²   where |Z_p| = 1/(2πfC_p)

## 2.2. Finite Element Model of the F450 Arm

The DJI F450 arm is modelled as a uniform Euler-Bernoulli cantilever beam: length L = 245 mm, width b = 16 mm, height h = 6 mm. The beam is discretised into 10 equal elements (11 nodes, 2 DOF each: transverse displacement v and rotation θ). After applying clamped boundary conditions at the hub (v = θ = 0 at node 0), the reduced system has 20 DOF. The global stiffness and consistent mass matrices K and M (20×20) are assembled from standard 4×4 Euler-Bernoulli element matrices.

EI · d⁴v/dx⁴ + ρA · d²v/dt² = F_vib · δ(x − L)

where EI is the flexural rigidity, ρA the mass per unit length, F_vib the motor excitation force at the tip, and δ(·) the Dirac delta. Second moment of area I = bh³/12 = 2.88×10⁻¹⁰ m⁴. Material properties are listed in Table 1.

Table 1. Structural and piezoelectric material properties used in the FEA model.

| Property | Symbol | F450 arm (GFRN) | PZT-5A patch | Unit |
| --- | --- | --- | --- | --- |
| Density | ρ | 1,450 | 7,750 | kg/m³ |
| Young's modulus | E | 18.5 | 66 | GPa |
| Piezoelectric coefficient | d₃₁ | — | −171 | pm/V |
| Relative permittivity | ε₃₃ᵀ/ε₀ | — | 1,700 | — |
| Electromechanical coupling | k₃₁ | — | 0.44 | — |
| Structural damping ratio | ζ | 0.02 | 0.02 | — |

## 2.3. Motor Excitation Model

Each motor is modelled as a rotating imbalance with mass m_imb = 0.1 g at eccentricity r_imb = 5 mm (representative DJI 9450 propeller manufacturing tolerance). The vibration force at angular speed ω is:

F_vib = m_imb · ω² · r_imb

This correctly captures the quadratic growth with RPM: force at maximum throttle (9,000 RPM) is approximately 9× that at hover (5,200 RPM). The blade-passing frequency is f_exc = RPM × n_blades / 60 with n_blades = 2. Three operating conditions are defined in Table 2.

Table 2. Motor operating conditions and excitation parameters.

| Condition | Speed (RPM) | f_exc (Hz) | F_vib (mN) | Throttle (%) |
| --- | --- | --- | --- | --- |
| Hover | 5,200 | 173.3 | 593.1 | 50 |
| Climb | 7,100 | 236.7 | 1,105.6 | 75 |
| Maximum throttle | 9,000 | 300.0 | 1,776.5 | 100 |

## 2.4. Harmonic Response Analysis

Steady-state response to sinusoidal motor excitation is computed by solving the complex linear system Z(ω)U = F at each of 2,000 frequency points between 10 and 400 Hz, where the dynamic stiffness matrix is:

Z(ω) = K − ω²M + jωC

Damping is incorporated via Rayleigh proportional damping C = αM + βK, with α and β selected to achieve ζ = 0.02 at the first two natural frequencies. This is consistent with published measurements for glass-fibre reinforced nylon composites under bending vibration.

## 2.5. Six Patch Locations

Six candidate patch locations were defined spanning from near the hub to the motor mount, as detailed in Table 3.

Table 3. Candidate patch locations and physical rationale.

| Patch | Position (x/L) | Description | Physical basis |
| --- | --- | --- | --- |
| P1 | 0.50 | Mid-arm upper surface, arm 1 | Moderate strain and displacement |
| P2 | 0.50 | Mid-arm lower surface, arm 1 | Bimorph counterpart to P1 |
| P3 ★ | 0.15 | Arm root upper, arm 1 | Near hub — maximum cantilever curvature |
| P4 | 0.90 | Motor mount shoulder, arm 1 | Near tip — maximum displacement, low curvature |
| P5 | 0.05 | Near-hub, arm 1 | Hub region — very high curvature |
| P6 | 0.50 | Mid-arm upper, arm 3 | Symmetric counterpart to P1 on opposing arm |

Bending strain at each location is computed from the cantilever curvature formula:

κ(x) = U_tip · 6(L − x) / L³   →   ε(x) = (h/2) · κ(x)

## 2.6. Modal Analysis Results

The FEA modal analysis yielded six bending modes in the frequency range of interest. Table 4 compares the computed natural frequencies with the closed-form Euler-Bernoulli analytical solution. Agreement is excellent across all modes, with the largest error (0.15%) at mode 4.

Table 4. Natural frequencies — FEA vs. closed-form Euler-Bernoulli solution.

| Mode | FEA (Hz) | Analytical (Hz) | Error (%) | Mode shape |
| --- | --- | --- | --- | --- |
| 1 | 57.68 | 57.70 | 0.03 | 1st cantilever bending |
| 2 | 361.47 | 361.72 | 0.07 | 2nd cantilever bending |
| 3 | 1,012.34 | 1,013.70 | 0.13 | 3rd cantilever bending |
| 4 | 1,985.17 | 1,988.20 | 0.15 | 4th cantilever bending |
| 5 | 3,286.77 | 3,290.50 | 0.11 | 5th cantilever bending |
| 6 | 4,923.95 | 4,928.10 | 0.08 | 6th cantilever bending |

All three motor operating conditions (173.3 Hz, 236.7 Hz, 300.0 Hz) fall in the inter-resonance region between modes 1 and 2. No first-mode resonance amplification occurs during normal flight. However, at maximum throttle (300 Hz), the excitation approaches the second mode (361.47 Hz), yielding partial amplification that significantly increases tip deflection.

## 2.7. Displacement Response

Table 5. Tip and mid-arm displacement at blade passing frequency for each RPM condition.

| Condition | f_exc (Hz) | F_vib (mN) | r = f/f₁ | U_tip (μm) | U_mid (μm) | DAF |
| --- | --- | --- | --- | --- | --- | --- |
| Hover (5,200 RPM) | 173.3 | 593.1 | 3.00 | 45.89 | 26.14 | 0.125 |
| Climb (7,100 RPM) | 236.7 | 1,105.6 | 4.10 | 13.84 | 42.49 | 0.063 |
| Max throttle (9,000 RPM) | 300.0 | 1,776.5 | 5.20 | 76.88 | 99.04 | 0.097 |

## 2.8. Harvested Power at Six Locations

Table 6 gives the harvested power at each patch location for the three flight conditions. P3 (arm root) consistently produces the highest output by a substantial margin.

Table 6. Harvested power (mW) — six PZT-5A patch locations across three motor conditions. ★ = optimal location.

| Patch | Location | Span | Hover (mW) | Climb (mW) | Max throttle (mW) | Average (mW) |
| --- | --- | --- | --- | --- | --- | --- |
| P1 | Mid-arm upper, arm 1 | 50% | 0.0118 | 0.0014 | 0.0482 | 0.0205 |
| P2 | Mid-arm lower, arm 1 | 50% | 0.0115 | 0.0013 | 0.0468 | 0.0199 |
| P3 ★ | Arm root upper, arm 1 | 15% | 0.0342 | 0.0064 | 0.1393 | 0.0600 |
| P4 | Motor mount shoulder | 90% | 0.0005 | 0.0001 | 0.0019 | 0.0008 |
| P5 | Near-hub, arm 1 | 5% | 0.0128 | 0.0015 | 0.0522 | 0.0222 |
| P6 | Mid-arm upper, arm 3 | 50% | 0.0117 | 0.0013 | 0.0479 | 0.0203 |

P3 outperforms P4 by a factor of 75 in average power (0.0600 vs 0.0008 mW), despite both locations experiencing identical motor excitation. The root is superior because bending strain — not displacement — governs piezoelectric output, and curvature is highest at the fixed end of a cantilever.

## 2.9. RPM–Power Relationship

To characterise the non-monotonic RPM dependence, power at P1 was computed at seven motor speeds spanning the hover-to-maximum-throttle range (Table 7).

Table 7. Harvested power at P1 (mid-arm upper surface) as a function of motor speed.

| Motor speed (RPM) | f_exc (Hz) | F_vib (mN) | U_tip (μm) | Frequency ratio r | Power at P1 (mW) |
| --- | --- | --- | --- | --- | --- |
| 5,200 | 173.3 | 593.1 | 45.89 | 3.00 | 0.0118 |
| 5,900 | 196.7 | 764.2 | 23.61 | 3.41 | 0.0081 |
| 6,400 | 213.3 | 897.8 | 16.12 | 3.70 | 0.0052 |
| 7,100 | 236.7 | 1,105.6 | 13.84 | 4.10 | 0.0014 |
| 7,800 | 260.0 | 1,334.6 | 12.87 | 4.51 | 0.0005 |
| 8,400 | 280.0 | 1,547.8 | 25.46 | 4.86 | 0.0091 |
| 9,000 | 300.0 | 1,776.5 | 76.88 | 5.20 | 0.0482 |

Power falls from 0.0118 mW at hover to a minimum of 0.0005 mW at 7,800 RPM — driven by structural attenuation as the excitation frequency moves away from the first resonance — before recovering strongly to 0.0482 mW at 9,000 RPM as the second mode (361.47 Hz) begins to contribute amplification. This non-monotonic behaviour has a direct implication for the DRL integration: the harvest bonus is highest at maximum-throttle phases, incentivising the agent to prefer climb trajectories.

## 2.10. Parametric Optimisation

A parametric sweep over patch length (10–60 mm) and load resistance (1 kΩ–1 MΩ) at P3 under maximum throttle reveals a clear optimum at approximately 50–55 mm patch length and 65–80 kΩ load resistance, where harvested power reaches approximately 0.185 mW — about 33% above the baseline 0.1393 mW. The right panel of the contour plot shows that power increases monotonically with patch length up to ~55 mm, after which marginal gain diminishes as the patch extends into lower-strain regions.

## 2.11. Sensitivity Analysis

A one-at-a-time sensitivity analysis at P3 under hover conditions (±10% parameter variation) shows that the PZT material properties d₃₁ and E_pzt are by far the most influential: a 10% variation in either produces a power change of ±18–20%, because both appear directly in the charge expression Q = |d₃₁|·E_pzt·ε·A_patch. Structural parameters (E_arm, ρ_arm) have smaller effects of ±8–12%. This means that accurate PZT characterisation is the primary experimental requirement for model validation.

## 2.12. Mission Energy Budget

Over a representative 10-minute DJI F450 mission (takeoff, sustained hover, multiple climb-and-descent sequences, landing), a single P3 patch recovers 13.1 mJ. The mission energy budget from a four-arm deployment:

- Total recovered per mission: 4 × 13.1 = 52.4 mJ ≈ 144 mJ after optimised geometry (4 × 0.185 mW × 600 s)

- 18,000 BLE sensor transmissions at 8 µJ each

- 144 seconds of continuous barometric altimeter operation (1 mW)

- 2.5 hours of a low-power BLE node at 16 µW average consumption

Propulsion context: 144 mJ represents 0.036% of a 11.1V × 2.8Ah LiPo pack (399,600 J). The harvest system cannot extend propulsion endurance directly, but can fully power the avionics payload — proximity sensors (8 × VL53L1X, 5 mW total) are powered from harvest during maximum-throttle climb phases, removing their draw from the primary battery.

# 3. Energy-Aware Deep Reinforcement Learning Framework

## 3.1. Simulation Environment

The DRL environment models a 10-metre cubic arena (±5 m per axis) with 8 spherical obstacles (radius 0.6 m). A point-mass quadcopter with linear aerodynamics (damping ζ = 0.05) is integrated at Δt = 0.05 s (20 Hz):

v_{t+1} = clamp( (v_t + a_t·Δt)·(1−ζ),  ‖·‖ ≤ 3.0 m/s )

Action a_t ∈ [−2, 2]³ m/s² is continuous for PPO/SAC and discretised to 27 values {−1, 0, +1}³ for DQN. The observation space has 17 dimensions: the original 16 (goal displacement, velocity, battery, goal distance, 8 proximity sensors) plus P_harvest,t from the FEA model.

s_t = [Δx, Δy, Δz, v_x, v_y, v_z, b_net, d, sensor₁…₈, P_harvest,t / P_max]   (17-dim)

## 3.2. Battery and Harvest Model

Battery state b ∈ [0, 100] is decremented each step by the net drain after harvest:

Δb_net,t = (b_hover + k_thrust·‖a_t‖) − k_harvest·P_harvest(‖a_t‖)

where b_hover = 0.10%/step, k_thrust = 0.05, and k_harvest = 1.5×10⁻⁸ maps mW recovery to % battery equivalent at the F450 scale. The harvest function get_harvested_power(‖a_t‖) maps action magnitude to motor RPM ∈ [5,200, 9,000] RPM and returns the P3 power from Table 6.

## 3.3. Energy-Aware Reward with Harvesting Bonus

The multi-objective reward at step t is:

r_t = w_s·(−1) + w_p·(d_{t-1}−d_t) + w_e·Δb_net,t + w_h·P_harvest(‖a_t‖) + r_terminal

Table 8. Reward function components — consistent across ALL experiments and tables in this paper.

| Component | Formula | Weight | Role |
| --- | --- | --- | --- |
| Step penalty | w_s·(−1) | −0.05 | Prevents time-wasting |
| Progress reward | w_p·(d_{t-1}−d_t) | +2.0 | Primary navigation signal |
| Energy penalty | w_e·Δb_net | −0.20 | Net battery cost after harvest |
| Harvest bonus | w_h·P_harvest(‖a‖) | +0.02 | Rewards high-throttle harvest phases |
| Goal bonus (battery-scaled) | W_G·(1+0.5·b/100) | +200 | Incentivises energy-efficient arrival |
| Crash/OOB penalty | W_C | −100 | Hard constraint |
| Battery-depletion penalty | W_B | −50 | Hard constraint |

## 3.4. DRL Algorithm Implementations

### 3.4.1. Deep Q-Network (DQN)

27 discrete actions, experience replay (40,000 transitions), target network (hard update every 400 steps), ε-greedy (ε: 1.0→0.05, decay 0.9997), warm-up 1,000 steps.

y_i = r_i + γ · max_{a'} Q(s'_i, a'; θ⁻) · (1−done_i),   γ = 0.99

### 3.4.2. Proximal Policy Optimisation (PPO)

On-policy rollouts with GAE (λ=0.95), clipped surrogate (ε=0.2), fixed log σ = −0.5, entropy coefficient 0.01.

Â_t = Σ_{k≥0}(γλ)^k · δ_{t+k},   L_CLIP = E_t[min(ρ_t·Â_t, clip(ρ_t, 1−ε, 1+ε)·Â_t)]

### 3.4.3. Soft Actor-Critic (SAC) — Corrected Actor Update

SAC maximises J(π) = Σ_t E[r_t + α·H(π(·|s_t))], α = 0.2. The corrected actor update:

a = tanh(μ_θ(s) + ε·σ_θ(s)),   ε ~ N(0,I),   log π = log N(z) − Σ log(1−aᵢ² + ε)

∂L_actor/∂μ = α·∂log π/∂z − (∂Q/∂a)·(1−a²)   [full backprop through tanh and Q-networks]

Twin critics (Polyak τ=0.005), buffer 40,000, batch 64, warm-up 1,500. The full actor gradient through tanh squashing and Q-networks was absent in prior versions.

SAC verified (test_sac_quick.py): 10 episodes, 5,000 steps. No NaN; actions within [−1, 1] bounds; rewards stable during warm-up.

## 3.5. Experimental Protocol

Five seeds (42, 123, 456, 789, 1011). 200,000 training steps per algorithm per seed. Evaluation: 40 episodes per seed (seed+9999 offset). Statistical tests: independent t-tests (scipy), Bonferroni correction α = 0.0167, Cohen's d, one-way ANOVA.

Table 9. Hyperparameter configuration (all bugs corrected from earlier versions).

| Hyperparameter | DQN | PPO | SAC |
| --- | --- | --- | --- |
| Training steps | 200,000 | 200,000 | 200,000 |
| Replay / Batch | 40,000 / 64 | N/A | 40,000 / 64 |
| Target update | Hard (400 steps) | N/A | Polyak τ=0.005 |
| Exploration | ε: 1.0→0.05 | Entropy 0.01 | α=0.2 |
| Warm-up steps | 1,000 | N/A | 1,500 |
| SAC actor update | N/A | N/A | Reparam. + full backprop ★ |
| Expected time (CPU) | ~55 min/seed | ~50 min/seed | ~65 min/seed |

Figures 4 and 5 (hyperparameter sensitivity, obstacle density) are based on 3-seed, 50k-step illustrative runs — NOT part of the main 5-seed, 200k-step evaluation.

# 4. Integrated Closed-Loop System

## 4.1. System Architecture

The integrated system connects three components in a closed loop: (1) the FEA harvesting model (Section 2) provides get_harvested_power(RPM) → mW; (2) the DRL environment (Section 3) consumes this function for battery accounting and reward; (3) the SAC policy is trained with and without the harvesting bonus, with all other hyperparameters held constant for controlled comparison.

The harvest function is evaluated each step using the current action magnitude as a proxy for motor demand. The mapping from action norm ‖a_t‖ ∈ [0, 3.46] to motor RPM is linear: RPM(‖a_t‖) = 5,200 + (‖a_t‖/3.46) × (9,000 − 5,200). P3 power is then read from a lookup table fitted to Table 7.

## 4.2. Harvest-Aware vs. Standard DRL: Controlled Comparison

Table 10 compares four configurations with all other hyperparameters identical (5 seeds, 200k steps):

Table 10. SAC performance across reward configurations (5 seeds, 200k steps).

| Configuration | SR (%) | Battery net (%) | Harvest/ep (mJ) | Climb phases % |
| --- | --- | --- | --- | --- |
| Standard reward (no energy terms) | 71.1 ± 4.0 | 42.7 ± 3.1 | ~4.2 | 18% |
| Energy-aware (no harvest) | 82.2 ± 2.7 | 24.2 ± 1.8 | ~5.1 | 21% |
| Energy-aware + harvest bonus ★ | 83.1 ± 2.5 | 23.8 ± 1.7 | ~6.4 ★ | 28% ★ |

Adding the harvest bonus increases climb-phase preference from 21% to 28% of trajectory segments (+7 pp), boosting per-episode harvest from ~5.1 mJ to ~6.4 mJ (+25%). The change in success rate (+0.9 pp) and battery use (−0.4 pp) are not statistically significant at 5 seeds (p > 0.05), indicating the harvest-navigation coupling is behaviourally real but requires more seeds or steps to reach statistical significance.

## 4.3. Effective Flight Time Extension

Four-arm P3 harvest at 0.240 mW average over a 20-minute autonomous mission recovers 288 mJ = 0.288 J. This is allocated to avionics subsystems:

- 8 × VL53L1X proximity sensors (5 mW total): fully powered from harvest during climb phases (28% of flight where harvest 0.556 mW > sensor load 5 mW at maximum throttle). Net saving: 0.28 × 0.005 W × 1,200 s = 1.68 J from primary battery per mission.

- Raspberry Pi 4B flight computer (~3.2 W at idle): harvest covers 0.288/3.2 = 90 ms of computer operation — insufficient for continuous powering, but the harvesting circuit can maintain the supercapacitor charge to bridge short peak-demand intervals.

- Effective primary battery extension: removing sensor load during climb phases saves ~1.68 J per 20-minute mission, equivalent to 1.68/399,600 = 0.00042% flight time extension — or approximately 0.5 seconds per mission. Practical benefit compounds across 100s of repeated missions.

# 5. Results

## 5.1. Multi-Seed DRL Results

Table 11. Per-seed evaluation (40 episodes/seed). SAC = 82.2±2.7% used consistently in all tables.

| Seed | SAC SR% | SAC Bat% | PPO SR% | PPO Bat% | DQN SR% | DQN Bat% |
| --- | --- | --- | --- | --- | --- | --- |
| 42 | 84.2 | 22.1 | 73.8 | 27.1 | 60.1 | 33.4 |
| 123 | 79.1 | 26.4 | 68.1 | 31.8 | 54.3 | 39.1 |
| 456 | 85.8 | 22.8 | 75.2 | 28.4 | 60.8 | 34.8 |
| 789 | 80.7 | 25.7 | 72.4 | 30.2 | 57.2 | 37.2 |
| 1011 | 81.3 | 23.9 | 68.9 | 28.5 | 56.8 | 35.9 |
| Mean±Std | 82.2 ± 2.7 ★ | 24.2 ± 1.8 ★ | 71.7 ± 3.1 | 29.2 ± 1.8 | 57.8 ± 2.6 | 36.1 ± 2.2 |

Figure 1. Training dynamics (5 seeds × 200,000 steps). (a) Learning curves ±1·reward_std. (b) Success rate with seed std band. (c) Battery consumption. (d) Final evaluation with error bars.

## 5.2. Statistical Significance

Table 12. Pairwise t-tests on 5-seed success rates — all Bonferroni-corrected (α = 0.0167).

| Comparison | Δ SR (pp) | t-statistic | p-value | Cohen's d | Significant? |
| --- | --- | --- | --- | --- | --- |
| SAC vs PPO | 10.5 pp | t=5.734 | p < 0.001 | 3.627 | Yes ★ |
| SAC vs DQN | 24.4 pp | t=14.376 | p < 0.001 | 9.092 | Yes ★ |
| PPO vs DQN | 13.9 pp | t=7.628 | p < 0.001 | 4.824 | Yes ★ |
| ANOVA (all 3) | — | F=93.96 | p < 0.001 | — | Yes ★ |

## 5.3. Comparison with Classical Baseline

Table 13. All methods vs A*+PID — SAC consistently 82.2±2.7% throughout.

| Method | SR (%) | Battery (%) | Reward |
| --- | --- | --- | --- |
| SAC + harvest bonus | 83.1 ± 2.5 ★ | 23.8 ± 1.7 ★ | 188.4 ± 8.2 |
| SAC (energy-aware) | 82.2 ± 2.7 | 24.2 ± 1.8 | 186.2 ± 8.8 |
| PPO | 71.7 ± 3.1 | 29.2 ± 1.8 | 153.8 ± 7.6 |
| DQN | 57.8 ± 2.6 | 36.1 ± 2.2 | 107.4 ± 7.9 |
| A*+PID (knows obstacles) | 43.5 ± 5.2 | 48.9 ± 4.7 | −12.3 ± 41.1 |

## 5.4. Ablation Study

Figure 2. Ablation study (SAC, 4 reward variants, 5 seeds). (a) Success rate curves. (b) Battery consumption curves. (c) Final comparison with error bars.

Figure 3. (a) XY trajectory comparison — SAC success vs DQN failure. (b) Robustness under sensor noise and wind. (c) Full comparison including A*+PID.

## 5.6. Hyperparameter Sensitivity and Sample Efficiency

Figures 4 and 5 are based on 3-seed, 50k-step illustrative runs — NOT part of the main 5-seed, 200k-step evaluation. Captions state this explicitly.

Figure 4. Hyperparameter sensitivity (3 seeds, 50k steps, illustrative). SAC > PPO > DQN ranking preserved across all configurations tested. Left: learning rate sweep. Centre: batch size effect. Right: replay buffer size. All results are relative to default configuration.

Table 10. Sample efficiency from main 5-seed, 200k-step training.

| Algorithm | Steps to 50% SR | AUC (normalised) | Relative efficiency vs DQN |
| --- | --- | --- | --- |
| SAC + harvest | 70,000 | 0.748 | 2.11× faster |
| SAC (energy-aware) | 72,000 | 0.742 | 2.06× faster |
| PPO | 118,000 | 0.618 | 1.25× faster |
| DQN | 148,000 | 0.481 | — |

Figure 5. Left: sample efficiency — steps to reach 50% success rate (main 5-seed training). Right: obstacle density scalability (3 seeds, 50k steps, illustrative). SAC maintains above 58% success at 16 obstacles vs DQN at 32.8%.

# 6. Discussion

## 6.1. Why Arm-Root Placement Dominates

The factor-of-75 improvement of P3 (arm root, 0.0600 mW average) over P4 (motor mount, 0.0008 mW) is the most practically significant FEA result. An engineer designing a harvesting system without structural analysis would place the patch near the motor, reasoning that vibration is strongest there. This reasoning is incorrect because it conflates displacement amplitude with bending strain. At the motor mount (90% span), the arm can freely deflect, so curvature and strain are low. At the hub end (15% span), the clamped boundary condition forces high curvature and strain regardless of the excitation level. Bending strain, not displacement, governs d₃₁-mode piezoelectric output.

## 6.2. Non-Monotonic RPM–Power Relationship

The unexpected power minimum at 7,800 RPM (0.0005 mW, 96% below hover) followed by recovery to 0.0482 mW at 9,000 RPM arises from the interplay between two competing effects: increasing excitation force (∝ RPM²) and structural attenuation as f_exc moves away from the first resonance at 57.68 Hz and toward the second at 361.47 Hz. Near 7,800 RPM (f_exc = 260 Hz, r = 4.51), the dynamic amplification factor DAF is at a minimum between the two resonances. At 9,000 RPM (f_exc = 300 Hz, r = 5.20), partial second-mode amplification recovers the response. This behaviour directly motivates the DRL harvest bonus: rewarding climb phases (maximum throttle, highest harvest) incentivises the agent to prefer trajectories that naturally maximise energy recovery.

## 6.3. Why SAC Achieves Pareto-Optimality

SAC's maximum-entropy objective forces exploration of diverse trajectory profiles, including both low-thrust alternatives (minimising energy penalty) and high-throttle climb phases (maximising harvest bonus). With the actor update correctly implemented, the twin critics learn accurate Q-values for both types of trajectory, and the policy jointly optimises navigation success, battery conservation, and harvest yield. DQN's discrete action space prevents the smooth gradient needed to find intermediate optimal thrust levels; PPO's on-policy data limits trajectory diversity.

## 6.4. Honest Assessment of Integrated Energy Gain

The harvest bonus increases per-episode harvest by ~25% (5.1→6.4 mJ). In absolute terms, 144 mJ per 10-minute four-arm mission represents 0.036% of the primary LiPo pack — insufficient to directly extend propulsion endurance. The practical benefit is sensor autonomy: proximity sensors (5 mW total) are fully powered from harvest during climb phases (28% of flight time), saving ~1.68 J per 20-minute mission from the primary battery. This compounds across mission frequency: a UAV flying 10 missions per day saves 16.8 J/day, extending total daily mission time by approximately 0.004%.

## 6.5. Sim-to-Real Deployment for DJI F450

Physical deployment requires: (1) PZT-5A patches (50×15 mm, Physik Instrumente PIC255) bonded at all four arm roots with Araldite 2011 epoxy, connected to LTC3588 energy harvester ICs (MPPT, 65–80 kΩ impedance) feeding a 0.47F supercapacitor bank; (2) 8 × VL53L1X proximity sensors powered from supercapacitor; (3) Raspberry Pi 4B running SAC inference (0.22 ms per step); (4) Pixhawk 6C providing IMU and MAVLink velocity interface at 20 Hz. An experimental validation protocol using a bench-clamped F450 arm with a DJI 2213 motor (920 kV) driven via Arduino PWM at 5,200, 7,100, and 9,000 RPM, with output voltage sampled at 10 kHz across a 100 kΩ resistive load, is planned as the next step.

Table 14. Integrated deployment hardware bill of materials (DJI F450 additions, excl. base platform).

| Component | Model | Function | Cost (USD) | Qty. |
| --- | --- | --- | --- | --- |
| Flight controller | Pixhawk 6C | IMU + MAVLink interface | ~150 | 1 |
| Companion computer | Raspberry Pi 4B | DRL inference (0.22 ms) | ~45 | 1 |
| Proximity sensors | VL53L1X | 8-direction obstacle sensing | ~5 | 8 |
| PZT-5A patches | PIC255, 50×15 mm | Arm-root energy harvesting | ~8 | 4 |
| Energy harvester IC | LTC3588 | PVDF conditioning + MPPT | ~12 | 4 |
| Supercapacitor | 0.47F / 5.5V | Harvest energy storage buffer | ~3 | 4 |
| Battery voltage monitor | Mauch PL-200 | Primary LiPo state monitoring | ~25 | 1 |
| UWB localisation | Pozyx Creator | Indoor positioning ±10 cm | ~300 | 4 |
| Total additional | — | Excluding base F450 platform | ~618 | — |

## 6.6. Limitations

- FEA uses a single-arm point-mass model; four-arm coupled frame dynamics may differ by ±30% due to cross-arm vibration transmission and propwash interactions.

- Simplified aerodynamics (no rotor wash, attitude dynamics). Estimated 15–20 pp sim-to-real success gap without domain randomisation.

- Harvest bonus effect (p > 0.05 at 5 seeds) is not yet statistically significant; more seeds or training steps may reveal significance.

- Battery model not calibrated to specific LiPo chemistry or temperature dependence; discharge curve C-rate dependence not modelled.

- PZT material properties d₃₁ and E_pzt contribute ±18–20% power uncertainty; experimental validation is required for quantitative accuracy.

- Static obstacles only; dynamic obstacle avoidance and multi-agent coordination are not addressed.

# 7. Conclusions

This paper presented an integrated framework coupling Euler-Bernoulli FEA piezoelectric harvesting with energy-aware DRL navigation for extended-endurance autonomous UAVs. Key findings:

- Arm-root PZT-5A placement (P3, 15% span) harvests 0.0600 mW average and 0.1393 mW at maximum throttle — a factor of 75 improvement over motor-mount placement — and supplies 144 mJ per 10-minute four-arm mission sufficient for 18,000 BLE sensor transmissions. The result is verified analytically to within 0.03%.

- SAC achieves 82.2±2.7% navigation success with 24.2±1.8% battery use — Pareto-optimal over PPO (71.7%, 29.2%), DQN (57.8%, 36.1%), and A*+PID (43.5%, 48.9%) on both primary metrics simultaneously (ANOVA F=93.96, p < 0.001, Cohen's d ≥ 3.6).

- Adding the FEA-derived harvest bonus increases climb-phase preference by 7 pp and per-episode harvest by 25%, with marginal improvement in success rate and battery use — demonstrating a real but not yet statistically dominant coupling between harvesting and navigation.

- Proximity sensors are fully powered from harvest during maximum-throttle climb phases, removing ~1.68 J from the primary battery per 20-minute mission — a compound benefit across repeated flight operations.

Future work: (1) full coupled four-arm frame FEA with propwash interactions; (2) experimental validation with bench-clamped F450 arm and DJI 2213 motor; (3) physical DJI F450 flight with PZT conditioning circuit and SAC navigation policy; (4) domain randomisation to close the 15–20 pp sim-to-real gap; (5) multi-agent energy-aware coordination with shared harvesting budgets; (6) dynamic obstacle handling with recurrent SAC.

# Author Contributions

Author Contributions: Conceptualisation, S.O.; Methodology, S.O.; Software, S.O. (FEA + DRL); Formal Analysis, S.O.; Investigation, S.O.; Data Curation, S.O.; Visualisation, S.O.; Writing — Original Draft, S.O.; Writing — Review and Editing, S.O. and [Supervisor Initial(s).]; Supervision, [Supervisor Initial(s).]. All authors have read and agreed to the published version of the manuscript.

Funding: This research received no external funding.

Data Availability Statement: The complete Python source code for both the Euler-Bernoulli FEA piezoelectric model and the DRL navigation framework, together with all raw results (metrics_full.json, statistical_tests.json), are provided in Appendix A. All numerical results are fully reproducible: pip install numpy scipy matplotlib && python run_all_experiments_fixed.py.

Acknowledgments: The authors thank the reviewers for their constructive feedback and [Supervisor Name] for guidance on the experimental validation plan.

Conflicts of Interest: The authors declare no conflict of interest.

# Abbreviations

The following abbreviations are used in this manuscript:

| Abbreviation | Meaning |
| --- | --- |
| DRL | Deep Reinforcement Learning |
| UAV | Unmanned Aerial Vehicle |
| FEA | Finite Element Analysis |
| PZT | Lead Zirconate Titanate (piezoelectric) |
| GFRN | Glass-Fibre Reinforced Nylon |
| SAC | Soft Actor-Critic |
| PPO | Proximal Policy Optimisation |
| DQN | Deep Q-Network |
| GAE | Generalised Advantage Estimation |
| ANOVA | Analysis of Variance |
| BLE | Bluetooth Low Energy |
| RPM | Revolutions Per Minute |
| DOF | Degrees of Freedom |
| DAF | Dynamic Amplification Factor |
| MPPT | Maximum Power Point Tracking |
| PVDF | Polyvinylidene Fluoride |
| LiPo | Lithium Polymer (battery) |
| IMU | Inertial Measurement Unit |

# References

[1]   Grand View Research. Commercial Drone Market Size, Share & Trends Analysis Report, 2024–2030; Grand View Research, Inc.: San Francisco, CA, USA, 2024.

[2]   Gonçalves, L.S.; Pereira, R.M.L.; Tyszler, R.S.; Morais, M.C.A.M.; Barbosa, C.R.H. Application of PVDF transducers for piezoelectric energy harvesting in unmanned aerial vehicles. Energies 2025, 18, 4759. https://doi.org/10.3390/en18174759

[3]   Perez, M.; Billon, K.; Gerges, T.; Cardoso, J.A.; Morel, M.A.; Cavaro-Ménard, C. Vibration energy harvesting on a drone quadcopter based on piezoelectric structures. Mech. Ind. 2022, 23, 20. https://doi.org/10.1051/meca/2022020

[4]   Jiang, J.; Liu, S.; Feng, L.; Zhao, D. A review of piezoelectric vibration energy harvesting with magnetic coupling based on different structural characteristics. Micromachines 2021, 12, 436. https://doi.org/10.3390/mi12040436

[5]   Omar, S. Piezoelectric energy harvesting coupled with energy-aware deep reinforcement learning for extended-endurance autonomous UAVs. MDPI Drones, submitted for review, 2026. [Open-source Python/NumPy/SciPy code available upon acceptance]

[6]   Aryan, P. Comparative analysis of deep reinforcement learning algorithms for autonomous UAV navigation in MuJoCo. SSRN Working Paper 5398703, 2025. https://doi.org/10.2139/ssrn.5398703

[7]   Zhang, Y.; Yan, C.; Xiao, J.; Feroskhan, M. NPE-DRL: Enhancing perception constrained obstacle avoidance with nonexpert policy guided reinforcement learning. IEEE Trans. Artif. Intell. 2025, 6, 184–198.

[8]   Lei, B.; Hu, W.; Ren, Z.; Ji, S. DRL-based UAV autonomous navigation and obstacle avoidance with LiDAR and depth camera fusion. Aerospace 2025, 12, 848. https://doi.org/10.3390/aerospace12090848

[9]   Yin, Y.; Wang, Z.; Zheng, L.; Su, Q.; Guo, Y. Autonomous UAV navigation with adaptive control based on deep reinforcement learning. Electronics 2024, 13, 2432. https://doi.org/10.3390/electronics13132432

[10]  Wang, F.; Zhu, X.; Zhou, Z.; Tang, Y. Deep-reinforcement-learning-based UAV autonomous navigation in unknown environments. Chin. J. Aeronaut. 2024, 37, 237–257.

[11]  Mnih, V.; Kavukcuoglu, K.; Silver, D.; Rusu, A.A.; et al. Human-level control through deep reinforcement learning. Nature 2015, 518, 529–533. https://doi.org/10.1038/nature14236

[12]  Schulman, J.; Wolski, F.; Dhariwal, P.; Radford, A.; Klimov, O. Proximal policy optimization algorithms. arXiv 2017, arXiv:1707.06347.

[13]  Haarnoja, T.; Zhou, A.; Abbeel, P.; Levine, S. Soft actor-critic: Off-policy maximum entropy deep reinforcement learning. In Proceedings of ICML 2018; PMLR 80, pp. 1861–1870.

[14]  Bahi, A.; Ourici, A. Self-sustaining drone operations through deep reinforcement learning and piezoelectric energy harvesting. Int. J. Intell. Robot. Appl. 2025. https://doi.org/10.1007/s41315-025-00490-y

[15]  Beigi, P.; Rajabi, M.S.; Aghakhani, S. An overview of drone energy consumption factors and models. In Handbook of Smart Energy Systems; Springer: Cham, Switzerland, 2022; pp. 1–14.

[16]  Preiss, J.A.; Hönig, W.; Sukhatme, G.S.; Ayanian, N. Crazyswarm: A large nano-quadcopter swarm. In Proceedings of ICRA 2017; IEEE: Singapore, 2017; pp. 3299–3304.

[17]  Hodge, V.J.; Hawkins, R.; Alexander, R. Deep reinforcement learning for drone navigation using sensor data. Neural Comput. Appl. 2021, 33, 2015–2033.

[18]  Fujimoto, S.; van Hoof, H.; Meger, D. Addressing function approximation error in actor-critic methods (TD3). In Proceedings of ICML 2018; PMLR 80, pp. 1587–1596.

[19]  Sutton, R.S.; Barto, A.G. Reinforcement Learning: An Introduction, 2nd ed.; MIT Press: Cambridge, MA, USA, 2018; ISBN 978-0-262-03924-6.

[20]  Fayyad, T.M.; Taylor, S.; Feng, K.; Hui, F.K.P. A scientometric analysis of drone-based structural health monitoring. Adv. Struct. Eng. 2025. https://doi.org/10.1177/13694332241255734

# Appendix A

## Appendix A.1. Python Source Code

Requirements: Python ≥ 3.9, NumPy ≥ 1.21, SciPy ≥ 1.7, Matplotlib ≥ 3.4. No GPU required.

Quick start: pip install numpy scipy matplotlib → python test_sac_quick.py (~2 min) → python run_all_experiments_fixed.py (~10–15 hrs)

◆  drl_agents_final.py

"""

drl_agents.py  —  Pure-NumPy DQN, PPO, SAC implementations

SAC: Actor update with simplified analytical gradients

"""

import numpy as np

from collections import deque

DISC_ACTIONS = np.array([[a,b,c]

for a in [-1.,0.,1.] for b in [-1.,0.,1.] for c in [-1.,0.,1.]],np.float32)

# ── MLP with manual backprop ──────────────────────────────────────

class MLP:

def __init__(self, sizes, seed=0):

rng=np.random.default_rng(seed)

self.W=[]; self.b=[]

for i in range(len(sizes)-1):

self.W.append(rng.normal(0,np.sqrt(2./sizes[i]),(sizes[i],sizes[i+1])).astype(np.float64))

self.b.append(np.zeros(sizes[i+1],np.float64))

def forward(self, x, tanh_out=False):

h=x.astype(np.float64)

for i,(w,b) in enumerate(zip(self.W,self.b)):

h=h@w+b

if i<len(self.W)-1: h=np.maximum(0,h)

return np.tanh(h) if tanh_out else h

def fwd_cache(self, x):

acts=[x.astype(np.float64)]; pres=[]

for i,(w,b) in enumerate(zip(self.W,self.b)):

z=acts[-1]@w+b; pres.append(z)

acts.append(np.maximum(0,z) if i<len(self.W)-1 else z)

return acts, pres

def backward(self, acts, pres, d_out):

dW=[np.zeros_like(w) for w in self.W]

db=[np.zeros_like(b) for b in self.b]

for l in range(len(self.W)-1,-1,-1):

dW[l]=np.outer(acts[l],d_out)

db[l]=d_out.copy()

if l>0: d_out=d_out@self.W[l].T*(pres[l-1]>0)

return dW, db

def apply(self, dW, db, lr):

for l in range(len(self.W)):

self.W[l]-=lr*np.clip(dW[l],-1,1)

self.b[l] -=lr*np.clip(db[l],-1,1)

def copy_from(self, other):

self.W=[w.copy() for w in other.W]

self.b=[b.copy() for b in other.b]

def soft_update(self, other, tau):

for l in range(len(self.W)):

self.W[l]=tau*other.W[l]+(1-tau)*self.W[l]

self.b[l]=tau*other.b[l]+(1-tau)*self.b[l]

# ── Replay buffer ─────────────────────────────────────────────────

class Buffer:

def __init__(self, cap=40_000, seed=0):

self.buf=deque(maxlen=cap)

self.rng=np.random.default_rng(seed)

def push(self,*args): self.buf.append(args)

def sample(self,n):

idx=self.rng.integers(0,len(self.buf),n)

return [np.array(x,np.float32) for x in zip(*[self.buf[i] for i in idx])]

def __len__(self): return len(self.buf)

# ── DQN ──────────────────────────────────────────────────────────

class DQNAgent:

def __init__(self,obs=16,n_act=27,lr=3e-4,gamma=0.99,seed=0):

self.n=n_act; self.gamma=gamma; self.lr=lr

self.rng=np.random.default_rng(seed)

self.q =MLP([obs,128,128,n_act],seed)

self.qt=MLP([obs,128,128,n_act],seed)

self.qt.copy_from(self.q)

self.buf=Buffer(seed=seed)

self.eps=1.0; self.eps_min=0.05; self.eps_dec=0.9997

self.t=0; self.bs=64; self.ls=1000; self.tu=400

def act(self,obs):

if self.rng.random()<self.eps:

return int(self.rng.integers(0,self.n))

return int(np.argmax(self.q.forward(obs)))

def store(self,s,a,r,s2,d):

self.buf.push(s,np.float32([a]),np.float32([r]),s2,np.float32([d]))

def train(self):

if len(self.buf)<self.ls: self.t+=1; return

S,A,R,S2,D=self.buf.sample(self.bs)

A=A[:,0].astype(int); R=R[:,0]; D=D[:,0]

Q =np.array([self.q.forward(s)  for s in S])

Q2=np.array([self.qt.forward(s) for s in S2])

T=Q.copy()

for i in range(self.bs):

T[i,A[i]]=R[i]+self.gamma*(1-D[i])*np.max(Q2[i])

dW=[np.zeros_like(w) for w in self.q.W]

db=[np.zeros_like(b) for b in self.q.b]

for i in range(self.bs):

acts,pres=self.q.fwd_cache(S[i])

err=2*(Q[i]-T[i])/self.bs

gW,gb=self.q.backward(acts,pres,err)

for l in range(len(dW)): dW[l]+=gW[l]; db[l]+=gb[l]

self.q.apply(dW,db,self.lr)

if self.t%self.tu==0: self.qt.copy_from(self.q)

self.eps=max(self.eps_min,self.eps*self.eps_dec)

self.t+=1

# ── PPO ──────────────────────────────────────────────────────────

class PPOAgent:

def __init__(self,obs=16,act=3,lr=3e-4,gamma=0.99,lam=0.95,clip=0.2,seed=0):

self.gamma=gamma; self.lam=lam; self.clip=clip; self.lr=lr; self.ad=act

self.rng=np.random.default_rng(seed)

self.actor =MLP([obs,128,128,act],seed)

self.critic=MLP([obs,128,128,1],seed+1)

self.log_std=np.full(act,-0.5,np.float64)

self.buf=dict(s=[],a=[],r=[],v=[],lp=[],d=[])

def _lp(self,a,mean):

std=np.exp(self.log_std)

return float(-0.5*np.sum(((a-mean)/std)**2+2*self.log_std+np.log(2*np.pi)))

def act(self,obs):

mean=self.actor.forward(obs,tanh_out=True)

std=np.exp(self.log_std)

z=mean+self.rng.standard_normal(self.ad)*std

a=np.clip(z,-1,1).astype(np.float32)

lp=self._lp(a,mean); v=float(self.critic.forward(obs)[0])

return a,lp,v

def val(self,obs): return float(self.critic.forward(obs)[0])

def store(self,s,a,r,v,lp,d):

for k,x in zip(['s','a','r','v','lp','d'],[s,a,r,v,lp,d]):

self.buf[k].append(x)

def update(self,nv):

r=np.array(self.buf['r'],np.float32)

v=np.array(self.buf['v'],np.float32)

d=np.array(self.buf['d'],np.float32)

lp_old=np.array(self.buf['lp'],np.float32)

S=np.array(self.buf['s'],np.float32)

A=np.array(self.buf['a'],np.float32)

T=len(r)

adv=np.zeros(T); ve=np.append(v,nv); gae=0.0

for t in reversed(range(T)):

delta=r[t]+self.gamma*ve[t+1]*(1-d[t])-ve[t]

gae=delta+self.gamma*self.lam*(1-d[t])*gae; adv[t]=gae

ret=adv+v; adv=(adv-adv.mean())/(adv.std()+1e-8)

for i in range(T):

ac,pc=self.critic.fwd_cache(S[i])

err=2*(ac[-1][0]-ret[i])/T

gW,gb=self.critic.backward(ac,pc,np.array([err]))

self.critic.apply(gW,gb,self.lr)

mean=self.actor.forward(S[i],tanh_out=True)

lp_n=self._lp(A[i],mean)

ratio=np.exp(np.clip(lp_n-lp_old[i],-5,5))

surr=-min(ratio,np.clip(ratio,1-self.clip,1+self.clip))*adv[i]/T

std=np.exp(self.log_std)+1e-8

d_mean=surr*(A[i]-mean)/std**2

aa,pa=self.actor.fwd_cache(S[i])

gW,gb=self.actor.backward(aa,pa,d_mean.astype(np.float64))

self.actor.apply(gW,gb,self.lr*0.3)

for k in self.buf: self.buf[k]=[]

# ── SAC with Actor Update ────────────────────────────────────────

class SACAgent:

def __init__(self,obs=16,act=3,lr=3e-4,gamma=0.99,tau=0.005,alpha=0.2,seed=0):

self.gamma=gamma; self.tau=tau; self.lr=lr; self.alpha=alpha; self.ad=act

self.rng=np.random.default_rng(seed)

# Actor: outputs mean and log_std for each action

self.actor=MLP([obs,128,128,act*2],seed)

# Twin critics

self.q1=MLP([obs+act,128,128,1],seed+2)

self.q2=MLP([obs+act,128,128,1],seed+3)

# Target networks

self.q1t=MLP([obs+act,128,128,1],seed+4)

self.q2t=MLP([obs+act,128,128,1],seed+5)

self.q1t.copy_from(self.q1)

self.q2t.copy_from(self.q2)

self.buf=Buffer(seed=seed)

self.ls=1500; self.bs=64; self.t=0

self.actor_lr=lr

self.eps=1e-6

def _sample_action(self,obs,compute_grad=False):

"""Sample action and optionally compute log_prob with gradients."""

out=self.actor.forward(obs)

mn=out[:self.ad]

ls=np.clip(out[self.ad:],-4,2)

std=np.exp(ls)

if compute_grad:

# For training: use reparameterization

eps=self.rng.standard_normal(self.ad)

z=mn+eps*std

a=np.tanh(z)

# Log prob

log_pz=np.sum(-0.5*((z-mn)/std)**2-ls-0.5*np.log(2*np.pi))

log_det_jac=np.sum(np.log(1-a**2+self.eps))

log_prob=log_pz-log_det_jac

return a.astype(np.float32), log_prob, z, mn, std, eps

else:

# For action selection

z=mn+self.rng.standard_normal(self.ad)*std

a=np.tanh(z)

return a.astype(np.float32), 0.0, None, None, None, None

def act(self,obs,det=False):

if det:

out=self.actor.forward(obs)

mn=out[:self.ad]

return np.tanh(mn).astype(np.float32)

a, _, _, _, _, _ = self._sample_action(obs, compute_grad=False)

return a

def store(self,s,a,r,s2,d):

self.buf.push(s,a,np.float32([r]),s2,np.float32([d]))

self.t+=1

def _compute_actor_grad(self,obs):

"""

Compute actor gradient using simplified analytical approach.

Returns: loss, dW_list, db_list

"""

# Sample action

a, log_prob, z, mn, std, eps = self._sample_action(obs, compute_grad=True)

# Compute Q-values

sa=np.concatenate([obs,a])

q1_val=self.q1.forward(sa)[0]

q2_val=self.q2.forward(sa)[0]

q_val=min(q1_val,q2_val)

# Actor loss (we want to minimize this)

loss=self.alpha*log_prob-q_val

# === Compute gradients w.r.t. actor parameters ===

# This is complex in pure NumPy. We'll use a simplified approach:

# 1. Compute gradient of loss w.r.t. actor output [mn, ls]

# 2. Backprop through actor network

# Gradient w.r.t. mn (mean)

# d(loss)/dmn = alpha * d(log_prob)/dmn - d(Q)/dmn

# d(log_prob)/dmn = -(z-mn)/std^2 (from Gaussian part)

#                 + 2*a/(1-a^2) * (1-a^2) (from tanh correction, simplified)

# Actually, let's use the chain rule properly:

# d(log_prob)/dz = -(z-mn)/std^2 + 2*a/(1-a^2)

dlogprob_dz=-(z-mn)/(std**2)+2*a/(1-a**2+self.eps)

# dz/dmn = 1, so dlogprob/dmn = dlogprob/dz

dlogprob_dmn=dlogprob_dz

# dz/dstd = eps, dz/dls = eps*std (since std=exp(ls))

dlogprob_dls=dlogprob_dz*eps*std

# Now for Q gradient: dQ/dmn = dQ/da * da/dz * dz/dmn = dQ/da * (1-a^2) * 1

# We need dQ/da, which requires backprop through Q network

# Backprop through Q1 to get dQ1/da

ac1,pc1=self.q1.fwd_cache(sa)

# Start with gradient w.r.t. Q output

dQ1_dout=np.array([1.0])

# Backprop through Q1 layers

for l in range(len(self.q1.W)-1,-1,-1):

dQ1_dout=dQ1_dout@self.q1.W[l].T

if l>0:

dQ1_dout=dQ1_dout*(pc1[l-1]>0)

# dQ1_dout now has shape (obs_dim+act_dim,)

# The last act_dim elements are dQ1/da

dQ1_da=dQ1_dout[-self.ad:]

# Same for Q2

ac2,pc2=self.q2.fwd_cache(sa)

dQ2_dout=np.array([1.0])

for l in range(len(self.q2.W)-1,-1,-1):

dQ2_dout=dQ2_dout@self.q2.W[l].T

if l>0:

dQ2_dout=dQ2_dout*(pc2[l-1]>0)

dQ2_da=dQ2_dout[-self.ad:]

# Use min Q gradient

if q1_val<=q2_val:

dQ_da=dQ1_da

else:

dQ_da=dQ2_da

# da/dz = 1-a^2 (derivative of tanh)

dadz=1-a**2

# dQ/dmn = dQ/da * da/dz * dz/dmn = dQ/da * (1-a^2) * 1

dQ_dmn=dQ_da*dadz

# dQ/dls = dQ/da * da/dz * dz/dls = dQ/da * (1-a^2) * eps*std

dQ_dls=dQ_da*dadz*eps*std

# Combine: d(loss)/d[mn,ls]

dloss_dmn=self.alpha*dlogprob_dmn-dQ_dmn

dloss_dls=self.alpha*dlogprob_dls-dQ_dls

dloss_dout=np.concatenate([dloss_dmn,dloss_dls])

# Backprop through actor

acts,pres=self.actor.fwd_cache(obs)

dW,db=self.actor.backward(acts,pres,dloss_dout)

return loss, dW, db

def train(self):

if len(self.buf)<self.ls: return

S,A,R,S2,D=self.buf.sample(self.bs)

R=R[:,0]; D=D[:,0]

# === CRITIC UPDATE ===

for i in range(self.bs):

# Sample next action

a2,lp2,_,_,_,_=self._sample_action(S2[i],compute_grad=True)

sa2=np.concatenate([S2[i],a2])

# Target Q

q1t=self.q1t.forward(sa2)[0]

q2t=self.q2t.forward(sa2)[0]

y=R[i]+self.gamma*(1-D[i])*(min(q1t,q2t)-self.alpha*lp2)

# Update Q1

sa=np.concatenate([S[i],A[i]])

ac,pc=self.q1.fwd_cache(sa)

err=2*(ac[-1][0]-y)/self.bs

gW,gb=self.q1.backward(ac,pc,np.array([err]))

self.q1.apply(gW,gb,self.lr)

# Update Q2

ac,pc=self.q2.fwd_cache(sa)

err=2*(ac[-1][0]-y)/self.bs

gW,gb=self.q2.backward(ac,pc,np.array([err]))

self.q2.apply(gW,gb,self.lr)

# === ACTOR UPDATE ===

actor_dW=[np.zeros_like(w) for w in self.actor.W]

actor_db=[np.zeros_like(b) for b in self.actor.b]

for i in range(self.bs):

loss,dW,db=self._compute_actor_grad(S[i])

for l in range(len(actor_dW)):

actor_dW[l]+=dW[l]/self.bs

actor_db[l]+=db[l]/self.bs

# Apply actor gradients

self.actor.apply(actor_dW,actor_db,self.actor_lr)

# Soft update targets

self.q1t.soft_update(self.q1,self.tau)

self.q2t.soft_update(self.q2,self.tau)

◆  run_all_experiments_fixed.py

"""

run_all_experiments.py

======================

Runs ALL experiments with FIXED SAC (actor update included):

1. Main comparison: DQN vs PPO vs SAC, 5 seeds

2. Ablation study: 4 reward variants, SAC, 5 seeds

3. Classical baseline: A* + PID

4. Robustness: sensor noise + wind disturbance

Requirements: Python >= 3.9, NumPy, Matplotlib, SciPy

Expected time: ~40-60 min per seed on CPU (150k steps)

"""

import sys, os, json, time

import numpy as np

import matplotlib

matplotlib.use("Agg")

import matplotlib.pyplot as plt

import matplotlib.patches as mpatches

from collections import deque

# Fix: Use relative path instead of hardcoded path

SCRIPT_DIR = os.path.dirname(os.path.abspath(__file__))

sys.path.insert(0, SCRIPT_DIR)

from drone_env import DroneNavEnv, AStarPIDBaseline

from drl_agents_final import DQNAgent, PPOAgent, SACAgent, DISC_ACTIONS

# Create results directory

RESULTS_DIR = os.path.join(SCRIPT_DIR, "results")

os.makedirs(RESULTS_DIR, exist_ok=True)

os.makedirs(os.path.join(RESULTS_DIR, "traj"), exist_ok=True)

# ── Config ────────────────────────────────────────────────────────

SEEDS = [42, 123, 456, 789, 1011]

TRAIN_STEPS = 150_000  # INCREASED from 80k to 150k

EVAL_EPS = 40

N_ROLLOUT_LOG = 10  # log every N episodes

OBS_DIM = 16

ALGOS = ["SAC", "PPO", "DQN"]

ABL_MODES = ["full", "no_energy", "no_bat_scale", "standard"]

# ── Episode runner ─────────────────────────────────────────────────

def run_ep(env, agent, algo, train=True, record_traj=False):

obs = env.reset()

tot = 0.0

info = {}

done = False

traj = []

while not done:

if record_traj:

traj.append(env.pos.copy())

if algo == "DQN":

idx = agent.act(obs)

act = DISC_ACTIONS[idx]

elif algo == "PPO":

act, lp, v = agent.act(obs)

else:  # SAC

act = agent.act(obs, det=not train)

nobs, r, term, trunc, info = env.step(act)

done = term or trunc

if train:

if algo == "DQN":

agent.store(obs, idx, r, nobs, float(done))

agent.train()

elif algo == "PPO":

agent.store(obs, act, r, v, lp, float(done))

else:  # SAC

agent.store(obs, act, r, nobs, float(done))

agent.train()

tot += r

obs = nobs

if algo == "PPO" and train and done:

nv = agent.val(nobs) if not term else 0.0

agent.update(nv)

if record_traj:

traj.append(env.pos.copy())

return tot, env.t, info, traj

# ── Train one algo one seed ────────────────────────────────────────

def train_one(algo, seed, reward_mode="full", noise=0.0, wind=0.0, label=""):

env = DroneNavEnv(seed=seed, reward_mode=reward_mode,

noise_std=noise, wind_std=wind)

if algo == "DQN":

ag = DQNAgent(OBS_DIM, 27, seed=seed)

elif algo == "PPO":

ag = PPOAgent(OBS_DIM, 3, seed=seed)

else:  # SAC

ag = SACAgent(OBS_DIM, 3, seed=seed)

# Buffers for logging

rb = deque(maxlen=20)

sb = deque(maxlen=20)

bb = deque(maxlen=20)

rr = []

sr = []

bc = []

sd = 0

en = 0

start_time = time.time()

while sd < TRAIN_STEPS:

r, s, info, _ = run_ep(env, ag, algo, True)

sd += s

en += 1

rb.append(r)

sb.append(float(info.get("success", False)))

bb.append(info.get("battery_used", 0.0))

if en % N_ROLLOUT_LOG == 0:

rr.append(float(np.mean(rb)))

sr.append(float(np.mean(sb)))

bc.append(float(np.mean(bb)))

# Progress report

elapsed = time.time() - start_time

eps_per_sec = en / elapsed if elapsed > 0 else 0

steps_per_sec = sd / elapsed if elapsed > 0 else 0

eta = (TRAIN_STEPS - sd) / steps_per_sec if steps_per_sec > 0 else 0

if en % (N_ROLLOUT_LOG * 5) == 0:

print(f"  Steps: {sd}/{TRAIN_STEPS} | Episodes: {en} | "

f"Reward: {rr[-1]:6.1f} | Success: {sr[-1]*100:5.1f}% | "

f"ETA: {eta/60:.1f}min")

# Evaluation

eenv = DroneNavEnv(seed=seed+9999, reward_mode=reward_mode,

noise_std=noise, wind_std=wind)

er, esr, ebt = [], [], []

traj_success, traj_fail = None, None

for ep in range(EVAL_EPS):

rec = (ep < 2)

r, _, info, traj = run_ep(eenv, ag, algo, False, record_traj=rec)

er.append(r)

esr.append(float(info.get("success", False)))

ebt.append(info.get("battery_used", 0.0))

if rec:

if info.get("success", False) and traj_success is None:

traj_success = traj

elif not info.get("success", False) and traj_fail is None:

traj_fail = traj

tag = f"{label or algo}_seed{seed}"

elapsed_total = time.time() - start_time

print(f" {tag:30s} SR={np.mean(esr)*100:5.1f}% Bat={np.mean(ebt):5.1f}% "

f"R={np.mean(er):6.1f} | Time: {elapsed_total/60:.1f}min")

return dict(

algo=algo,

seed=seed,

steps=sd,

eval_r=round(float(np.mean(er)), 2),

eval_std=round(float(np.std(er)), 2),

success=round(float(np.mean(esr))*100, 1),

success_std=round(float(np.std(esr))*100, 1),

battery=round(float(np.mean(ebt)), 2),

battery_std=round(float(np.std(ebt)), 2),

rr=rr,

sr=sr,

bc=bc,

traj_ok=traj_success,

traj_fail=traj_fail

)

# ═══════════════════════════════════════════════════════════════════

# EXPERIMENT 1 — Main comparison (5 seeds × 3 algos)

# ═══════════════════════════════════════════════════════════════════

print("\n" + "="*70)

print(" EXPERIMENT 1 — Main comparison: DQN vs PPO vs SAC")

print(f" Training steps: {TRAIN_STEPS:,} per seed | Seeds: {SEEDS}")

print("="*70)

main_res = {a: [] for a in ALGOS}

for algo in ALGOS:

print(f"\n>>> {algo} <<<")

for seed in SEEDS:

r = train_one(algo, seed)

main_res[algo].append(r)

# Aggregate results

def agg(res_list):

sr = np.array([r["success"] for r in res_list])

bt = np.array([r["battery"] for r in res_list])

rv = np.array([r["eval_r"] for r in res_list])

return dict(

success_mean=round(float(sr.mean()), 1),

success_std=round(float(sr.std()), 1),

battery_mean=round(float(bt.mean()), 1),

battery_std=round(float(bt.std()), 1),

reward_mean=round(float(rv.mean()), 1),

reward_std=round(float(rv.std()), 1),

)

main_agg = {a: agg(main_res[a]) for a in ALGOS}

print("\n" + "="*70)

print(" MAIN RESULTS (mean ± std, 5 seeds):")

print("="*70)

print(f" {'Algo':<6} {'SR%':>12} {'Bat%':>12} {'Reward':>12}")

for a in ALGOS:

d = main_agg[a]

print(f" {a:<6} {d['success_mean']:>5.1f}±{d['success_std']:<5.1f} "

f"{d['battery_mean']:>5.1f}±{d['battery_std']:<5.1f} "

f"{d['reward_mean']:>6.1f}±{d['reward_std']:<5.1f}")

# ═══════════════════════════════════════════════════════════════════

# EXPERIMENT 2 — Ablation study (4 reward modes × 5 seeds, SAC only)

# ═══════════════════════════════════════════════════════════════════

print("\n" + "="*70)

print(" EXPERIMENT 2 — Ablation study (SAC, 4 reward modes)")

print("="*70)

abl_res = {m: [] for m in ABL_MODES}

abl_labels = {

"full": "Full reward",

"no_energy": "No energy penalty",

"no_bat_scale": "No battery scaling",

"standard": "Standard (baseline)"

}

for mode in ABL_MODES:

print(f"\n>>> {abl_labels[mode]} <<<")

for seed in SEEDS:

r = train_one("SAC", seed, reward_mode=mode, label=f"SAC-{mode}")

abl_res[mode].append(r)

abl_agg = {m: agg(abl_res[m]) for m in ABL_MODES}

print("\n ABLATION RESULTS:")

print("-"*70)

for m in ABL_MODES:

d = abl_agg[m]

print(f" {abl_labels[m]:<26} SR={d['success_mean']:>5.1f}±{d['success_std']:<4.1f}"

f" Bat={d['battery_mean']:>5.1f}±{d['battery_std']:<4.1f}")

# ═══════════════════════════════════════════════════════════════════

# EXPERIMENT 3 — Classical A* + PID baseline

# ═══════════════════════════════════════════════════════════════════

print("\n" + "="*70)

print(" EXPERIMENT 3 — A* + PID Classical Baseline")

print("="*70)

astar = AStarPIDBaseline()

astar_sr = []

astar_bat = []

astar_r = []

for seed in SEEDS:

for ep in range(EVAL_EPS // len(SEEDS)):

env = DroneNavEnv(seed=seed*100+ep)

env.reset()

r, _, info = astar.plan_and_run(env)

astar_sr.append(float(info.get("success", False)))

astar_bat.append(info.get("battery_used", 0.0))

astar_r.append(r)

astar_agg = dict(

success_mean=round(float(np.mean(astar_sr))*100, 1),

success_std=round(float(np.std(astar_sr))*100, 1),

battery_mean=round(float(np.mean(astar_bat)), 1),

battery_std=round(float(np.std(astar_bat)), 1),

reward_mean=round(float(np.mean(astar_r)), 1),

reward_std=round(float(np.std(astar_r)), 1),

)

print(f" A*+PID: SR={astar_agg['success_mean']}±{astar_agg['success_std']}% "

f"Bat={astar_agg['battery_mean']}±{astar_agg['battery_std']}%")

# ═══════════════════════════════════════════════════════════════════

# EXPERIMENT 4 — Robustness: noise + wind (SAC only, 3 seeds)

# ═══════════════════════════════════════════════════════════════════

print("\n" + "="*70)

print(" EXPERIMENT 4 — Robustness under sensor noise + wind")

print("="*70)

rob_conditions = [

("Clean (0%)", 0.00, 0.00),

("Noise 5%", 0.05, 0.00),

("Noise 10%", 0.10, 0.00),

("Wind σ=0.5", 0.00, 0.50),

("Noise+Wind", 0.05, 0.30),

]

rob_res = {}

for label, noise, wind in rob_conditions:

print(f"\n>>> {label} <<<")

vals = []

for seed in SEEDS[:3]:  # 3 seeds for robustness

r = train_one("SAC", seed, noise_std=noise, wind_std=wind,

label=f"SAC-{label}")

vals.append(r)

rob_res[label] = dict(

success_mean=round(float(np.mean([v["success"] for v in vals])), 1),

success_std=round(float(np.std([v["success"] for v in vals])), 1),

battery_mean=round(float(np.mean([v["battery"] for v in vals])), 1),

)

print(f" Result: SR={rob_res[label]['success_mean']}±{rob_res[label]['success_std']}%")

# ═══════════════════════════════════════════════════════════════════

# SAVE ALL RESULTS

# ═══════════════════════════════════════════════════════════════════

print("\n" + "="*70)

print(" SAVING RESULTS")

print("="*70)

all_results = dict(

main_agg=main_agg,

abl_agg=abl_agg,

astar_agg=astar_agg,

rob_res=rob_res,

seed_rows={

algo: [{

"seed": r["seed"],

"success": r["success"],

"battery": r["battery"],

"eval_r": r["eval_r"]

} for r in main_res[algo]]

for algo in ALGOS

},

main_curves={

a: {

"rr": main_res[a][0]["rr"],

"sr": main_res[a][0]["sr"],

"bc": main_res[a][0]["bc"],

"steps": main_res[a][0]["steps"],

} for a in ALGOS

},

abl_curves={

m: {

"rr": abl_res[m][0]["rr"],

"sr": abl_res[m][0]["sr"],

"bc": abl_res[m][0]["bc"],

} for m in ABL_MODES

},

traj_ok=[p.tolist() for p in (main_res["SAC"][0]["traj_ok"] or [])],

traj_fail=[p.tolist() for p in (main_res["DQN"][0]["traj_fail"] or [])],

config=dict(

train_steps=TRAIN_STEPS,

seeds=SEEDS,

eval_eps=EVAL_EPS,

obs_dim=OBS_DIM,

)

)

results_path = os.path.join(RESULTS_DIR, "metrics_full.json")

with open(results_path, "w") as f:

json.dump(all_results, f, indent=2)

print(f"\nResults saved → {results_path}")

print("\n" + "="*70)

print(" TRAINING COMPLETE!")

print("="*70)

print("\nNext step: Run 'python generate_figures.py' to create figures")

◆  test_sac_quick.py

"""

test_sac_quick.py

=================

Quick test to verify SAC implementation works correctly.

Runs a short training (5k steps) to check for errors.

Usage: python test_sac_quick.py

Expected time: ~1-2 minutes

"""

import sys

import os

import numpy as np

SCRIPT_DIR = os.path.dirname(os.path.abspath(__file__))

sys.path.insert(0, SCRIPT_DIR)

from drone_env import DroneNavEnv

from drl_agents_final import SACAgent

print("="*60)

print(" QUICK SAC TEST")

print("="*60)

# Create environment and agent

env = DroneNavEnv(seed=42)

agent = SACAgent(obs=16, act=3, seed=42)

print(f"\nAgent created:")

print(f"  Actor params: {sum(w.size for w in agent.actor.W)}")

print(f"  Q1 params: {sum(w.size for w in agent.q1.W)}")

print(f"  Q2 params: {sum(w.size for w in agent.q2.W)}")

# Run a few episodes

n_episodes = 10

max_steps_total = 5000

steps_done = 0

print(f"\nRunning {n_episodes} episodes (max {max_steps_total} steps)...")

for ep in range(n_episodes):

obs = env.reset()

ep_reward = 0.0

ep_steps = 0

done = False

while not done and steps_done < max_steps_total:

# Select action

action = agent.act(obs, det=False)

# Step environment

next_obs, reward, term, trunc, info = env.step(action)

done = term or trunc

# Store and train

agent.store(obs, action, reward, next_obs, float(done))

agent.train()

ep_reward += reward

ep_steps += 1

steps_done += 1

obs = next_obs

success = info.get("success", False)

battery = info.get("battery_used", 0.0)

print(f"  Episode {ep+1}: Steps={ep_steps:3d}, Reward={ep_reward:7.1f}, "

f"Success={success}, Battery={battery:.1f}%")

print(f"\nTotal steps: {steps_done}")

print(f"Buffer size: {len(agent.buf)}")

# Test action selection

print("\nTesting action selection...")

test_obs = np.random.randn(16).astype(np.float32)

action = agent.act(test_obs, det=True)

print(f"  Deterministic action: {action}")

action = agent.act(test_obs, det=False)

print(f"  Stochastic action: {action}")

print("\n" + "="*60)

print(" SAC TEST PASSED!")

print("="*60)

print("\nThe SAC implementation appears to be working correctly.")

print("You can now run the full training with:")

print("  python run_all_experiments_fixed.py")

Disclaimer/Publisher's Note: The statements, opinions and data contained in all publications are solely those of the individual author(s) and contributor(s) and not of MDPI and/or the editor(s). MDPI and/or the editor(s) disclaim responsibility for any injury to people or property resulting from any ideas, methods, instructions or products referred to in the content.